3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513254

Prague, CZ, October 13th – 17th, 2025

Source: 	vivo

Title: 	Discussion on 6G Sensing

Agenda Item:	8.11

Document for:	Approval

Introduction

The ISAC related study has been ongoing for some time in working groups. However, this topic has never been studied in RAN4. With 6G SI just started in [1], there is guidance in [2] for ISAC that for topics that are not RAN1/2 dependent, RAN4 can start as early as Q4’25.

In this paper, some of the current study of ISAC in 3GPP has been briefly summarized, and some possible preliminary considerations in RAN4 has been proposed.

Discussion

Background

General

The 5G ISAC related study has been ongoing for some time from late 5G. However, this topic has never been studied in RAN4 yet. Now that the 6G ISAC is coming, and it is likely that for RAN4, the 6G ISAC would be discussed even before 6G ISAC. However, the understanding is that 5G ISAC study in other working groups would still be the basic part of 6G, and RAN4 need to take into overall consideration.

Observation 1: It is the first-time sensing being discussed in RAN4.

Observation 2: Although 6G ISAC is the current agenda, the 5G ISAC study as in other working groups studied still need to be considered by RAN4.

Unlike legacy communication systems which is focused on transmission and reception of information, sensing has significantly different scenarios and fundamentally different targets, thus a lot of difference would also be foreseen in the specification work.

In this sense, it is suggested to start with the basic concepts, scenarios, and deployments, which is originated from SA, to help people understand the background and study what should do.

SA1 study background

In SA, comprehensive investigations have been done on the potential of integrated sensing and communication technology for enabling new services and use cases for various industries and documented in [3]. In this Rel-19 technical report, dozens of use cases were defined with KPIs (performance requirements). Among them, the sensing of UAV, which is later paid more attention in Rel-20, appeared in several use cases.

Another key contribution is [4] which is a Rel-19 TS on Service requirements. It should be noted that the “requirements” defined here are significantly different from what RAN4 defined. Take one table in [4] as an example below:

It can be seen that these requirements defined in SA are service-level and system-level, and are much more abstract. By comparison, RAN4 is responsible for RF and performance verification, and much focused on physical layer and RF. So those SA requirements cannot be simply be adopted by RAN4 as it is.

Observation 3: SA has already defined many use cases for 5G ISAC and related service-level performance requirements.

There are also other studies in ISAC work such as in SA2 and SA3, but not mentioned here since they seems have even less impact on RAN4.

RAN1 study background

In RAN1, an SI [5] on the channel modelling for ISAC has been done. The main objective is channel define modelling aspects to support object detection and/or tracking as in [4]. The final feature CR is in [6] and the result is in written into [7].

This channel model is actual the baseline for many of the next stage work.

Observation 4: RAN1 has defined channel modelling for ISAC in Rel-19 in 38.901.

In Rel-20, a new SI leading by RAN1 is passed in [8]. In this SI, the use case of ISAC was restricted to UAV case and gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver). The detailed objective is as following:

It can be seen:

- The metrics, measurements, and relevant measurement quantization for UAV would the first single to study.

- No UE impacts were considered

- No RAN4 objective in this SI.

Based on this situation, it is apparent that some of the most basic physical layer concepts are still absent for RAN4, and how to define RAN4 requirements are still not part of this SI’s scope. However, this is still an active RAN1 ISAC SI that may provide RAN4 some of the ideas and support for next stage work, particually if we cconsider the possible postpone of 6G ISAC discussion in RAN1.

Observation 5: RAN1 has an active 5G Rel-20 SI on ISAC which focus on UAV cases, that might potentially provide some guidance to RAN4.

6G RAN leading SID

A RAN level SI was approved to use for the output to ITU-R, and later also include the general RAN study of 6G Scenarios and requirements in [11] and a TR is in [12].

In this SI, new metric, if exist, would be developed for different scenarios and ISAC is also part of the objective.

6G RAN1 leading SID

In RAN#108, a SID on Study on 6G Radio was approved in [1]. There is following ISAC related objective:

It can be seen that RAN4 finally have considerable participance in the objectives. E.g., for phy functions and procedures which is led by RAN1, and RAN4 aspects including RF, co-existence, and testability.

Observation 6: RAN4 is part of the group involved in 6G RAN SI.

For RAN1 leading objective, it is apparent that starting before RAN1 is problematic for RAN4. In addition, this topic would not be started until RAN1#124b according to RAN1’s plan, making it even more difficult for RAN4 to start the work.

In [2], RAN4 Chair’s guidance is provided as following:

It can be seen that the suggestion is RAN4 can start with topics which do not have RAN1/2 strong dependencies.

Admittedly, this is still challenging, since many of the most basic details are still absent, and confirm RF impact in many cases are not that possible. However, there are still basic issues that can be discussed, and start to do identification is still a good choice.

Proposal 1: RAN4 start to identify topics which are not RAN1/2 dependent and start with what can be identified.

However, it's clear that these descriptions remain at a very high and general level. Given the vast diversity of potential 6G scenarios, defining specific requirements based on such broad outlines is nearly impossible. For now, the analysis is limited to a high-level perspective. To achieve any further refinement, we would need to narrow the scope to a level of specificity at least comparable to that of Rel-20 in 5G.

Observation 7: Only general and preliminary analysis can be done given the vast and unconfirmed scenarios and use cases in 6G ISAC.

Latest progress for 6G ISAC scenarios:

In RAN#109, the following agreements on the scenarios were captured in the chariman’s notes:

“conclusion: proposal 1 and 2, moderator's proposals 1, 2, 3 (without [ ]) and first bullet of moderator's proposal 4 are endorsed”

And the first bullet of proposal 4 in moderator’s summary in [9] is:

- 6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic.

A series of pictures were drawn to show those scenarios as following:

Observation 8: The current scenarios of 6G ISAC is far more broad than 5G case.

RF Impact & Co-existence

RF impacts:

Since currently even the most basic physical layer parameters e.g. waveform and reference signals are still open, it is still early to confirm the basic RF impacts. However, some preliminary guidelines can be considered:

- For the Tx part, if the signals have higher PAPR, the MPR/emission requirements may need to be considered. This is related to waveforms and reference signals, and also other procedures.

- For the Rx part, particularly monostatic sensing, the coexistence of strong self-leakage and weak echoes within the same frequency would means a high level of self-Interference cancellation is needed, and the test scenarios for blocking, intermodulation, and sensitivity degradation may need to be further considered. This is noted to be different from SBFD.

All these are closely related to use cases and physical layer specifications, and the following proposal is provided.

Observation 9: The impact of sensing for BS/UE RF requirements is highly dependent on particular use cases and physical layer specifications.

Proposal 2: The RF impact analysis can generally postponed until more focused use cases and physical layer progress be made.

In addition, it should be noted although R20 RAN1 SI has limited the scope to gNB based mono-static and clarified that UE impacts not expected, this is certainly not the case for 6G in current stage.

The demodulation/CSI requirements may also be impacted and need further discussion. However, these impacts are usually postponed to WI stage and thus no hurry to start.

Proposal 3: As a 6G SI, both BS and UE would have to be taken into consideration. The demod/CSI requirements need not be considered in SI stage.

Co-existence

Traditionally, the co-existence is usually used to verify various interference scenario to confirm the protection of legacy service, and also serve as an important tool to set up RF requirements such as ACLR/ACS. If we stick to this legacy use case, then for sensing it is likely that might be no impact is forseen as long as no new sensing dedicated signal is introduced.

However, some sort of co-existence study may still be needed for ISAC, e.g. if new deployment scenarios and signals introduced such as.

- Communication–sensing co-frequency interference

- Coexistence of multiple RATs

- Multi BS coexistence

There are a lot to considers, at least including use cases, scenarios, basic assumptions, KPIs, etc. It is suggested to wait until more agreements can be obtained from RAN and RAN1.

Proposal 4: The need for co-existence can be discussed later when more input from RAN and RAN1 can be obtained.

New Metric and Testability

In SA TS [4], a set of sensing requirements has been introduced, such as velocity estimation accuracy. These requirements are very high-level system objectives and are not directly aligned with the aspects that RAN4 typically addresses.

Traditionally, 3GPP requirements have been defined for communication systems, while sensing introduces fundamentally different types of metrics. The sensing-related KPIs are closer to those used in the sensor industry, and currently there is no precedent within RAN4 for such metrics.

Currently there is a RAN level study on 6G scenarios and requirements as in [11][12]. It is expected that the sensing related scenarios and requirements would also be studied. The Rel-20 ISAC SI is also likely to somehow study new possible requirements. In all, it is up to RAN1 & RAN setting up the sensing metric.

At this stage, preliminary considerations suggests that certain tests and validations is likely to be required in RAN4. However, they remain use-case dependent and is not yet clearly studied and defined. Moreover, the testing methodologies may differ significantly from conventional OTA testing. Whether new approaches need to be introduced, and how to integrate them into existing frameworks, requires further depth study. It is also noted that these discussions, though still depended on use cases which are not solidly defined, are somehow less depended on RAN1 compared to previous other cases.

Observation 10: It may be necessary to define new test metrics and the metric discussion is also RAN/RAN1 related.

For the testability, the specific test methods and feasibility study could draw on existing practices in the sensor domain.

For example, Radar Target Simulators (RTS) are already used for automotive mmWave radar testing. An RTS is a test instrument that emulates target echoes under laboratory or semi-anechoic chamber conditions. It can simulate target presence, position, velocity, and multiplicity without requiring actual moving objects. By applying controlled delay, frequency shift, and amplitude adjustment to the radar’s transmitted waveform, the RTS replays a synthesized “virtual target” signal back to the device under test. This allows reproducible and controllable evaluation of radar or ISAC systems under a wide variety of target scenarios, including those difficult or impossible to realize in real-world environments.

One example of commercial RTS solutions for 76 GHz automotive radar sensors [10] is as following:

Although this is for 76GHz and also some other differences, extending such methods to lower FR2 bands appears possible, while extension to sub-6 GHz bands may be more challenging.

Proposal 5: The testing methodology for ISAC could use established practices from the sensor industry as a starting point reference.

Proposal 6: Postpone the feasibility and performance metric discussion for ISAC until RAN1/RAN has more detailed design.

RRM impact

Based on the discussion in section 2.1.2, the observation is how to define RAN4 requirements are still not part of the Rel-20 SI’s scope. Based on 6G SID [1], RAN4 will take part in the in the objective a) and e), i.e., physical functions and procedures which is led by RAN1, and RAN4 aspects including RF, co-existence, and testability led by RAN4. However, based on RAN1 time frame, the detail discussion ISAC would not be started until RAN1#124b. From RRM requirement perspective, it is difficult to start the requirement related discussion without any detail RAN1/2 design hence it is suggested to postpone RRM discussion until the Q2/Q3 of 2026.

Proposal 7: Postpone RRM discussion on ISAC till until more detail design from RAN1 is available.

Conclusion

Background

Observation 1: It is the first-time sensing being discussed in RAN4.

Observation 2: Although 6G ISAC is the current agenda, the 5G ISAC study as in other working groups studied still need to be considered by RAN4.

Observation 3: SA has already defined many use cases for 5G ISAC and related service-level performance requirements.

Observation 4: RAN1 has defined channel modelling for ISAC in Rel-19 in 38.901.

Observation 5: RAN1 has an active 5G Rel-20 SI on ISAC which focus on UAV cases, that might potentially provide some guidance to RAN4.

Observation 6: RAN4 is part of the group involved in 6G RAN SI.

Proposal 1: RAN4 start to identify topics which are not RAN1/2 dependent and start with what can be identified.

Observation 7: Only general and preliminary analysis can be done given the vast and unconfirmed scenarios and use cases in 6G ISAC.

Observation 8: The current scenarios of 6G ISAC is far more broad than 5G case.

RF Impact

Observation 9: The impact of sensing for BS/UE RF requirements is highly dependent on particular use cases and physical layer specifications.

Proposal 2: The RF impact analysis can generally postponed until more focused use cases and physical layer progress be made.

Proposal 3: As a 6G SI, both BS and UE would have to be taken into consideration. The demod/CSI requirements need not be considered in SI stage.

Proposal 4: The need for co-existence can be discussed later when more input from RAN and RAN1 can be obtained.

New Metric and Testability

Observation 10: It may be necessary to define new test metrics and the metric discussion is also RAN/RAN1 related.

Proposal 5: The testing methodology for ISAC could use established practices from the sensor industry as a starting point reference.

Proposal 6: Postpone the feasibility and performance metric discussion for ISAC until RAN1/RAN has more detailed design.

RRM

Proposal 7: Postpone RRM discussion on ISAC till until more detail design from RAN1 is available.

- References

[1] RP-251881, New SID: Study on 6G Radio, RAN#108

[2] R4-2511652, Plan on RAN4 6G discussion and meeting arrangement in Rel-20, RAN4 Chair (Apple), RAN4#116

[3] TR 22.837, Feasibility Study on Integrated Sensing and Communication

[4] TR 22.137, Service requirements for Integrated Sensing and Communication

[5] RP-234069, New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, Nokia, Nokia Shanghai Bell, RAN#102

[6] R1-2505006, CR to introduce channel model for ISAC, Xiaomi, AT&T, RAN1#121

[7] 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

[8] RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR, AT&T (Moderator), RAN#108

[9] RP-252943, Moderator's summary for RAN led 6G SI: Requirements of new and existing services, Moderator (CMCC), RAN#109

[10] https://www.keysight.com/us/en/assets/7018-05310/brochures/5992-1648.pdf

[11] RP-250810, Study on 6G Scenarios and Requirements, CMCC, RAN#107

[12] TR 38.914, Study on 6G Scenarios and Requirements