3GPP TSG-RAN WG4 Meeting#116bis 	R4-2513255

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussion on 6G testability and OTA

Agenda Item:	8.12

Document for:	Approval

- Introduction

- 6GR core and performance requirements

- General RF aspects [RAN4]

    - Intra-3GPP co-existence studies

    - Study RF related system parameters and requirements

- BS RF requirement aspects including band [RAN4]

    - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation

    - Study how to improve 6G BS core,  conformance specifications, including structure and drafting principles

- UE RF requirement aspects including band and band combination [RAN4]

    - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR

    - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination

    - Study UE RF capabilities considering different device types and implementations

- RRM aspects for 6GR [RAN4, RAN1, RAN2]

    - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR

    - Study how to improve 6G requirement specification, including structure and drafting principles

- Demodulation and performance aspects [RAN4]

    - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS

    - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

- Aspects related to the testability [RAN4]

    - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity

- Other aspects

    - Handling irregular channel bandwidths including the definition [RAN4, RAN1]

    - Definition of ‘frequency range(s)’[RAN4]

According to RAN4 6G guideline in R4-2511652, and the approved 6G SID [1], the OTA and testability aspects of 6G should be discussed.

In this contribution, we provide our views on the study points of 6G OTA and testability.

- Discussion

  - Summary of UE OTA in 5G

- Projects and scope for UE radiated testability

In NR, the OTA and testability discussion started from the beginning of 5G, and FR2 test methodologies were key scope to ensure the FR2 UEs can be measured properly. After that several follow-up OTA projects to address the testability issue of different UE types, different UE features and frequency ranges. In Table 1, we briefly summarize the projects and scope of UE OTA in 5G:

Table I UE OTA testability projects and main scope in NR

| Projects | Release | Note (OTA related aspects) |
| --- | --- | --- |
| 750044 FS_NR_testMethods | R15 | SI, study test methodologies to cover FR2 RF/RRM/Demod requirements, and preliminary MU assessment. |
| 801001 FS_NR_MIMO_OTA_test | R16 | SI, study CDL channel models and new multi-probe test methodologies for FR1 and FR2 MIMO OTA testing. |
| 850071 FS_FR2_enhTestMethods | R16 | SI, study enhanced FR2 OTA test method to resolve the issues like high DL power, polarization mismatch, extreme temperature, testing time reduction and so on. |
| 880078 NR_MIMO_OTA | R17 | Specify (down-select) test parameters and channel models to define FR1 and FR2 MIMO OTA requirements. Finalize channel model validation. Specify static MIMO OTA requirements. |
| 911010 NR_FR1_TRP_TRS | R17 | Specify FR1 TRP TRS test methods. Finalize channel model validation. Specify TRP TRS OTA requirements of browsing mode. |
| 950068 FS_NR_FR2_OTA_enh | R18 | FR2 OTA SI, to enhance FR2 OTA method to cover UEs with multi-Rx chain reception for up to 4DL layers. |
| 970081 NR_MIMO_OTA_enh | R18 | WI update the channel model validation aspects and finalize lab alignment activity, and then mainly focus on requirement for both FR1 and FR2 MIMO OTA (free space). |
| 970082 NR_FR1_TRP_TRS_enh | R18 | WI specify enhanced test method to cover TxD, RedCap, CA, new method with reverberation chamber, lab alignment activity and SISO OTA requirements of talk mode. |
| 1030074 FS_NR_FR2_OTA_Ph3 | R19 | FR2 OTA SI, to enhance FR2 OTA method to cover the measurement of FR2 non-handheld UE that can transmit simultaneously with multi-panel. |
| 1030085 TRP_TRS_MIMO_OTA_Ph3 | R19 | FR1 OTA WI, to specify the OTA test method to cover XR and FR1 NTN devices, study FR1 dynamic MIMO OTA method with dynamic channel model and link adaptations. More requirements for SISO OTA (browsing mode and talk mode) and MIMO OTA (free space). |
| 1060084 Ambient_IoT_Solutions | R19 | A-IoT WI to specify A-IoT BS, device and CW radiated requirements and corresponding test method. |
| 1020093 NR_AIML_air | R19 | AI/ML WI specify FR2 Beam management requirements and test methods for FR2 AI BM. |
| 1090067 NR_UE_OTA_enh | R20 | Enhancement of UE OTA test method and requirements for NR, enhancement OTA test method to cover Ku and Ka bands, and also VSAT UE Types. More requirements for SISO and MIMO OTA. |

Started from 5G day-1, UE testability is one of the important topics in RAN4, which has been comprehensively studied to cover the new frequency range, new radiated requirements, new UE types, and advanced/complex UE features. After many years’ experiences of testability and OTA discussions, RAN4 should further work on 6G testing based on study outcome and lessons learnt from 5G.

Observation 1: Testability topic was one of the important topics in RAN4, which was comprehensively studied many years to cover the new frequency range, new radiated requirements, new UE types, and advanced/complex UE features.

- Pain points of 5G OTA testing and requirements

5G OTA test method can be roughly divided into four parts: FR2 OTA (AI and non-AI), FR1 TRP/TRS, MIMO OTA, and other radiated methodologies. We would like to briefly summary some of the pain points of the above directions.

For FR2 OTA, the following aspects are some limitations:

- The MU of the test setup is large which leads to large Test Tolerance

- The DFF method can just support small size of devices

- The DFF/IFF method can support multi-AoA system structure, but can not meet the required generation of multi BS beams for AI-based cases

- The cost of FR2 system is high and testing is time-consuming

For FR1 SISO UL/DL OTA, i.e., TRP/TRS or PRP/PRS, the following aspects are some limitations:

- The testing is based on locked antenna, which performance is not representative of real UE radiated performance in the network with adaptive antenna switching

- Current main method is still AC, which requires large chamber size if the frequency goes high or device size is large

- The head/hand phantom used for browsing mode and talk mode can just partially present human body effect, more usage scenarios can not be covered

- The testing efficiency needs improvement

For FR1/FR2 DL MIMO OTA, the following aspects are some limitations:

- The testing is based on multi-probe system, at least 16 for FR1, the system is complex to emulate the multi-path environment, and cost of the system is much high

- Static MIMO OTA (i.e., fixed CDL mode with fixed RMC) performance is not representative of real UE radiated performance, the test method does not collect many industry interests for certification testing

- Only Free space mode, the phantom impact is still not considered

- The Test Zone is highly dependent on which channel model is generated for testing and number of probes supported in chamber

- The >30dB dynamic range control of a chamber with 32-PA bank to cover FR1 frequency range from 700MHz to 6GHz is difficult (this aspect related to specific channel model and test condition).

Observation 2: There are some limitations for FR1/FR2 OTA test methods, which should be addressed in the beginning of 6G.

Proposal 1: OTA test method development in 6G should consider the pain points in 5G and aim to verify the device performance close to the real usage scenario in the network, with using a higher accuracy, reduced cost&complexity, and faster testing methodologies.

  - Improvement of FR1 OTA

- Basic TRP/TRS

After years’ development of OTA, the basic TRP/TRS is still the most important aspect for radiated performance certification of smartphone in many regions/countries. However, recently devices are adopting more antennas and the antenna switching mechanism becomes smarter, the traditional antenna pattern of a locked/fixed Tx primary antenna is far away from the real UE radiated end-to-end performance in the network.

Proposal 2: For FR1 TRP/TRS or PRP/PRS single carrier performance verification, the test method should support/allow UE antenna switching. Repeatability issue of the antenna performance should also be studied.

CA band combination has been an important 5G day-1 feature supported by devices many years, but the radiated performance was not accurately verified. Considering the CA would be a 6G day-1 feature, especially potential new design of single-cell multi carriers, the real radiated performance of UE should be discussed in RAN4 to develop the corresponding OTA test methodologies.

OTA testing should validate performance not just on individual bands but also in various CA combinations. Considering it multiplies the test time and complexity, how to select specific band combinations and reducing redundant testing is also a key point.

Proposal 3: Test method to verify the radiated performance of devices under multi-carrier condition should be discussed in 6G day-1.

- Dynamic FR1 MIMO OTA

The DL dynamic MIMO OTA with link adaptation and dynamic channel model is studied in Rel-19 and captured in TR 38.762, which can be a good starting point for 6G to better quantify the UE DL throughput performance close to end users experience. In 5G, A DUT route is defined by a number of way points. Each way point is assigned a different CDL model, together with orientations, speed and direction of travel (DoT) of the DUT, as shown in figure 1 as example.

Figure 1: One example of UE route for dynamic MIMO OTA for 5G

For 6G, the above concept can be a good baseline and further discuss in RAN4 on more representative dynamic channel model to emulate the multi-path environment of UE route. Meanwhile, test system complexity and cost are the bottleneck of MIMO OTA testing, RAN4 should study some advanced algorithm to create more precise and dynamic spatial channel models with a smaller number of probes.

Proposal 4: Use concept of 5G dynamic channel model and link adaptation as starting point for 6G MIMO OTA test method.

Proposal 5: RAN4 should study advanced algorithm to create more precise and representative dynamic spatial channel models with a smaller number of probes.

- Radiated performance of FR1 MSD

The FR1 MSD value is derived with the assumption of antenna, however, the measurement is performed via conducted testing from antenna ports, which leads to a large gap of the measured results with the specified requirements in spec. This issue makes some MSD requirements less meaningful, and a lot of RAN4 efforts on developing MSD requirements was wasted.

To ensure the MSD requirements provide valueble guidance of network configuration and UE design (interference suppression), the requirement should be verified via OTA-based solution. We know that OTA testing is much time-consuming compared with conducted testing, thus improving test efficiency or some skipping rule can be further studied.

Proposal 6: RAN4 can study OTA-based verification of MSD requirement and provide meaningful results to guide network configuration and UE design. Testing efficiency or skipping rule should also be studied to reduce test burden.

  - New OTA test method for FR3

- FR1-like or FR2-like performance metric

For FR3, devices integrate antennas more closely and move away from conducted connectors, OTA testing would become the standard for evaluating radiated performance. In Rel-20, the enhanced OTA WI study the test method to cover Ka and Ku NTN bands (both FR1-NTN and FR2-NTN frequencies from ~10GHz to 20GHz) for NTN VSAT devices. Some of these study outcome can be considered for 6G.

Meanwhile, it would be valuable that 6G FR3 test method is designed as a general solution for different UE types and features, to minimize the test burden of the industry.

Observation 3: Some general aspects studied in Rel-20 OTA WI those related to above 10GHz testing may be reused for FR3.

- FR2 IFF Test system improvement

Technically, the most promising way is to extend the applicability of FR2 IFF method (24~40GHz) down to 8~10GHz.

CATR solution: CATR systems use a parabolic reflector to create a collimated (planar) wavefront in a relatively short distance, simulating a far-field condition within a compact chamber, this can be a premier solution for 8-24 GHz testing due to the minimized issue of high path loss by reflector and Large, high-quality quiet zone in the chamber. However, the low-frequency range will increase the manufacturing difficulty of reflector, which would be a key challenge of this method.

- FR1 DFF Test system improvement

Current AC system works well below 7GHz, it is difficult to extend the system capability to cover full FR3 ranges due to the limitation of measurement antenna and other RF elements:

- Current measurement antenna can support up to 10~12GHz, FFS higher frequency

- RF elements, e.g., amplifiers, should be updated

- Absorber may also be updated

- New MU assessment is required

Proposal 7: RAN4 should study the feasibility and complexity of extending FR1 AC system and FR2 IFF/DFF system to cover FR3 frequency range.

- Other aspects

For OTA, we typically measure the low/mid/high channel to show the performance of a UE within a band. However, if the CBW of 6G goes higher than 200MHz, whether the flatness of radiated performance of UE within the wide CBW should be verified by an additional metric, should be discussed.

Proposal 8: For wider CBW in FR3, RAN4 should study a reasonable approach to quantify the flatness of UE radiated performance within the CBW range.

  - Test method to support variety of device types

﻿5G starts from smartphone, and further extend to RedCap, XR, NTN, and A-IoT UE types, which introduce difficulty to enhance the test system after it is lunched to market. Currently, in RAN1 and RAN plenary discussions, majority companies believe 6G day-1 should support variety of device types. The testability issue of different device type should also be considered and RAN4 try to resolve that by a generic solution.

In RAN#109 meeting, in the moderator summary [RP‑252873], the following device types are mentioned:

RAN plenary meeting still needs more time to discuss whether 6G device type should be categorized. But in RAN4 testability study, no matter the devices are classified or not, the test methodologies should be generic to cover different device types to minimize the overall cost and complexity of OTA test system.

Observation 4: Different device types would be considered in 6G day-1. OTA test method development should cover different device types.

Proposal 9: Study the testability issue and corresponding test method to quantify different 6G device types, e.g., smartphone, CPE, XR, VUEs, wearables, sensing, NTN, AI agents, and robots.

  - Testability for AI-based features

The testability for AI-based features has been discussed in RAN4 for a long time. Several proposals for AI/ML based BM were discussed, e.g., reusing or extension of FR2 MPAC system, single-probe method similar to FR2 demodulation, simplified multi-AoA IFF FR2 RRM test setup, and Tx-beam-grouping based multi-probe method.

However, due to the cost and complexity of the system, as finally agreed in RAN4#116 meeting [R4-2511890], RAN4 adopt the legacy RRM FR2 OTA test setup with simplified CDL model to perform AI-based BM testing.

The detailed test method (including the channel model, validation, test procedure) of AI/ML based BM is still on-going according to the extension of WID [RP-252930], the outcome of the study can be a baseline for 6G. However, to meet the real requirement of AI/ML-based BM, RAN4 still needs to study how to overcome of the challenges of generating a complex scenario with multiple BS beams towards a CDL model in the chamber by limited number of probes rather than a massive number of probes and RF channels in the channel emulator.

For 6G, in our understanding, beam prediction for AI/ML-based random-access in 6G day 1 including both spatial and temporal domain would be studied. The corresponding requirements and testability should be studied in RAN4 based on lessons learnt from 5G.

Proposal 10: For 6G, an advanced test setup to simulate number of different BS beams and proper multi-path fading spatial channel model should be studied to verify AI/ML based BM.

For other AI-based use cases, e.g., AI/ML-based PA non-linearity improvement, if two-sided model is adopted, then RAN4 should also study how to specify a proper test framework to verify end-to-end system gain of the feature with both BS and UE involved.

  - Testability for BS RF OTA

The BS OTA test methods are studied together with BS requirements, and different from UE OTA many alternative test methods have been specified for conformance testing:

- Indoor Anechoic Chamber

- Compact Antenna Test Range

- Near Field Test Range

- One Dimensional Compact Range

- General chamber

- Reverberation chamber

- Plane Wave Synthesizer

It is expected that the similar approach will be adopted in 6G that the detailed BS OTA test methods embedded in BS requirements will be discussed. However, before that, some general pain points of BS OTA testing can be studied earlier, as least:

- Extending test system to cover new frequency range (FR3)

- Further improve the test efficiency and potential requirement skipping rule

- Alignment/harmonization of different test methodologies

Proposal 11: RAN4 should study the improvement of BS OTA test methods for FR3.

  - Test efficiency improvement

OTA test efficiency improvement is a key aspect to be studied, some of 5G approaches can be reused, e.g.,

- Single point offset approach

- Coarse measurement grid

- Spiral Scan procedure for TRP

- Reverberation chamber

- Fast Tx/Rx Spherical Coverage Method

Proposal 12: RAN4 should study some general solutions (not limited to specific UE type or features) to improve OTA testing efficiency.

- Conclusion

Observation 1: Testability topic was one of the important topics in RAN4, which was comprehensively studied many years to cover the new frequency range, new radiated requirements, new UE types, and advanced/complex UE features.

Observation 2: There are some limitations for FR1/FR2 OTA test methods, which should be addressed in the beginning of 6G.

Observation 3: Some general aspects studied in Rel-20 OTA WI those related to above 10GHz testing may be reused for FR3.

Observation 4: Different device types would be considered in 6G day-1. OTA test method development should cover different device types.

Proposal 1: OTA test method development in 6G should consider the pain points in 5G and aim to verify the device performance close to the real usage scenario in the network, with using a higher accuracy, reduced cost&complexity, and faster testing methodologies.

Proposal 2: For FR1 TRP/TRS or PRP/PRS single carrier performance verification, the test method should support/allow UE antenna switching. Repeatability issue of the antenna performance should also be studied.

Proposal 3: Test method to verify the radiated performance of devices under multi-carrier condition should be discussed in 6G day-1.

Proposal 4: Use concept of 5G dynamic channel model and link adaptation as starting point for 6G MIMO OTA test method.

Proposal 5: RAN4 should study advanced algorithm to create more precise and representative dynamic spatial channel models with a smaller number of probes.

Proposal 6: RAN4 can study OTA-based verification of MSD requirement and provide meaningful results to guide network configuration and UE design. Testing efficiency or skipping rule should also be studied to reduce test burden.

Proposal 7: RAN4 should study the feasibility and complexity of extending FR1 AC system and FR2 IFF/DFF system to cover FR3 frequency range.

Proposal 8: For wider CBW in FR3, RAN4 should study a reasonable approach to quantify the flatness of UE radiated performance within the CBW range.

Proposal 9: Study the testability issue and corresponding test method to quantify different 6G device types, e.g., smartphone, CPE, XR, VUEs, wearables, sensing, NTN, AI agents, and robots.

Proposal 10: For 6G, an advanced test setup to simulate number of different BS beams and proper multi-path fading spatial channel model should be studied to verify AI/ML based BM.

Proposal 11: RAN4 should study the improvement of BS OTA test methods for FR3.

Proposal 12: RAN4 should study some general solutions (not limited to specific UE type or features) to improve OTA testing efficiency.

- References

- RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109, Sep 2025.