3GPP TSG RAN4 meeting #116-bis 	         	              	     R4-2513268

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	8.10 Spectrum sharing

Source:	ISSDU

Title:	Discussion on Multi-RAT Spectrum Sharing for 6GR

Document for:	Discussion

Introduction

At RAN#108, a new Study Item (SID) on 6G Radio was approved [1]. The SID highlights that the evolution towards 6G Radio is essential to support future usage scenarios requiring extreme performance, flexibility, and scalability. In addition, RAN initiated studies on 6G requirements [2][3] to define quantitative targets for key performance indicators (KPIs) such as peak data rate, spectral efficiency, user-experienced data rate, area capacity, user/control plane latency, reliability, mobility, and energy efficiency. These targets are anticipated to exceed those of 5G in most cases. The SID also identifies a set of fundamental requirements and design principles to guide the development of 6GR from the outset. To meet these requirements, 3GPP is expected to design 6GR following the guiding principles outlined below:

- Focus on energy efficiency for NW and device

- Support enhanced spectral efficiency

- Enable wider bandwidth and improve support for diverse spectrum allocations

- Enable scalable and forward compatible design for diverse device types

- Common radio design to meet vertical needs (mobility broadband high priority)

- Enhanced overall coverage, enabling 7 GHz operation with same site grid as 3.5 GHz

- Harmonized TN/NTN

- Avoid excessive configurations/options/capabilities

Among the important topics listed in the 6GR Study Item, multi-RAT spectrum sharing (MRSS) between 5G and 6G has been identified as a key aspect since RAN#108. Furthermore, at RAN#109, RAN agreed that spectrum sharing is a dedicated agenda item for RAN4 under FS_6G_Radio, with the explicit note:

“The scope includes multi-RAT spectrum sharing for migration, mobility between 5G NR and 6G.”

This contribution focuses on this topic, presenting observations, proposals, and an evaluation example related to spectrum sharing across RATs, highlighting aspects that may involve RAN4.

Discussion

To illustrate the motivation of this contribution, Figure 1 depicts a simplified scenario of multi-RAT spectrum sharing (MRSS) between 5G NR and 6GR. In the case of fully overlapped spectrum (ρ=1) without power backoff (Δ=0), significant performance degradation may occur due to aggregated cross-RAT interference. By contrast, when coordinated sharing is applied with partial overlap (ρ<1) and interferer power backoff (Δ>0), the overall performance shows consistent improvement compared to the uncoordinated case. This scenario highlights the central principle of the study, aimed at providing RAN4 with a baseline framework for evaluating NR/6GR coexistence under both uncoordinated and coordinated conditions.

Figure 1: Example of uncoordinated vs coordinated spectrum sharing (ρ, Δ)

It is suggested that RAN4 could evaluate NR/6GR MRSS in representative FR1 scenarios such as 3.5 GHz deployments, considering both symmetric and asymmetric deployment conditions. Possible evaluation parameters may include spectrum overlap ratio (ρ), power backoff (Δ), and interferer EIRP sweeps, with KPIs focusing on metrics such as average spectral efficiency. Such a study would provide reference curves comparing uncoordinated coexistence (ρ=1, Δ=0) with coordinated sharing (ρ<1, Δ>0), and is mainly intended to facilitate further discussion within RAN4.

Figure 2 illustrates the baseline evaluation of 5G NR/6GR MRSS in FR1 with a 20 MHz carrier at 3.5 GHz, showing performance trends as a function of interferer EIRP. The comparison between uncoordinated coexistence (ρ=1, Δ=0) and coordinated sharing (ρ=0.6, Δ=3 dB) indicates that coordinated sharing provides consistent improvements across interference levels.

Observation 1: Uncoordinated NR/6GR spectrum reuse with full overlap (ρ=1) and no power backoff (Δ=0) results in noticeable cross-RAT interference.

Observation 2: Performance trends are sensitive to interferer EIRP and deployment symmetry (symmetric vs asymmetric).

Observation 3: Coordinated sharing with partial overlap (ρ<1) and moderate power backoff (Δ>0) shows consistent improvement across interference levels.

Proposal 1: Evaluate baseline NR/6GR coexistence in FR1 (e.g., 3.5 GHz), comparing uncoordinated (ρ=1, Δ=0) vs coordinated sharing (ρ<1, Δ>0).

Proposal 2: Include EIRP sweeps and deployment conditions in the baseline evaluation, with KPIs such as average spectral efficiency.

Proposal 3: Use reference curves of uncoordinated vs coordinated sharing as inputs to further RAN4 discussions and potential RAN1 alignment.

Figure 2: Example of NR/6GR coexistence performance - uncoordinated overlap vs coordinated sharing with partial overlap and power backoff

It is proposed that RAN4 extend the evaluation to coordinated sharing schemes in which partial overlap (ρ<1) is combined with selective or group-based power backoff (Δ>0) for interfering nodes. Additional profiles may compare periodic versus static adjustment of overlap and guard settings to assess robustness under time-varying interference. These extensions would allow RAN4 to quantify potential gains beyond the baseline case, providing further insight into the benefits of advanced coordination. In addition, evaluations may differentiate between FDD and TDD scenarios, e.g., 15 kHz numerology (FDD) vs 30 kHz numerology (TDD), with KPIs focusing on spectral efficiency.

Figure 3 extends the evaluation to selective coordination scenarios. In these cases, a subset of interferers applies power backoff (Δ>0) while others operate without backoff, combined with periodic versus static adjustment of overlap settings. Results show that periodic adaptation achieves higher robustness under time-varying interference compared with static profiles. Furthermore, the comparison between 15 kHz and 30 kHz subcarrier spacings highlights differences in coexistence performance, reflecting numerology-dependent sensitivity to Doppler and interference aggregation.

Observation 4: Baseline evaluation does not capture the full benefits of advanced coordination.

Observation 5: System robustness under time-varying interference differs between static and periodic adjustment.

Observation 6: Coexistence performance shows numerology-dependent sensitivity to Doppler and interference aggregation.

Proposal 4: Extend the study to include partial overlap (ρ<1) with selective or group-based power backoff (Δ>0).

Proposal 5: Evaluate static versus periodic adaptation of overlap/guard settings.

Proposal 6: Include representative cases with 15 kHz (FDD) and 30 kHz (TDD) numerologies in the evaluation.

Figure 3: Advanced coordination - comparison of static vs periodic overlap adaptation under 15 kHz (FDD) and 30 kHz (TDD) numerologies

Conclusion

In this contribution, we have the following observations and proposals on Multi-RAT Spectrum Sharing for 6GR:

Observation 1: Uncoordinated NR/6GR spectrum reuse with full overlap (ρ=1) and no power backoff (Δ=0) results in noticeable cross-RAT interference.

Observation 2: Performance trends are sensitive to interferer EIRP and deployment symmetry (symmetric vs asymmetric).

Observation 3: Coordinated sharing with partial overlap (ρ<1) and moderate power backoff (Δ>0) shows consistent improvement across interference levels.

Observation 4: Baseline evaluation does not capture the full benefits of advanced coordination.

Observation 5: System robustness under time-varying interference differs between static and periodic adjustment.

Observation 6: Coexistence performance shows numerology-dependent sensitivity to Doppler and interference aggregation.

Proposal 1: Evaluate baseline NR/6GR coexistence in FR1 (e.g., 3.5 GHz), comparing uncoordinated (ρ=1, Δ=0) vs coordinated sharing (ρ<1, Δ>0).

Proposal 2: Include EIRP sweeps and deployment conditions in the baseline evaluation, with KPIs such as average spectral efficiency.

Proposal 3: Use reference curves of uncoordinated vs coordinated sharing as inputs to further RAN4 discussions and potential RAN1 alignment.

Proposal 4: Extend the study to include partial overlap (ρ<1) with selective or group-based power backoff (Δ>0).

Proposal 5: Evaluate static versus periodic adaptation of overlap/guard settings.

Proposal 6: Include representative cases with 15 kHz (FDD) and 30 kHz (TDD) numerologies in the evaluation.

Overall, the proposed evaluations provide a consistent and RAN4-friendly framework to study NR/6GR spectrum sharing, and may support future alignment with RAN1 on assumptions regarding numerology and mobility.

References

[1] RP-251881: New SID: Study on 6G Radio

[2] RP-250810:  Revised SID: Study on 6G Scenarios and requirements

[3] 3GPP TR 38.914: Study on 6G Scenarios and Requirements, v0.1.0.