3GPP TSG-RAN4 Meeting #116bis	R4-2513273

Prague, Czech Republic, 13 October – 17 October 2025

Source:	Ericsson

Title:	6G spectrum and band combination considerations

Agenda item:	    		8.6

Document for:	Approval

# Introduction

This contribution provides views on 6G spectrum considerations and on band combination improvements.

# Spectrum considerations

## Frequency range

6G shall support existing NR frequency bands, meaning bands defined in FR1 and FR2. The cm-Wave frequency domain (e.g. 7-15 GHz)  is very attractive for future systems like 6G and we could expect that some frequency ranges will be allocated to IMT and regulated in the future (WRC-27 agenda includes consideration of 4.4-4.8 GHz, 7.125-8.4GHz and 14.8-15.35 GHz for IMT allocation).

In Rel-16 (2020-2021), RAN4 studied the 7-24 GHz frequency range and the outcomes were captured in TR 38.820 ([1]). In this scope, RAN4 discussed how to handle this 7-24 GHz frequency range, if the definition of the existing FR1 and/or FR2 should be extended, or if a new frequency range (e.g. FR3) should be introduced. If some suggestions have been captured in the TR, no conclusion has been made at that time.

Later, responding to ITU-R LS in the scope of WRC-23, RAN4 specifically studied the 6.425-7.125 GHz and 10.0-10.5 GHz frequency ranges, assuming FR1 like type of UE architecture when considering RF requirements (see TR 38.921 [2]).

More recently, responding to ITU-R LS in the scope of WRC-27, RAN4 studied the 7.125-8.4 GHz and 14.8-15.35 GHz frequency ranges. If, again, FR1 like type of UE architecture was considered for 7.125-8.4 GHz frequency range, both FR1 like and FR2 like have been discussed for the 14.8-15.35 GHz frequency range (see TR 38.922 [3]).

RF and RRM requirements are specified based on different concepts depending on the considered frequency range, FR1 or FR2 (for example, UE RF requirements are conducted in FR1, specified at the antenna connected, while there are OTA in FR2). Based on the considered architecture, this would influence how frequency range should be defined/extended in the 7-24 GHz.

Even though as yet there is no identified or band WI, RAN4 should at this stage further study how to handle future bands in the 7-24 GHz frequency range, agreeing on criteria to either extend an existing frequency range, or introducing a new frequency range (which would also impact the other RAN WGs).

Proposal 1: In the scope of the 6G, RAN4 should study how to handle bands in the frequency range 7-24 GHz, extending existing FR1 and/or FR2 frequency ranges and/or introducing a new frequency range (i.e. FR3), considering how RF and RRM requirements are currently specified in FR1/FR2 as input.

## Regulatory Survey

In the scope of the 7-24 GHz frequency range study, RAN4 made a regulatory landscape survey, captured in TR 38.820 ([1]). Since then, WRC-23 concluded on the allocation of the 10.0-10.5 GHz frequency range to IMT (in Region 2). Except from this, no other update of interest for RAN4 has been made in ITU.

As the 6G study is supposed to conclude by RAN#116 (June 2027), RAN4 should not expect any outcome from WRC-27 in that timeframe (WRC-27 dates is scheduled October 18- November 12, 2027). It would then be premature to study again the regulatory landscape in 4-24GHz, RAN4 would miss some key information. RAN4 should better wait for the conclusions of WRC-27 or if, in the meantime, an Administration decides to early regulate any frequency range in the 7-24 GHz.

Observation 1: It would be premature to investigate further regulations in the 7-24 GHz frequency range in the scope of the 6G study.

Proposal 2: Postpone further regulatory survey in the 7-24 GHz frequency range until Regulators published a new decision.

# Band Combination considerations

## Improvement of MSD handling

In 5G MSD values are nowadays chosen based on existing similar (close) combinations, and in the cases where similar combinations don´t exist new MSD values are typically recommended by UE vendors. For HPUE it has now been agreed to use LUT (Look-Up Tables) to derive HPUE MSD values from each band combination’s existing PC3 MSD values. However, the existing PC3 MSD values used as base to derive new values and for the look-up tables are not consistent and stringent between EN-DC and NR CA, and/or between similar/close band combinations.

It would be good for 6G to consider better guidance on how to set the PC3 MSD baseline values using values and suggestion in a table format. Co-existence studies still need to be performed for 2DL and 3DL combinations, but actual MSD values chosen for intermodulation, harmonic and crossband issues found could use some more rules and guidance.

More strict guidance and rules can be achieved by designating bands into well-defined band groups. The split into defined band groups could be lower low bands (600-800), upper low bands (800-1000), lower mid bands (1400-1700), mid bands (1700-2200), upper mid bands (2200-2700), lower high bands (3300-4200) and upper high bands (4400-7200).

Proposal 3: Study how to improve and simplify selection of MSD values based on the proposed band group concept

## BCS concept

In retrospect It would have been preferable if we in 5G used EN-DC type of configuration tables format also for NR CA. If BCS’s (Bandwidth Combination Sets) were not used for NR CA it would have saved us a lot of work; overly complex, unreadable and sometimes faulty configuration tables, missing fallbacks, etc.

Not using BCS’s would mean that NR CA combinations are assumed to support all channel BW’s defined for each of the bands in the band combinations. This can create potential issues when new channel BW’s are added for existing bands after a band combination is defined. But in practice, this has not been seen as a major issue and has also worked well for EN-DC.

For Intra-band combinations, there is however still a need to keep defining which channel BW’s to be included. especially regarding the UL support.

Proposal 4: Don’t specify BCS’s and specific channel BW’s per band for 6G Inter-band combinations. but keep specific channel BW’s and BCS’s for intra-band combinations.

## Transfer of existing 5G combos

To save many years of work already spent defining band combinations, there is a point in transferring existing 5G NR CA band combinations directly into 6G.

Since the BCS concept is to be avoided in 6G, at least the existing BCS 4 and 5 combinations could be automatically transferred into 6G. But it can also be studied how and if transitions can be done for band combinations not yet defined as BCS 4 and 5 and where additional studies and co-existence checks might be needed before such a transfer.

Proposal 5: Study how existing NR CA band combinations can be transferred directly into a 6G specification baseline

## JSON database

Proposal 6: JSON database is to be used instead of configuration tables for all 6G band combinations.

## Procedural improvements

Block approval of PC3 band combinations contributions are recommended to be continued. However, it is preferred that HPUE baskets are also aligned with this way of working.

It would also be beneficial if there are considerations also to merge HPUE baskets WI’s with the relevant PC3 baskets WI’s.

Observation 2: Discuss further how to align HPUE and PC3 work items and the meeting procedurals for them.

# Conclusion

Considerations on 6G spectrum were provided with the following observations and proposals.

Observation 1: It would be premature to investigate further regulations in the 7-24 GHz frequency range in the scope of the 6G study.

Observation 2: Discuss further how to align HPUE and PC3 work items and the meeting procedurals for them.

Proposal 1: In the scope of the 6G, RAN4 should study how to handle bands in the frequency range 7-24 GHz, extending existing FR1 and/or FR2 frequency ranges and/or introducing a new frequency range (i.e. FR3), considering how RF and RRM requirements are currently specified in FR1/FR2 as input.

Proposal 2: Postpone further regulatory survey in the 7-24 GHz frequency range until Regulators published a new decision.

Proposal 3: Study how to improve and simplify selection of MSD values based on the proposed band group concept

Proposal 4: Don’t specify channel BW’s per band for 6G Inter-band combinations. but keep specific channel BW and BCS’s for intra-band combinations.

Proposal 5: Study how existing NR CA band combinations can be transferred directly into a 6G specification baseline

Proposal 6: JSON database is to be used instead of configuration tables for all 6G band combinations.

# References

- TR 38.820, Study on the 7 to 24 GHz frequency range for NR

- TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz

- TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz