# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513725

# Prague, CZ, 13-17 Oct, 2025

Agenda Item:	8.10

Source: 	Xiaomi

Title: 	Overview for 6GR spectrum sharing

Document for:	Discussion

- Introduction

In the endorsed 6G RAN4 work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. Spectrum sharing is one of the area with following candidate scope:

According to the 6G SID, 6GR target frequency includes FR1, FR2-1 and frequency range between 7GHz ~ 24GHz:

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

Compared to 5G system, the brand new clean spectrum for 6GR day 1 deployment is rare. Following spectrum sharing/co-existence scenarios are observed:

- Spectrum sharing/co-existence with legacy system (4G and 5G) on existing reframing frequency in FR1 and FR2

- 6G system co-existence with incumbent services in FR3 e.g. radar, military, and so on

In this contribution, our preliminary view on spectrm sharing is provided on following areas:

- Multi-RAT Spectrum sharig (interference handling and sync raster)

- 5G/6G Mobility (RRM mobility and measurement)

- TN-NTN spectrum sharing

- Discussion

  - Multi-RAT spectrum sharing

According to the 6G SID, following objective is dedicated for 5G NR and 6GR for spectrum migration:

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Observation 1: 6G standalone, Intra-RAT CA, MRSS (5G-6G), inter-RAT mobility between 5G and 6GR are considered as Day 1 feature for 6GR deployment.

Obseravtion 2: The inclusion of LTE/6G interworking and co-existence is FFS and subject to RAN-P decision.

Proposal 1: On spectrum sharing, focus on MRSS between 5G and 6G case in both FR1 (400MHz ~ 7.125GHz) and FR2-1 (24.25GHz ~ 52.6GHz)

Proposal 2: For below 3GHz, further discuss co-existence between 4G and 6G

On spectrum sharing, one of issue is how to handle interference between 6G and 5G/4G for always on signals:

- 6G/5G system: DSS (dynamic spectrum sharing) – PBCH and control channel, CSI-RS for L1 measurement/L3 mobility

- 6G/4G system: Sem-static spectrum sharing- PBCH, CRS (LTE) and control channel

Taking 5G DSS experience into account:

- RE-level rate-matching solution was introduced for CRS interference, and RB level rate-matching was introduced for other always on-signal e.g., PBCH.

- MBSFN and scheduled control channel under different symbols for NR and LTE to avoid interference.

The drawback of rate-matching is increased overhead for spectrum sharing with throughput loss and the UE receiver performance also degraded due to always on signal interference.

RAN4 introduced CRS-IM receiver to handle CRS interference from LTE cells (serving and neighbouring cells) with following scenario:

•	Scenario 1: Serving and interference cells are both operating in DSS (NR+LTE) mode

•	Scenario 2: Serving cell is operating in NR mode and interference cell is operating in LTE mode

| Scenario 1: | Scenario 2: |
| --- | --- |

Figure 5.1.1-1. Scenarios for CRS interference in overlapping spectrum for LTE and NR

Several schemes were evaluated on the performance impact for CRS interference handing:

| Scenario 1 | Reference scheme | Rel-15 serving cell CRS-RM without interference cell CRS handling |
| --- | --- | --- |
|  | Scheme #1 | Rel-16 CRS-RM for 1 interference cell (The rate matched CRS is always the first dominant interference) |
|  | Scheme #2 (Optional) | Rel-16 CRS-RM for 1 interference cell (The rate matched CRS is NOT always the first dominant interference. i.e. 50% probability for rate matching (RM) for the first dominant interference and 50% probability for RM for the second dominant interference) |
|  | Scheme #3 | Rel-15 RB symbol level CRS-RM for 2 interference cells |
|  | Scheme #4 | CRS-IC with network assistance |
|  | Scheme #5 | CRS-IC without network assistance |
|  | Scheme #6 | LLR weighting with network assistance |
|  | Scheme #7 | LLR weighting without network assistance |

The performance difference was summarized in below figure from the evaluation results:

For 6G co-existence with 5G and potential 4G, RAN4 can further evaluate potential solution e.g., advanced receiver and supporting in day 1.

Proposal 3: RAN4 study potential RAN4 centric solutions on handling interference between 4G/5G and 6G for always on signal e.g., control channel, PBCH and CSI-RS.

- Scenario 1: spectrum sharing between 5G/6G

- Scenario 2: 6G and 5G co-existence with neighbour cell interference

Another issue on PBCH design for initial access, how to differentiate 5G PBCH and 6G PBCH for MRSS if SSB block is similar between 5GR and 6GR and then SS raster location maybe overlapped between 5G SSB and 6G SSB. It will require additional effort for UE to discriminate 5G PBCH and 6G PBCH. Those details are pending on RAN1 design on SSB block.

Proposal 4: RAN4 needs to further evaluate sync raster design for 6GR on 5G migration bands pending on RAN1 progress

- Postpone the discuss until there is sufficient progress from RAN1 for MRSS and initial cell access i.e., no early than Q2’ 2026

  - 5G/6G Mobility

In NR, one of scenarios of inter-RAT measurement without gap is LTE CRS are fully contained within UE’s activated BWP.  For an example, in case of LTE DSS scenario the measurement on LTE CRS can be performed in gap-less way when NR UE is located in the same frequency carriers as the neighbour LTE cells. However, in 6G MRSS, when 6G UE performing the inter-RAT measurement on MRSS 5G neighbour cell, it is possible that the 5G SSB was not contained by the UE’s activaed BWP. It’s worth to further study the potentail inter-RAT measurement under MRSS sceanario.

Proposal 5: RAN4 shall study potentail inter-RAT RRM measurement impact including w/o and with gap under MRSS scenario.

  - TN-NTN spectrum sharing

Direct service from satellite to UE become popular use mode for NTN communication. “Direct-to-handset” services can provide NTN communication over TN spectrum as supplementary service e.g. TN coverage is not available.

Within ITU-R WP4C, Agenda Item 1.13 focuses on studying potential primary allocations for the Mobile-Satellite Service (MSS) within the 694 MHz – 2.7 GHz range. The objective is to enable MSS systems—particularly Direct-Connectivity MSS (DC-MSS) that link standard user equipment (UE) directly to NGSO or GSO satellites—to operate in frequency bands already harmonized for terrestrial IMT. Following the WRC-27 meeting, this scenario may also become a key focus area for 3GPP. Specifically, 3GPP would likely need to address terrestrial network (TN) and non-terrestrial network (NTN) spectrum sharing within TN frequency bands, aiming to provide additional spectrum resources for NTN systems. Within 3GPP, the relevant spectrum sharing scenario involves TN and NTN systems operating within the same or neighbouring geographic areas within a single country.

Considering the issue of spectrum scarcity in 3GPP, the 5G and 6G NTN spectrum sharing/coexistence issue in same/adjacent channel could be one of the potential issues of 6GR NTN system. This issue also caused by the limited NTN spectrum resources; satellite operators could consider the possible of using the 5G NTN spectrum in the 6G NTN system to address spectrum scarcity at least in the earlier stage of 6GR.

3GPP may also need to consider spectrum sharing for NTN with following candidate scenarios:

- NTN operation use TN bands/spectrum

- 5G NTN and 6G NTN share NTN spectrum

How to enable spectrum sharing above two scenarios including static and, dynamic and/or geographic isolation are subject to regulation update and detailed design from RAN1/RAN2 for 6G NTN design.

| illustration of TN and NTN spectrum sharing and multi-RAT NTN spectrum sharing |  |
| --- | --- |

Proposal 6: RAN4 shall study potential solutions and impact on co-channel interference handling and mobility which also related to regulation update including following scenarios:

- NTN operation use TN bands/spectrum

- 5G NTN and 6G NTN share dedicated NTN spectrum

- Conclusion

In this contribution, our preminary view on spectrum sharing are provided on potentail RAN4 impact/scope.

Observation 1: 6G standalone, Intra-RAT CA, MRSS (5G-6G), inter-RAT mobility between 5G and 6GR are considered as Day 1 feature for 6GR deployment.

Obseravtion 2: The inclusion of LTE/6G interworking and co-existence is FFS and subject to RAN-P decision.

Proposal 1: On spectrum sharing, focus on MRSS between 5G and 6G case in both FR1 (400MHz ~ 7.125GHz) and FR2-1 (24.25GHz ~ 52.6GHz)

Proposal 2: For below 3GHz, further discuss co-existence between 4G and 6G

Proposal 3: RAN4 study potential RAN4 centric solutions on handling interference between 4G/5G and 6G for always on signal e.g., control channel, PBCH and CSI-RS.

- Scenario 1: spectrum sharing between 5G/6G

- Scenario 2: 6G and 5G/4G co-existence with neighbour cell interference

Proposal 4: RAN4 needs to further evaluate sync raster design for 6GR on 5G migration bands pending on RAN1 progress

- Postpone the discuss until there is sufficient progress from RAN1 for MRSS and initial cell access i.e., no early than Q2’ 2026

Proposal 5: RAN4 shall study potentail inter-RAT RRM measurement impact including w/o and with gap under MRSS scenario.

Proposal 6: RAN4 shall study potential solutions and impact on co-channel interference handling and mobility which also related to regulation update including following scenarios:

- NTN operation use TN bands/spectrum

- 5G NTN and 6G NTN share dedicated NTN spectrum

- Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone