3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2513289

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:	8.13

Source: 	Murata Manufacturing Co Ltd.

Title: 	Discussion on general MSD calculation rule for 6G

Document for:	Discussion

Introduction

Until 5G Rel-19, MSD requirement for PC3 is derived case by case manner, which leads to increase of RAN4 workload.

In this contribution, we propose high level general MSD calculation rule for 6G.

Discussion

Current MSD analysis procedure

MSD analysis and calculation for CA/DC have been done whenever new CA/DC combinations are requested from operators, and this requires high RAN4 workload.

Mostly, several companies proposed the MSD based on their own architecture and parameters assumptions, and RAN4 determined the MSD by linearly averaging the MSD inputs from proponent companies.

Sometimes, the MSD from the existing similar band combination is reused.

On the other hand, there are some cases where the MSD is slightly different from each other even though the band combination in terms of frequency and architecture would be very similar.

To make matters worse, there are too many MSD cases in the specification documents, it is not efficient to refer or check the requirement whenever it is needed.

Therefore, general MSD calculation rule for 6G is expected to help simplify the MSD calculation as well as MSD test procedures.

New MSD analysis proposal

The MSD is highly dependent on the filter performance, linearity of RF devices, PA spectrum output and RF front end architecture. Of course, the aggressor and victim frequency play a major role determining the MSD.

The basic idea is that it might be possible to generate the general MSD equations if the above parameters are pre-defined depending on the RF front end architecture with UL and DL frequency information.

For example, same linearity performance (IIPn) and RF front end architecture can be used to derive MSD due to IMD for same band group combination as in Table 2 with band group definition in Table 1.

Table 1 Band group definition in FR1

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a (LB) | FR1-b (MB) | FR1-c (HB) | FR1-d (VHB) | FR1-e (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

Table 2 PC3 MSD due to 2 UL IMD (example)

| Aggressor 1 | Aggressor 2 | Victim | IMD order | MSD |
| --- | --- | --- | --- | --- |
| LB | LB | MB | 2 | A |
| MB | VHB | MB | 4 | B |
| MB | HB | MB | 4 | C |

Similarly, harmonic and harmonic mixing MSD can be derived as in Table 3.

Table 3 PC3 MSD due to harmonic and harmonic mixing (example)

| UL | DL | Harmonic Order | MSD |
| --- | --- | --- | --- |
| LB | MB | 3/1 | D |
| MB | VHB | 2/1 | E |
| VHB | HB | 2/3 | F |

For crossband isolation MSD, the filter attenuation and PA spectrum noise are dominant. In this case, the distance between UL-DL or fractional frequency information might be used such as LUT shown in Table 4.

Table 4 PC3 MSD due to crossband isolation (example)

| (FreqDL-FreqUL)/ (FreqDL+FreqUL) (%) | MSD |
| --- | --- |
| <1 | G |
| 1≤ <2 | H |
| 2≤ <4 | I |
| 4≤ <6 | J |
| 6≤ <8 | K |
| 8≤ <10 | L |
| ≥ 10 | M |

Conclusion

In this contribution, we proposed the general MSD rule for 6G to improve the work efficiency of RAN4.

Proposal 1: RAN4 to discuss the general MSD rule for 6G as proposed in Table 2, Table 3 and Table 4 for a starting point.

References