3GPP TSG-RAN WG4 Meeting #116bis	R4-2513290

Prague, CZ, 13th – 17th October 2025

Agenda item:	8.3

Source: 		NTT DOCOMO, INC.

Title: 	6G RAN4 Study on system parameter

Document for:	Discussion

- Introduction

In RAN4#116, the plan and meeting arrangement for 6G RAN4 study were introduced through contribution R4-2511652[1]. Starting from this meeting, discussions on 6G begin within each RAN4 topic. In this contribution, we present our view on 6G in the context of RAN4 study on system parameter.

- Discussion

According to R4-2511652[1], the scope of system parameters as 6G topics was shared. We contribute the 6G concept from each RAN4 perspective.

- Waveform and modulation

In the RP#108, it was agreed that waveform and modulation for 6G Radio (6GR) would be discussed using 5G NR as the baseline. From the status report RP#109[3], it was agreed that the study would focus on OFDM-based waveforms. Furthermore, consistent with NR, it was agreed that CP-OFDM shall be supported as mandatory for both downlink (DL) and uplink (UL), and that DFT-s-OFDM shall be included in the initial study for UL high throughput scenarios. It was also agreed that other waveform candidates shall not be excluded from consideration at this stage. The difference in waveform types is only between DL and UL. There is no difference in waveform type between FR1 and FR2; both use the same waveforms. To avoid increased complexity at the time of introduction, FR3 should basically adopt the same waveform as FR1 and FR2.

Observation 1: In 6GR, regardless of frequency band, CP-OFDM is defined as the only waveform for DL, while both CP-OFDM and DFT-s-OFDM are defined for UL. Other waveform candidates are also not excluded from consideration.

If only CP-OFDM and DFT-s-OFDM are defined by RAN1 for NR, it is likely that the NR RF requirements can be reused for 6G in RAN4. On the other hand, if waveform enhancements such as FDSS extensions are introduced, RAN4 should discuss whether there are any impacts from the following perspectives:

- EVM

- ACLR

- SEM

- MPR

- Reference sensitivity

- ACS

- Test Model

Observation 2: If only CP-OFDM and DFT-s-OFDM are defined by RAN1, NR RF requirements can likely be reused for 6G in RAN4. However, if waveform enhancements such as FDSS are introduced, RAN4 should assess potential impacts on EVM, ACLR/SEM, MPR, and reference sensitivity/ACS.

Next, we present our views on modulation. According to the status report from RP#109, the following agreements have been made for 6GR [3]:

##### Modulation, joint channel coding and modulation Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Based on the above agreements, RAN4 needs to define EVM and demodulation requirements using these as the baseline. For items where modulation requirements differ depending on device type or power class (e.g., UL 256QAM is not specified for PC3), the applicable scope for 6GR should be discussed again based on deployment scenarios.

Observation 3: RAN4 needs to define RF and demodulation requirements as based on RAN1 agreements. For some NR requirements, there are issues where the applicable scope for 6GR may need to be reconsidered depending on device type or power class.

Proposal 1: When defining new waveforms and modulations, RAN4 should specify any additional RF and demodulation requirements as necessary.

- Numerology

In 5G NR, multiple SCS patterns per band were introduced to pursue operational flexibility. However, in real network, the use of multiple SCSs has been limited, resulting in increased complexity in parameter configurations. As already discussed in RAN1 during the August meeting, for 6G, a simpler design is being considered, where only one SCS pattern per band would be introduced.

Since RAN4 needs to define RF requirements and demodulation specifications based on one or multiple SCS patterns introduced by RAN1, it is essential to proceed with discussions in coordination with RAN1, considering implementation aspects such as the impact of fading and guard band width.

Observation 4: For 6G, to achieve a simpler design compared to NR, introducing only one SCS per band is under consideration.

Proposal 2: RAN4 should develop specifications considering implementation feasibility, including the impact of fading and minimum guard band width, depending on the SCS patterns introduced.

In addition to FR1 and FR2, transmission bandwidth configuration needs to be considered for FR3 as well. For FR1 and FR2, the specifications have already been introduced in NR. If filter performance has improved compared to the early stage of 5G NR, increasing the number of transmission bandwidth configuration could potentially lead to higher throughput. However, if the number of transmission bandwidth configuration is increased in 6G, while throughput may improve, it could require changes to the filters, making it impossible to support 6G with common BS hardware and software upgrades only. From a cost perspective, as operators, we aim to introduce 6G without changing the existing 5G BS hardware, relying only on software upgrades. Therefore, any increase in transmission bandwidth configuration needs to be considered within the scope of what can be supported by software upgrades on existing BS hardware.

For FR3, since no specification has been defined yet, new considerations are required. The transmission bandwidth configuration for FR3 should be specified to maximize throughput while considering feasibility from the perspective of filter performance and windowing.

Observation 5: Increasing the Transmission Bandwidth Configuration can improve throughput; however, from an implementation perspective, challenges related to filter characteristics and windowing need to be addressed.

Proposal 2: Any increase in transmission bandwidth configuration needs to be considered within the scope of what can be supported by software upgrades on existing BS hardware for FR1 and FR2. For FR3, the transmission bandwidth configuration should be specified to maximize throughput while considering feasibility from the perspective of filter performance and windowing.

- Synchronization signal and raster

In NR, the channel raster is defined separately from the synchronization raster, and specifications for FR1 and FR2 have already been introduced. In NR, the channel raster corresponds to the minimum SCS for each FR. If synchronization raster were placed for every channel raster, the UE would need to search for SSBs across many frequency positions during cell search. Therefore, in NR, the synchronization raster was specified to be as minimal as necessary.

For 6G, the same principle should apply synchronization raster. It should be defined as sparsely as possible to reduce the UE power consumption during cell search. In addition to the NR approach, for 6G, it may be considered to either not define a synchronization raster for FR2 or define it with a much sparser configuration.

Observation 5: For 6G, synchronization raster needs to be defined as sparsely as possible to minimize UE power consumption during cell search.

Proposal 3: RAN4 should coordinate with RAN1 to ensure that synchronization raster definitions for 6G are kept minimal and consider options such as not defining synchronization raster for FR2 or defining them with a much sparser configuration compared to NR.

- Device types

Various device types were expected in 5G. They have different feasibility in terms of maximum transmit power, number of antennas, and specific features. Therefore, some requirements also differ between these devices in 3gpp spec, but for FR1 in 5G, they are differentiated primarily by suffix, and for FR2 in 5G, they are differentiated primarily by power class.

A variety of device types will be required for 6G as well. With handheld UE as the default, there are devices that can support more features (such as FWA and CPE), devices that support only limited features (such as (e)redcap), and devices with other different uses (such as NTN, Vehicular, ATG, UAV, HST). A framework in single RAT that combines both commonality and differentiation will be expected to define requirements optimized for these devices.

Observation 6: For 6G, which is more advanced than 5G, more features may be feasible on certain devices only, so it is more important to define requirements optimized for various device types.

Proposal 4: RAN4 should consider a 6G framework that combines both commonality and differentiation will be expected for different device devices.

- Conclusion

In this contribution, we offer our 6GR view as follows.

Observation 1: In 6GR, regardless of frequency band, CP-OFDM is defined as the only waveform for DL, while both CP-OFDM and DFT-s-OFDM are defined for UL. Other waveform candidates are also not excluded from consideration.

Observation 2: If only CP-OFDM and DFT-s-OFDM are defined by RAN1, NR RF requirements can likely be reused for 6G in RAN4. However, if waveform enhancements such as FDSS are introduced, RAN4 should assess potential impacts on EVM, ACLR/SEM, MPR, and reference sensitivity/ACS.

Observation 3: RAN4 needs to define RF and demodulation requirements as based on RAN1 agreements. For some NR requirements, there are issues where the applicable scope for 6GR may need to be reconsidered depending on device type or power class.

Proposal 1: When defining a new waveform, RAN4 should specify any additional RF and demodulation requirements as necessary.

Observation 4: For 6G, to achieve a simpler design compared to NR, introducing only one SCS per band is under consideration.

Proposal 2: RAN4 should develop specifications considering implementation feasibility, including the impact of fading and minimum guard band width, depending on the SCS patterns introduced.

Observation 5: For 6G, synchronization raster needs to be defined as sparsely as possible to minimize UE power consumption during cell search.

Proposal 3: RAN4 should coordinate with RAN1 to ensure that synchronization raster definitions for 6G are kept minimal and consider options such as not defining synchronization raster for FR2 or defining them with a much sparser configuration compared to NR.

Observation 6: For 6G, which is more advanced than 5G, more features may be feasible on certain devices only, so it is more important to define requirements optimized for various device types.

Proposal 4: RAN4 should consider a 6G framework that combines both commonality and differentiation will be expected for different device devices.

References

[1] 3GPP, R4-25116522, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple).

[2] 3GPP, RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator).

[3] 3GPP, RP-252475, “Status report for Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone