3GPP TSG-RAN WG4 Meeting #116bis	R4-2513332

# Prague, Czech, October 13th – 17th 2025

Source:	Ericsson

Title:	General view on 6G Demodulation Requirements

Agenda item:	8.8

Document for:	Discussion

# Introduction

At the TSG-RAN#109 meeting, it was agreed that both 5G-Advanced and 6G reduced UE MPR (Maximum Power Reduction) values, along with the adjusted Tx EVM (Error Vector Magnitude) requirement, will be considered as part of the 6G study [1]. RAN4 will further discuss and, if agreed, finalize the scope and other necessary details in 2025.Q4 under the 6G agenda items. In addition, it was agreed to evaluate a non-AI–based solution for 5G-Advanced with no impact on RAN1.

In RAN4#116, the study item “NR Spatial Channel Modelling (SCM) for demodulation” was just completed and a new WI/SI “NR Demodulation enhancement phase 6” will start from Rel-20 which would continue SCM study and introduce requirements with SCM. At the same time, AI/ML topics also have discussions on how to choose channel model to test the AI model performance. It shows that the importance of SCM for MIMO or AI relevant topics. The industries eagerly need a set of feasible and reliable spatial channel model to check the product demodulation performance in the lab before it is in the network.

FRC (Fixed Reference Channels) tables are widely used in demodulation specifications to indicate the detailed test signal information, but they are easy to have typos and misalignment during the CR or big CR creation. It is worthy to improve it during 6GR to reduce maintenance effort.

In this contribution, we would give general view on potential issues for demodulation work in 6GR including DPoD (Digital Post Distortion), SCM and FRC on non-AI receivers in 6G stage. The views for AI based receiver could be found in [3].

# Discussion

## 2.1	DPoD with BS demodulation

### 2.1.1	Problem description

The power amplifier (PA) is typically the most power-hungry component in a wireless transceiver. Therefore, any improvement in PA energy efficiency directly enhances the overall efficiency of radio transceivers. To increase coverage and improve energy efficiency in radio networks, it is desirable to operate PAs in a regime that maximizes output power and efficiency. However, this often requires operating in a more non-linear region. Such operation, while energy-efficient, introduces non-linear distortions that degrade link quality and make reliable signal reception at the receiver more difficult, as shown in Figure 1.

Figure 2.1-1: Power amplifier input-output characteristics

The trade-off between power efficiency and throughput remains one of the key implementation challenges in radio transmitters. Digital Pre-Distortion (DPD) is a widely used technique that applies an inverse distortion through a pre-distorter before the signal is fed into the PA, aiming to achieve an overall linear RF transmitter behavior. Implementing DPD typically requires good estimation of the PA’s nonlinear characteristics, obtained through a feedback loop from the PA output, along with signal processing. For wideband signals at higher frequency bands, the required signal processing becomes even more complex and power intensive. Moreover, in uplink (UL) communication, user equipment (UE) or mobile devices often have limited computational resources and power consumption, making the implementation of advanced DPD potentially challenging. As a result, power back-off is commonly applied to mitigate PA nonlinearity by shifting the operating point into a more linear region. However, this approach reduces UE output power and degrades the UL coverage.

PA should operate in a linear region to ensure reliable signal reception. For PA to be linear, UE needs to back-off power which affects UL coverage and energy efficiency.

Advanced DPD can be energy- and computation-intensive for UEs, making it a potentially challenging solution for UL transmissions to avoid power back-off.

Proposal 1	RAN4 to clarify the extent to which 6G UEs may support digital pre-distortion.

Apart from the PA, several UE RF front-end components can introduce non-linearities that degrade UL performance, including the LNA, oscillator, IQ modulator/demodulator, mixers, and duplexers. Additionally Crest Factor Reduction (CFR), if exists, applied to limit PAPR, can introduce in-band distortions. These components can give rise to additional impairments such as phase noise and IQ imbalance.

### 2.1.2	Proposed technical solutions

To address the issue described in the previous section, Digital Post-Distortion (DPoD) provides a solution for compensating UE RF front-end nonlinearity. DPoD techniques can be applied at the BS receiver, where the receiver reconstructs the transmitted signal by modeling and compensating the nonlinear distortion introduced by the UE’s non-linear components, as shown in Figure 2. These methods require knowledge of the UE nonlinearity model (for example, PA model) at the BS receiver. Further, BS has more computational resources and energy availability to compensate this.

Figure 2.1-2: UL Post-Distortion Concept

Proposal 2	DPoD at the BS receiver offers solution to compensate UE RF non-linearity to reduce UEs MPR for higher order and thus improve UL high data rate availability.

Further, the usefulness of a DPoD depends on what limits the UE’s PA linearity. If the primary limitation comes from in-channel distortion caused by PA non-linearities, then DPoD could help mitigate the issue by compensating for the distortion at the receiver side. However, DPoD cannot correct the unwanted emissions, such as out-of-band radiation and spurious emissions that impact neighboring channels.

The practical gain of DPoD is limited to in-channel distortion and not unwanted emissions.

A simple solution would be to adjust the EVM requirements for the UE on in-band distortion in the presence of DPoD, thereby reducing the needed UE MPR. However, RAN4 should evaluate not only the limit to which EVM can be adjusted, but also whether there are dependencies affecting this limit, such as resource block (RB) allocation or operating frequency, and whether the extent to which EVM can be adjusted varies depending on the UE implementation. Additionally, if there are additional UE requirements related to the UE transmit signal, while ensuring that the UE remains compliant with ACLR and SEM specifications. Further, RAN4 should avoid adding additional margins in the BS test to incorporate non-linearities or impairments in the UE. This would ensure strict RAN4 performance requirements while keeping the baseline of creating minimum requirements that can be passed by both UE and BS.

For well performing algorithm based DPoD, RAN4 needs to consider relevant model(s) for UE transmitter considering various impairments beside the PA non-linearities to ensure if the EVM is adjusted the distortion can be compensated with DPoD for not affecting the network performance.

Proposal 3	RAN4 could consider adjusting the UE EVM requirements but also evaluate any additional constraints or dependencies affecting this limit.

The wide variety of UE RF front-end models in the market could make it challenging to evaluate specific models or to define RAN4 performance requirements and test parameters accordingly.

In addition, there is a need to revisit the UE EVM measurement procedure and corresponding equalizer to resemble the algorithmic DPoD and thus ensure that the UE transmitter behavior with adjusted EVM can be compensated in the BS DPoD receiver. This aspect should be considered if the study triggers normative changes in the UE requirements.

The current UE EVM measurement procedure and corresponding equalizer should be revisited to fully capture transmitter behavior modeled in the algorithmic DPoD and if UE requirements are modified.

Furthermore, the proposed uplink post-distortion solution should be strictly network-controlled. In other words, the UE may only adjust its EVM requirement and apply MPR reduction when explicitly signaled by the network. In the absence of such signaling, the UE must continue to comply with the existing RF requirements.

Proposal 4	UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

### 2.1.3	Performance benefits

In this section, we present our initial evaluation of digital post-correction (DPC) with uplink throughput results plotted against the transmitted SNR. The system parameters used for the simulations are given in the Appendix.

Figure 2.1-3: Comparison between digital post-correction and no correction in the presence of PA non-linearities

From our initial evaluation in Figure 3, we observed a good potential with DPC, and the non-linearities (NL) of the UE PA can be compensated at the BS. In Figure 1,{20, 22} dBm power refers to a BS receiver compensating for the distortions introduced by a UE PA operating with {10, 8} dB power-backoff, respectively. Further, the key performance gains from the DPoD are observed in the high SNR region, where 64/256-QAM is typically used. This highlights the effectiveness in handling high-order modulation under non-ideal transmitter conditions.

UL post-distortion may have potential to achieve good performance without UE power-backoff, enabling potentially higher UE output power and more power-efficient operation for higher order modulation.

However, the UL PA is not the only source of non-linearity in the UE transmitter. Other RF impairments, such as oscillator phase noise and IQ imbalance, can also have a significant impact on performance. Therefore, we emphasis that it is important for RAN4 to consider the entire UE front-end RF chain when evaluating non-linear components and to investigate the applicability of post-distortion techniques for compensating these impairments.

Uplink performance is impacted not only by PA non-linearities but also by other RF impairments (e.g., oscillator phase noise, IQ imbalance), requiring a holistic view of the UE front-end.

Proposal 5	RAN4 should study UL post-distortion techniques that jointly compensate for multiple non-linear components in the UE RF chain to improve uplink performance and efficiency.

### 2.1.4	Benefit of the proposed solution

With the UL post-distortion (DPoD) feature, UEs can compensate for PA non-linearities more effectively, improving distortion tolerance. As a result, the required UE MPR can be reduced, allowing UEs to transmit at higher effective power without violating linearity requirements. This has potentially two key benefits:

- Increased UL coverage: Higher transmit power extends the uplink range for higher order modulations.

- Improved UE energy efficiency: Lower MPR reduces the back-off from PA saturation, enabling the UE to operate more efficiently, possibly reducing power consumption.

Proposal 6	RAN4 to evaluate the potential MPR reduction enabled by DPoD and its impact on UL coverage and UE transmit energy efficiency.

In order to properly evaluate the potential MPR reduction, an up-to-date UE and MPR modelling approach may be needed to capture the true benefits.

Current UL EVM requirements are specified assuming that the UE transmitter operates within the linear region of the PA, where distortions are minimal. With UL post-distortion (DPoD) applied at the base station, much of the PA non-linearity can be compensated in the network. This could enable the adjustment of UE EVM requirements but would require changes in EVM measurement procedure and equalizer by inclusion DPoD receiver.

Proposal 7	RAN4 to evaluate the extent to which UE EVM requirements can be adjusted when DPoD compensates for PA and other RF components non-linearity at the base station.

The current UE EVM requirements and EVM measurement procedure including equalizer may need updating to account for factors such as the UE front-end characteristics and the interference environment, in order to more accurately reflect real-world performance.

### 2.1.5	Potential study areas for this feature

There are several open issues to be addressed, in order to study and eventually define RAN4 core and performance requirements.

- UE RF front-end with associated non-linearities: For UL post-distortion at the BS receiver to function effectively, accurate modeling of the UE RF front-end transmitter is essential, with emphasis on the PA model and other non-linear RF elements such as oscillators, mixers etc. Further, PA non-linearity varies with frequency and is influenced by real-world impairments such as phase noise, LO leakage, IQ imbalance, EVM variation across subcarriers, and inter-symbol interference, as well as by temperature, operating frequency, and load mismatch. Also, CFR, when applied to limit PAPR, can also introduce in-band distortion.

Furthermore, different UEs are expected to have different RF front-end behaviors and DPD implementations. Therefore, it is critical to establish the variation that could be expected between different UEs.​

Proposal 8	Beyond just the PA model, the entire UE RF front-end needs to be studied by RAN4, with particular attention to the potential variation in impairments across different UEs.

In 3GPP RAN4 document [2], two polynomial PA models are described: two primary polynomial models are discussed for characterizing power amplifier (PA) nonlinearity: memoryless and memory polynomial models.

- Memoryless Polynomial Model: This model assumes that the PA's output at any given time depends solely on the instantaneous input, neglecting any past input values. While computationally efficient, it may not accurately capture the dynamic behaviors of real-world PAs, especially under varying operating conditions.

- Memory Polynomial Model: In contrast, the memory polynomial model incorporates past input values, effectively modeling the memory effects inherent in PAs. These memory effects arise due to factors like thermal dynamics, biasing, and parasitic capacitances, leading to nonlinear behaviors that depend on both current and past inputs. By including these memory terms, the memory polynomial model provides a more accurate representation of PA behavior.

Both memoryless and memory polynomial PA models can be considered in RAN4 for initial studies and evaluations.

- Estimation for UE non-linearity: For UL post-distortion feature, UE non-linearities must be estimated at the BS receiver. For example, the BS can implement an inverse polynomial of the PA model used by the UE to compensate the non-linearities introduced by the PA. However, this might be a preliminary approach and similarly other non-linearities from UE RF front-end should be estimated.

This can be done with reference signals or transmitted data from the UE. Reference signals can make it easier to isolate and measure specific non-linear behaviors such as PA distortion, IQ imbalance, and phase noise. Data-based estimation, on the other hand, captures non-linear effects under realistic traffic patterns and modulation schemes, providing insight into the UE’s performance in practical operating conditions. Both approaches can give comprehensive understanding of UE transmitter impairments and help in designing effective post-distortion techniques.

Proposal 9	RAN4 to study feasibility of UE non-linearity estimation methods with reference signals or actual data and assess their suitability for supporting post-distortion and compensation techniques.

- Impact of phase noise and IQ imbalance: Phase noise (PN) refers to rapid, short-term fluctuations in the phase of a signal. It originates from radio frontend components, such as oscillators, phase-locked loops (PLLs), mixers and amplifiers. In 3GPP TR 38.803, two phase noise models: Example 1 and Example 2 are provided for initial evaluation. Further, phase noise can degrade uplink performance typically with high modulation for both FR1 and FR2. Therefore, PN-induced impact should be studied.

IQ imbalance is an RF impairment that occurs in the in-phase (I) and quadrature (Q) signal paths of a transmitter or receiver. Ideally, the I and Q branches should have equal gain and a perfect 90° phase difference. In practice, mismatches cause:

- Gain imbalance: One branch (I or Q) has slightly higher or lower amplitude.

- Phase imbalance: The phase shift between I and Q deviates from the ideal 90°.

These imbalances distort the constellation, and degrading metrics such as EVM and ACLR. It’s especially important in high-order modulations (e.g., 256QAM) where even small imbalances can significantly impact performance.

Proposal 10	RAN4 to study inclusion of oscillator phase noise and IQ imbalance in UL post-distortion schemes, evaluating feasibility and performance gains for high-order modulations and higher-frequency operation.

- UE RF Front-End Variations: For UL post-distortion studies, it is important to recognize the UE dependency of RF front-end characteristics. UE’s performance is shaped by multiple non-linear components such as the PA, oscillator, and IQ modulator, and in practice, different UEs on the market exhibit varying front-end designs and impairment levels. This variability makes it challenging to define standardized baseline or reference models that are both representative and practical. RAN4 should therefore account for this diversity when developing study assumptions and evaluating compensation techniques.

Proposal 11	RAN4 to evaluate RF front-end variation from multiple UEs in the market into account and define suitable baseline/reference models for UL post-distortion studies.

This can be handled by introducing new EVM measurement procedure with updated equalizer taking to account the DPoD receiver.

### 2.1.6	BS demodulation impact

To define base station performance requirements for this feature, RAN4 should consider several aspects such as:

- Frequency range: The study should cover both FR1 and FR2. Higher PA non-linearity is generally expected at higher frequencies, but this depends on PA type, device design and operating conditions. At higher frequencies additional impairment sources become important (oscillator/phase-noise, PA nonlinearities interacting with phase noise and DAC quantization/noise).

Proposal 12	RAN4 to evaluate UE PA non-linearity and related impairments across both FR1 and FR2 for UL-Post distortion compensation at BS receiver. Further, this would impact both BS and UE model.

- Waveform: As agreed in RAN4#116, CP-OFDM and DFT-s-OFDM are supported for UL transmission. For initial evaluation, both waveforms can be studied.

Proposal 13	 RAN4 to study CP-OFDM and DFT-s-OFDM for UL evaluation of this feature.

- Channel Model: As the DPoD feature aims to reduce MPR and allow UEs to transmit at higher power, it is important to evaluate which channel models are most suitable for the selected test metrics.

Proposal 14	RAN4 to evaluate channel models for link-level simulation with DPoD feature, considering their impact on test metrics under higher UE transmit power.

- Modulation: Based on initial evaluation, key performance gains from AI/ML DPoD are observed in the high-SNR region, where higher-order modulations such as 256-QAM are typically applied. Therefore, it is feasible to evaluate the feature with higher modulation orders, e.g., 64-QAM and 256-QAM.

Proposal 15	RAN4 to evaluate DPoD performance with higher modulation orders focusing on high-SNR conditions.

## 2.2	SCM for UE/BS demodulation in 6G

### 2.2.1	Possible progress of SCM in 5G

In Rel-19 SI “NR SCM for demodulation”, companies tried to derive spatial channel models which could show channel impact on receiver performance by different angular path from BS to UE and also can get good alignment among active companies. From the study conclusion, both CDL-based (rCDL-C1) and TDL-based (xTDL-C1) spatial models could achieve above expectations more or less depending on specified cases. In RAN#109, it is agreed that rCDL model is considered for Rel-20 SCM WI and SI [4, 5].

Based on the [4] even though there are open issues left, it could be possible to apply rCDL model for certain legacy 5G demodulation cases, such as SU-MIMO PDSCH, to have new requirements, and also continue the MU-MIMO SCM methodology study [5].

The study on SCM for demodulation is quite complicated regarding many implementation issues, but the companies get clearer understanding on CDL model derivation, impact and potential limitations. It is possible for companies to derive reasonable CDL models for typical TN environments regarding test case conditions. It can be foreseen that the methodology of CDL derivation for FR1 non-AI based receiver demodulation performance can achieve a relatively mature stage.

However, there could be still some complicated but important issues that might not be fixed in 5G-A stage due to quite large scope of SCM in Rel-20, for example if companies couldn’t derive a feasible methodology for CDL beam steering in CSI tests in Rel-20 period, and they should be continuously studied in 6G stage since it could also impact the CDL application on AI-based receiver.

The progress in Rel-20 would impact the following CDL models used in stage 6G.

Proposal 16	If there are important issues that are not treated in 5G-A stage, they should be captured in 6G study.

### 2.2.2	SCM for UE/BS demodulation in 6G non-AI receiver

The non-AI receiver demodulation requirements with CDL models would follow the corresponding study progress after Rel-19 and Rel-20. According to current SID and WID, there are following potential areas could be considered in 6GR for CDL model implementation. The first one is the new frequency range for such as FR2 and 7 – 15 GHz. The second one is the UL CDL models for BS demodulation.

FR2 CDL model for UE demodulation requirements

FR2 is mentioned in Rel-19 SID but finally got removed due to very tight timeline. In Rel-20, FR2 is only mentioned in MU-MIMO SID, but the FR2 CDL model derivation for SU-MIMO might need be started at the first. Regarding the timeline in Rel-20, it is also high risk to complete study on both FR1 and FR2. In that case, we would believe the study on FR2 is more reasonable in 6GR.

After FR1 study, the basic procedures for CDL-based SCM derivation could be used, but following issues and parameters need more evaluations regarding they are quite different from FR1 condition.

- The reference scenarios for FR2 could be Umi and indoor office, then corresponding angular relevant parameters are different from FR1. It could be easy to handle if we follow similar procedures in FR1.

- The FR2 BS antenna would be much larger than FR1 regarding more antenna elements could be used. Furthermore, multiple panels are also typical for FR2 BS. The analog beamforming impact should be considered.

- For FR2, the impact might be higher due to FR2 UE also have many potential different antenna sizes, types and architectures. It could be quite challenging to define proper channel model to cover many UE variants.

- For OTA conformance tests, only 2 antennas can be tested for now. It would be less sensitive to spatial richness and only support up to 2 layers transmission. In 38.827, the derived FR2 CDL models are applied for MIMO OTA tests, and the demodulation part can be partially tested under it. Unfortunately, it seems not all companies involved in the study could reshow similar results at the end of the study. Applying CDL model for FR2 demodulation seems not as attractive as for FR1. On the other hand, it could be a strange situation that FR1 and FR2 use different channel model for demodulation in 6GR if no FR2 CDL model is considered.

Regarding the complexities above, RAN4 could furtherly study the benefit of derivation CDL model specially for FR2 demodulations.

The FR2 BS and UE variants could have higher impact on channel model derivation than FR1.

Only up to 2 antennas can be tested for FR2 OTA which will lead to CDL models are not attractive than in FR1 condition.

Proposal 17 	RAN4 to study the complexity and benefit of applying CDL model for FR2 DL demodulation.

7 – 15 GHz CDL model for UE demodulation requirements

The channel model for frequency range 7 – 24 GHz is investigated in Rel-19 RAN1 SI and the new propagation model parameter values and procedures are updated, and a new scenario SMa (Suburban Macro) is added to TR38.901 v19.0.0 for up to 24GHz. The RF discussion would focus on 7 – 15 GHz spectrum range in 6GR.

The basic CDL-based SCM derivation procedures could follow FR1 and/or FR2, but no reference in TR 38.827. Thus, the desired AS, DS, and random reduction need further evaluation. The BS antenna type could depend on assumption in RF discussion. It needs more discussions on how beamforming (e.g., hybrid beamforming) impacts the model derivation.

How the conformance tests would be like in this new frequency range is still unclear. The corresponding testability could also impact the channel model derivation and implementation. Studying in 6GR is necessary in that case.

The new propagation parameter values and scenario for frequency range 7 – 24 GHz is introduced to TR38.901 v19.0.0 and RF would focus on 7 – 15GHz spectrum in 6G stage.

There is no reference in previous release on key parameters configuration and randomness reduction.

Proposal 18	Setup study on how to derive DL CDL channel model for frequency range 7~15GHz demodulation.

UL CDL model derivation for BS demodulation

The UL SCM model is mentioned in Rel-19 and Rel-20 SID, but it is not studied in Rel-19 since the timeline is very tight even only DL is considered. It could be noted that the BS antenna variants are much more than UE side, so it is not straightforward to reuse DL CDL (based on a typical BS antenna configuration) for UL direction. Regarding the timeline and study scope in Rel-20, it would be feasible to consider UL CDL derivation study in 6G stage.

It should be emphasized that the spatial channel model is also important for BS product performance verifications, but the complexity of BS types should also be noted. BS side is not same as UE side that a typical handheld UE might connect to different BS types and only need to consider the worst case (e.g., UMa) for demodulation performance. For BS side, different BS capabilities might be implemented due to different deployment scenarios (UMa, UMi, InO, etc.,) and antenna types (ULA, UPA, etc.), then different BS types and antennas would experience quite different channels in real network. It is necessary to consider different channel models regarding these differences.

Current demodulation requirements are only defined up to 4Tx8Rx but the actual BS antenna (e.g., AAS) contains much more antenna ports. If there is no proper methodology but just consider one-to-one mapping from physical antenna port to virtual antenna port, it would reduce the corresponding spatial domain resolution in the derived model and impact the performance requirement definition. The benefit of using CDL models would be limited in that case. This happened in Rel-19 DL rCDL model derivation either (e.g., on eType-II codebook), and further improvement should be considered in Rel-20 or 6GR. Once the DL side solves the issue, UL side might follow, but still, it should be discussed case by case.

Proposal 19	As the lower priority comparing to DL side, RAN4 to study the how to derive proper CDL models for UL demodulation performance regarding to different BS types, antenna variants and capabilities in 6G stage.

## 2.3	Fixed Reference Channels (FRC) table

We are spending a lot of meeting time discussing the correction of FRCs in the maintenance of LTE/NR demodulation requirements. In our experience, the reasons of maintenance CRs are

- Correction of reference channel numbering because of typo or overlapping with another FRC. For BS demodulation requirements, RAN4 had many correction CRs for mismatch between different BS specs such as TS 38.104 and TS 38.141-1/2.

- Correction of values (e.g., transport block size, channel bits, maximum throughput) because of the misunderstanding of RAN1 specification or scheduling.

- Correction of format

We think RAN4 still needs to specify FRC in the specification to ensure the TE vendors can develop the common test environment, but RAN4 should discuss the FRC format to make FRC more flexible and minimize the maintenance workload. Since we expect 6GR to utilize more on-demand and flexible signal scheduling, if we keep the same format in 6GR. FRC could be more complicated. For example, we are not sure the number of code blocks and CRC size are needed for FRC.

Especially for BS demodulation, we have same FRC in many specs such as TS 38.104, TS 38.141-1/2, TS 38.174, TS 38.176-1/2. RAN4 needs to discuss the way to avoid copying the same FRC table in many specs, e.g., considering one dedicated specification.

One of the reasons for many maintenance CRs could be the lack of description on FRC table, e.g., how to derive the maximum throughput, and the fourth decimal place is rounded or truncated. RAN4 should describe the detailed procedure on how to calculate FRC, as RAN4 has described the simplified delay procedure generation in TS 38.101-4 B.2.1.

Another reason we are proposing revisiting FRC table is because RAN plenary has SI on modernization of specification format and procedures for 6G, where RAN, SA, and CT groups are discussing a new format for 3GPP technical specifications. According to the conclusion, the exiting table format used in FRC may not be suitable for the specification format. Therefore RAN4 needs to discuss how to specify the FRC table including the format.

Proposal 20	RAN4 needs to discuss how to specify FRC table in the specification for both BS and UE demodulation performance, considering the discussion in SI modernization of specification format and procedures for 6G.

# Conclusions

In the previous sections we made the following observations:

DPoD with BS demodulation

Observation 1	PA should operate in a linear region to ensure reliable signal reception. For PA to be linear, UE needs to back-off power which affects UL coverage and energy efficiency.

Observation 2	Advanced DPD can be energy- and computation-intensive for UEs, making it a potentially challenging solution for UL transmissions to avoid power back-off.

Observation 3	The practical gain of DPoD is limited to in-channel distortion and not unwanted emissions.

Observation 4	The wide variety of UE RF front-end models in the market could make it challenging to evaluate specific models or to define RAN4 performance requirements and test parameters accordingly.

Observation 5	The current UE EVM measurement procedure and corresponding equalizer should be revisited to fully capture transmitter behavior modeled in the algorithmic DPoD and if UE requirements are modified.

Observation 6	UL post-distortion may have potential to achieve good performance without UE power-backoff, enabling potentially higher UE output power and more power-efficient operation for higher order modulation.

Observation 7	Uplink performance is impacted not only by PA non-linearities but also by other RF impairments (e.g., oscillator phase noise, IQ imbalance), requiring a holistic view of the UE front-end.

Observation 8	In order to properly evaluate the potential MPR reduction, an up-to-date UE and MPR modelling approach may be needed to capture the true benefits.

Observation 9	The current UE EVM requirements and EVM measurement procedure including equalizer may need updating to account for factors such as the UE front-end characteristics and the interference environment, in order to more accurately reflect real-world performance.

Observation 10	Both memoryless and memory polynomial PA models can be considered in RAN4 for initial studies and evaluations.

SCM for UE/BS demodulation

Observation 11	The progress in Rel-20 would impact the following CDL models used in stage 6G.

Observation 12	The FR2 BS and UE variants could have higher impact on channel model derivation than FR1.

Observation 13	Only up to 2 antennas can be tested for FR2 OTA which will lead to CDL models are not attractive than in FR1 condition.

Observation 14	The new propagation parameter values and scenario for frequency range 7 – 24 GHz is introduced to TR38.901 v19.0.0 and RF would focus on 7 – 15GHz spectrum in 6G stage.

Observation 15	There is no reference in previous release on key parameters configuration and randomness reduction.

Based on the discussion in the previous sections we propose the following:

DPoD with BS demodulation

Proposal 1	RAN4 to clarify the extent to which 6G UEs may support digital pre-distortion.

Proposal 2	DPoD at the BS receiver offers solution to compensate UE RF non-linearity to reduce UEs MPR for higher order and thus improve UL high data rate availability.

Proposal 3	RAN4 could consider adjusting the UE EVM requirements but also evaluate any additional constraints or dependencies affecting this limit.

Proposal 4	UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

Proposal 5	RAN4 should study UL post-distortion techniques that jointly compensate for multiple non-linear components in the UE RF chain to improve uplink performance and efficiency.

Proposal 6	RAN4 to evaluate the potential MPR reduction enabled by DPoD and its impact on UL coverage and UE transmit energy efficiency.

Proposal 7	RAN4 to evaluate the extent to which UE EVM requirements can be adjusted when DPoD compensates for PA and other RF components non-linearity at the base station.

Proposal 8	Beyond just the PA model, the entire UE RF front-end needs to be studied by RAN4, with particular attention to the potential variation in impairments across different UEs.

Proposal 9	RAN4 to study feasibility of UE non-linearity estimation methods with reference signals or actual data and assess their suitability for supporting post-distortion and compensation techniques.

Proposal 10	RAN4 to study inclusion of oscillator phase noise and IQ imbalance in UL post-distortion schemes, evaluating feasibility and performance gains for high-order modulations and higher-frequency operation.

Proposal 11	RAN4 to evaluate RF front-end variation from multiple UEs in the market into account and define suitable baseline/reference models for UL post-distortion studies.

Proposal 12	RAN4 to evaluate UE PA non-linearity and related impairments across both FR1 and FR2 for UL-Post distortion compensation at BS receiver. Further, this would impact both BS and UE model.

Proposal 13	 RAN4 to study CP-OFDM and DFT-s-OFDM for UL evaluation of this feature.

Proposal 14	RAN4 to evaluate channel models for link-level simulation with DPoD feature, considering their impact on test metrics under higher UE transmit power.

Proposal 15	RAN4 to evaluate DPoD performance with higher modulation orders focusing on high-SNR conditions.

SCM for UE/BS demodulation

Proposal 16	If there are important issues that are not treated in 5G-A stage, they should be captured in 6G study.

Proposal 17 	RAN4 to study the complexity and benefit of applying CDL model for FR2 DL demodulation.

Proposal 18	Setup study on how to derive DL CDL channel model for frequency range 7~15GHz demodulation.

Proposal 19	As the lower priority comparing to DL side, RAN4 to study the how to derive proper CDL models for UL demodulation performance regarding to different BS types, antenna variants and capabilities in 6G stage.

FRC table for demodulation

Proposal 20	RAN4 needs to discuss how to specify FRC table in the specification for both BS and UE demodulation performance, considering the discussion in SI modernization of specification format and procedures for 6G.

# References

[1]	RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

[2]	R4-165901, Further elaboration on PA models for NR, 3GPP TSG-RAN WG4 #80, Ericsson

[3] 	R4-2513060, RAN4 study on 6G AI/ML, Ericsson

[4]	RP-252957, WID: NR demodulation performance: Phase 6, China Telecomm, BT

[5]	RP-252956, SID: Study on spatial channel model for demodulation performance requirements for NR: Phase 2, Nokia

# Appendix

Table 1: System Parameters for link-level simulation of UL Post-distortion

| Parameters | Value |
| --- | --- |
| Direction | Uplink |
| Number of UEs | 1 |
| Channel | AWGN |
| Carrier frequency | 3.55 GHz |
| Bandwidth | 40 MHz |
| # of UL PRBs | 106 PRBs (1272 subcarriers) |
| MCS | 27 in Table 2 (256QAM) |
| Number of Tx antenna (UE) | 1 |
| Number of Rx antenna (BS) | 2 |
| Sub-carrier spacing | 30KHz |