3GPP TSG-RAN WG4 Meeting #116bis	R4-2513351

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	8.3

Source:            	Google

Title:                	Discussion on 6G system parameters

Document for:	Approval

# Introduction

In RAN#109, the latest revised SID for Study on 6G Radio was approved as one of Rel-20 study items [1]. The followings are the objectives quoted from the revised SID. In this contribution, we will focus on the 6GR minimum channel bandwidth and diverse device types related discussion.

- 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

In RAN1#122 and RAN#109, the minimum spectrum allocation and smallest maximum supported UE BW for 6GR has been discussed and the remaining issue is captured in the FL summary [2]. In this paper, we would like to share our view in the following.

Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. FFS: whether RF and BB UE BW are same or different Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

# Discussion

Minimum UE channel bandwidth

In the latest 5G NR TS 38.101-1, the UE channel bandwidth is defined by maximum transmission bandwidth in RBs in the following table. In the NR beginning stage Rel-15, only 5 MHz is the minimum UE channel bandwidth. However, due to the request from public safety and railway application, 3 MHz is approved to be introduced in Rel-18 as the minimum UE channel bandwidth.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

In RAN#109, a key topic of discussion is the smallest maximum supported bandwidth or minimum spectrum allocation for low-tier devices. Considering that there is already a request from market demand in Rel-18, we believe it is also reasonable to reuse 3 MHz as the baseline for 6GR minimum UE channel bandwidth. The exact N_RB number for maximum transmission bandwidth configuration should depend on the outcome of minimum guard band discussion. Hence, it is proposed to adopt 3MHz as the baseline for 6GR minimum UE channel bandwidth and the maximum transmission bandwidth configuration can be further discussed based on the outcome of minimum guard band.

Proposal 1: It is proposed that RAN4 can adopt 3MHz as the baseline for 6GR minimum UE channel bandwidth and the maximum transmission bandwidth configuration can be further discussed based on the outcome of minimum guard band.

Device types

In 6GR SID, one objective is to study UE RF capabilities considering different device types and implementations”. In the past, the closest concept for device type may be UE categories in 4G LTE but it become overly complex structure in the later release which increased ecosystem complexity. While 5G NR adopted a more flexible capability reporting mechanism to support the concept of device type, it led to a degree of feature fragmentation, with many potential features struggling to gain widespread adoption due to a lack of clear market positioning.

Therefore, we think that the design of 6G device types should not be too complicated and may strike a balance to providing sufficient structural guidance to simplify the ecosystem while maintaining the necessary flexibility to accommodate future innovative services and applications. To manage the complexity of supporting a diverse device ecosystem, the concept for limited device types configuration sets can be considered. This product-centric framework can provide a structured basis for RAN4 to define clear and consistent sets of baseline requirements. Here are some example sets with RAN4-related key parameters for device types, e.g., IoT, Wearable, Smartphone and FWA, shown in the following.

- Type IoT Set#1:

  - Tx ANT: 1Tx

  - Rx ANT: 1Rx or 2Rx

  - MIMO layer: 1 or 2

  - Max RF bandwidth: 20MHz

  - Power Class: FDD PC3, TDD PC2

- Type Wearable Set#2:

  - Tx ANT: 1Tx

  - Rx ANT: 1Rx

  - MIMO layer: 1

  - Max RF bandwidth: 20MHz

  - Power Class: FDD PC3, TDD PC2

- Type Smartphone Set#3:

  - Tx ANT: 2Tx

  - Rx ANT: 4Rx

  - MIMO layer: 4

  - Max RF bandwidth: 200MHz

  - Power Class: FDD PC3, TDD PC2

- Type FWA Set#4:

  - Tx ANT: 1-4Tx

  - Rx ANT: 4-8Rx

  - MIMO layer: 4-8

  - Max RF bandwidth: 400MHz

  - Power Class: FDD PC1, TDD PC1.5

In RAN#109, numerous companies shared views on the need for a structured approach, highlighting the lessons learned and the urgency of providing clear guidance for device types. Hence, in our view, since IoT/Wearable/Smartphone/FWA

are more prevalent in the current market, it is proposed to include at least IoT/Wearable/Smartphone/FWA as the initial set of 6G device types for RAN4 requirements discussion.

Proposal 2: It is proposed to first include at least IoT/Wearable/Smartphone/FWA as the initial set of 6G device types and other detailed device type configuration sets can be further discussed.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: It is proposed that RAN4 can adopt 3MHz as the baseline for 6GR minimum UE channel bandwidth and the maximum transmission bandwidth configuration can be further discussed based on the outcome of minimum guard band.

Proposal 2: It is proposed to first include at least IoT/Wearable/Smartphone/FWA as the initial set of 6G device types and other detailed device type configuration sets can be further discussed.

# Reference

- RP-251816, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109

- RP-252873, “Summary of 6G Diverse Device Types (including Minimum UE Bandwidth)”, Apple Inc, RAN#109