3GPP RAN4 Meeting #116bis		R4-2513352

Prague, Czech Republic, October 13-17, 2025

Agenda item:	8.11

Source:	Apple

Title:	Initial views on RF coexistence scenarios for 6G sensing

WI/SI:	FS_6G_Radio

Release:	Rel-20

Document for:	Discussion

# 1	Introduction

The Rel-20 6G study item includes the following objective related to sensing [1]:

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

According to the RAN4 Chair’s arrangement of 6G discussions in RAN4, this item is handled as follows [2]:

This contribution provides our initial views on RF aspects related to sensing coexistence.

# 2	Discussion

The RAN1 discussions on sensing have not made much progress yet, as seen in the open issues section of the status report to RAN #109 [3].  However, agreements from the NR study of sensing can be a useful first look into the types of scenarios considered for the sensing design [4], [5]:

Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Scenario; UMa-AV, Optional RMa-AV. Carrier frequency; Mandatory: one value either 4 GHz or 4.9GHz. optional for FR1: 6 GHz [optional for FR2: 28 GHz]. System bandwidth; 100 MHz. Numerology; SCS = 30kHz. BS Layout; Hexagonal grid, 7 macro sites, 3 sectors per site. See Note1 3 sectors with 30, 150, 270 degrees. Inter-BS (2D) distance; Uma-AV: 500m, optional 1000m, RMa-AV: 1732m. BS antenna height; 25m for UMa-AV, 35m for RMa-AV. Wrap-round; No wrap-round. Note1: target(s) are dropped only in the center site, and inter-BS interference is not modelled. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Target type; UAV with small size (0.3m x 0.4m x 0.2m). Target distribution when target(s) are dropped; N = 5 targets per sector in the center site. Optional: N is uniformly distributed from 1 to 10. Horizontal plane: uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m, optionally distributed between 1.5m and 300m. Mobility; horizontal speed: uniformly distributed between 0 and 180km/h vertical speed: 0km/h. Note: N=0 will be discussed in a later proposal in relation to false alarm. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Minimum BS-target 3D distance; 10 m. Minimum target-target (3D) distance; 10 m. Target outdoor/indoor proportion; 100% outdoor. LOS/NLOS; LOS and NLOS. Orientation; Random in horizontal domain. RCS model; RCS model 1 for UAV with small size.

It is also useful to consider the technical report on channel modelling (TR38.901), which has been updated to include ISAC scenarios [6] and the LTE technical report on UAVs (TR37.777) [7].

As RAN1 continues to perform NR ISAC evaluations with metrics such as positioning accuracy and velocity accuracy, the analysis of interest for RF coexistence is how much interference such networks can create to cellular users:  both in legacy (non-ISAC) networks and in other ISAC networks.  Thus, the NR ISAC evaluation parameters that describe cell layout, user density, and propagation conditions are useful to capture for further discussion.

Observation 1:	6G ISAC system deployment parameters, such as cell layout, user density, and propagation conditions, can at least be a superset of the corresponding parameters for NR ISAC.

RF coexistence analysis starts with an understanding of the assumptions taken for the victim cellular network and the aggressor system.  We envision at least two victim systems: (1) legacy (NR) cellular network and (2) 6G cellular network.  At least examining the first set of NR ISAC agreements from RAN1, the aggressors can be an ISAC base station or an ISAC UE.

Observation 2:	For 6G ISAC coexistence analysis, RAN4 can consider NR cellular network and 6G cellular network as two potential victim systems.  The aggressors can be an ISAC base station or an ISAC UE performing sensing.

The frequency domain parameters for 6G ISAC also need to be clarified.  Without a deeper understanding of the ISAC system parameters, such as applicable frequency band(s), signal waveform, and signal bandwidth, it is difficult to make progress in modelling the mechanisms for RF interference necessary for coexistence analysis.

Observation 2:	More agreements related to the 6G ISAC system design are needed to determine the assumptions on frequency band(s), signal waveform and signal bandwidth.

Proposal 1:	RAN4 should continue to monitor the progress of NR ISAC and 6G ISAC system design to derive the necessary assumptions for RF coexistence analysis.

# 3	Conclusions

This contribution provides our initial views on 6G ISAC RF coexistence.  The following observation and proposal are made:

Observation 1:	6G ISAC system deployment parameters, such as cell layout, user density, and propagation conditions, can at least be a superset of the corresponding parameters for NR ISAC.

Observation 2:	For 6G ISAC coexistence analysis, RAN4 can consider NR cellular network and 6G cellular network as two potential victim systems.  The aggressors can be an ISAC base station or an ISAC UE performing sensing.

Observation 2:	More agreements related to the 6G ISAC system design are needed to determine the assumptions on frequency band(s), signal waveform and signal bandwidth.

Proposal 1:	RAN4 should continue to monitor the progress of NR ISAC and 6G ISAC system design to derive the necessary assumptions for RF coexistence analysis.

# 4	References

RP-252912, “Revised SID: Study on 6G Radio,” NTT DOCOMO, CMCC, AT&T, Vodafone, 3GPP RAN #109, September 2025

R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20,” RAN4 Chair, 3GPP RAN4 #116, August 2025

RP-252475, “Status report for Study on 6G Radio,” NTT DOCOMO, CMCC, AT&T, Vodafone, 3GPP RAN #109, September 2025

RP-252819, “Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR,” Xiaomi, China Telecom, 3GPP RAN #109, September 2025

RP-252200, “Status report for SI: Study on Integrated Sensing And Communication (ISAC) for NR,” China Telecom, 3GPP RAN #109, September 2025

TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”

TR36.777, “Study on Enhanced LTE Support for Aerial Vehicles”