3GPP TSG-RAN WG4 Meeting #116bis	R4-2513380

Prague, Czech, 13th -17th August, 2025

Source: 	Huawei, HiSilicon

Title:  	RF requirements for A-IoT BS and CW

Agenda Item:	4.20.1.2

Document for:	Approval

- Introduction

The core part of WI on solutions for Ambient IoT in NR has been completed at RAN#109 and TS38.194 has been released [1]. Last meeting, a WF was agreed [2]. This contribution we discuss some maintenance issues on Ambient-IoT BS.

- Discussion

  - ACLR

For the NR Medium Range BS, both a relative ACLR limit of 45/45 dB and an absolute limit of -25 dBm/MHz are defined. The less stringent of the two shall apply. During the last meeting, some companies raised concern that absolute ACLR limits also need to be defined for A-IoT BS. This was captured in the WF.

Copied from R4-2511800 [3]

Issue 1-2: ACLR

- Agreement:

  - 100kHz offset, 40/45dB

  - ACLR absolute limit can be considered during the maintenance phase

The purpose of the absolute limit is to prevent overly strict requirements at lower transmit power levels. For example, a 5MHz NR carrier at 24dBm would have a relative ACLR limit of -28dBm/MHz (24dBm-45dB=-21dBm/5M). Given that this is more stringent than the -25 dBm/MHz absolute limit, the –25 dBm/MHz requirements shall apply.

For the A-IoT R2D carrier, which has a much narrower CBW, the relative ACLR2 (45dB) are calculated in the table below:

Table 1: A-IoT BS ACLR limit

| RAT | NR | A-IoT |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW | 5MHz | 200kHz | 400kHz | 600kHz | 800kHz |
| Pout | 24dBm | 24dBm | 24dBm | 24dBm | 24dBm |
| Relative ACLR | 45dB | 45dB | 45dB | 45dB | 45dB |
| ACLR limit | -21dBm/5M | -21dBm/200k | -21dBm/400k | -21dBm/600k | -21dBm/800k |
| Bandwidth conversion factor(dB) | -7 | 7 | 4 | 2.2 | 0.97 |
| ACLR limit calculated by relative ACLR | -28dBm/M | -14dBm/M | -17dBm/M | -18.8dBm/M | -20dBm/M |

As the table shows, under lower transmit power of 24dBm, the relative ACLR are always less strict than the -25dBm/MHz absolute ACLR limit. Therefore, it's unnecessary to define an absolute ACLR limit for A-IoT BS.

Proposal 1: No need to define ACLR absolute limit for A-IoT BS.

  - Foffset for A-IoT operation

In the current TS38.194, A-IoT BS ACLR is defined with 100kHz offset. This offset, however, is not explicitly defined for other transmitter and receiver requirements.

Table 2 A-IoT BS ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | Offset(kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 200/400/600/800 | 100 | BWChannel | A-IoT of same BW | Square (BWConfig) | 40 dB |
|  | 100 | 2 x BWChannel | A-IoT of same BW | Square (BWConfig) | 45 dB |

To ensure consistency, it can introduce a common F_offset for all A-IoT BS transmitter and receiver requirements.The frequency offset Foffset values can be 100kHz+（1/2）CBW.

Proposal 2: For A-IoT operation, A-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset  as defined in Table 3 below where CBW is defined in 5.3.1 and 5.3.2 of TS38.194.

Table 3 Foffset for A-IoT operation

| Carrier | Foffset |
| --- | --- |
| Ambient IoT | 100 kHz+CBW/2 |

  - ACS

In current TS38.194, the ACS requirements are as follows:

Copied from TS38.194 [3]

- Table 7.3.1.2-1: Base station ACS requirement

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note) | -53 | ±100 | 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | PREFSENS + 6dB (Note) | -53 | ±100 | 3 MHz DFT-s-OFDM NR signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1 |  |  |  |  |

- Table 7.3.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) | Type of interfering signal |
| --- | --- | --- |
| 200 | ±100 | 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | ±100 | 3 MHz DFT-s-OFDM NR signal |

Issue 1-2: ACLR

- Agreement: （RAN4#116，R4-2511800）

  - 100kHz offset, 40/45dB

  - ACLR absolute limit can be considered during the maintenance phase

For 3.52M D2R CBW, the interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth need to correct from +/-100kHz from be +/-1500kHz.

Given that 3 MHz NR is not widely deployed, it is proposed to use a 5 MHz NR signal as the interfering source.

Proposal 3: For 3.52M D2R CBW, correct the interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth from +/-100kHz to +/-1500kHz.

- Table 7.3.1.2-1: Base station ACS requirement

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note) | -53 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | PREFSENS + 6dB (Note) | -53 | ±2500 | 5 MHz DFT-s-OFDM NR signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1 |  |  |  |  |

- Table 7.3.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (kHz) | Type of interfering signal |
| --- | --- | --- |
| 200 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | ±2500 | 5 MHz DFT-s-OFDM NR signal |

  - In-band blocking

In current TS38.194, the in-band blocking requirements are as follows:

Copied from TS38.194 [3]

Table 7.3.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + x dB | -38 | ±7.5 | 3 MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| 3520 | PREFSENS + x dB | -38 | ±4760 | 3 MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| NOTE 1: PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1 NOTE 2: For a BS capable of single band operation only, "x" is equal to 6 dB. For a BS capable of multi-band operation, "x" is equal to 6 dB in case of interfering signals that are in the in-band blocking frequency range of the operating band where the wanted signal is present or in the in-band blocking frequency range of an adjacent or overlapping operating band. For other in-band blocking frequency ranges of the interfering signal for the supported operating bands, "x" is equal to 1.4 dB. |  |  |  |  |

In TS36104，for Medium Range LTE BS，the blocking requirements are as Table 7.6.1.1-1c and Table 7.6.1.1-2.

Table 7.6.1.1-1c in TS36104: Blocking performance requirement for Medium Range BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset to the lower/higher Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18,19, 21-23, 24, 27, 30, 33-45, 48, 50, 52-54, 65, 66, 68, 70, 106 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 8, 26, 28, 111 | (FUL_low -20) | to | (FUL_high +10) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |

…

Table 7.6.1.1-2 in TS36104: Interfering signals for blocking performance requirement

| E-UTRA channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 1.4 | ±2.1 | 1.4 MHz E-UTRA signal |
| 3 | ±4.5 | 3 MHz E-UTRA signal |
| 5 | ±7.5 | 5 MHz E-UTRA signal |
| 10 | ±7.5 | 5 MHz E-UTRA signal |
| 15 | ±7.5 | 5 MHz E-UTRA signal |
| 20 | ±7.5 | 5 MHz E-UTRA signal (Note 1) |
| 20 | ±30 | 20 MHz E-UTRA signal (Note 2) |
| Note 1: This type of interfering signal is not applied for Band 46. Note 2: This type of interfering signal is only applied for Band 46. |  |  |

In TS36104，for Medium Range NB-IoT standalone，the blocking requirements are as Table 7.6.1.1-3c and Table 7.6.1.1-4.

Table 7.6.1.1-3c: Blocking performance requirement for Medium Range BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7, 11, 13-14,18,19, 21, 24, 42, 43, 48, 54, 65, 66, 70, 103, 106 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 8, 26, 28, 111 | (FUL_low -20) | to | (FUL_high +10) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |

Table 7.6.1.1-4: Interfering signals for blocking performance requirement for NB-IoT standalone operation

| NB-IoT channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| f0.2 | ±7.5 | 5MHz E-UTRA signal |

Given that 3 MHz NR is not widely deployed, it is proposed to use a 5 MHz NR signal as the interfering source.

Since interfering signal is 5M NR , the interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge should be ±7.5MHz. And A-IoT BS are single-band single carrier BS, multi-band related requirements are unnecessary.

Proposal 4: Remove multi-band related sentence for A-IoT BS blocking requirements. Correct the interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth to +/-7.5MHz.

Table 7.3.2.2-1(in TS38.194): Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6 dB | -38 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| 3520 | PREFSENS + 6 dB | -38 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |
| NOTE 1: PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1 |  |  |  |  |

  - Narrowband intermodulation

In current TS38.194, the narrowband intermodulation requirements are as follows:

Copied from TS38.194 [3]

Table 7.6.2-1: Narrowband intermodulation performance requirement for A-IoT Medium Range BS

| Channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB* | -53 | ±340 | CW |
|  |  | -53 | ±880 | 5MHz E-UTRA signal, 1 RB** |
| 3520 | PREFSENS + 6dB* | -53 | ±270 | CW |
|  |  | -53 | ±780 | 3.0 MHz E-UTRA signal, 1 RB** |
| Note*: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1. Note**: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |  |  |

Since A-IoT is belong NR, better to use NR signal rather than E-UTRA signal.

Proposal 5: Remove multi-band related sentence for A-IoT BS narrowband intermodulation requirements. Correct the type of interfering signal from 3M LTE signal to 5M NR signal.

Table 7.6.2-1: Narrowband intermodulation performance requirement for A-IoT Medium Range BS

| Channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB* | -53 | ±340 | CW |
|  |  | -53 | ±880 | 5MHz NR signal, 1 RB** |
| 3520 | PREFSENS + 6dB* | -53 | ±270 | CW |
|  |  | -53 | ±780 | 5MHz NR signal, 1 RB** |
| Note*: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1. Note**: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |  |  |

- Summary

In this contribution, we discuss some maintenance issues on Ambient-IoT BS we give our opinions of A-IoT BS requirements, and has the following observations and proposals:

Proposal 1: No need to define ACLR absolute limit for A-IoT BS.

Proposal 2: For A-IoT operation, A-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset  as defined in Table 3 below where CBW is defined in 5.3.1 and 5.3.2 of TS38.194.

Table 3 Foffset for A-IoT operation

| Carrier | Foffset |
| --- | --- |
| Ambient IoT | 100 kHz+CBW/2 |

Proposal 3: For 3.52M D2R CBW, correct the interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth from +/-100kHz to +/-1500kHz.

- Table 7.3.1.2-1: Base station ACS requirement

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note) | -53 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | PREFSENS + 6dB (Note) | -53 | ±2500 | 5 MHz DFT-s-OFDM NR signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1 |  |  |  |  |

- Table 7.3.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (kHz) | Type of interfering signal |
| --- | --- | --- |
| 200 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | ±2500 | 5 MHz DFT-s-OFDM NR signal |

Proposal 4: Remove multi-band related sentence for A-IoT BS blocking requirements. Correct the interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth to +/-7.5MHz.

Table 7.3.2.2-1(in TS38.194): Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6 dB | -38 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| 3520 | PREFSENS + 6 dB | -38 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |
| NOTE 1: PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1 |  |  |  |  |

Proposal 5: Remove multi-band related sentence for A-IoT BS narrowband intermodulation requirements. Correct the type of interfering signal from 3M LTE signal to 5M NR signal.

Table 7.6.2-1: Narrowband intermodulation performance requirement for A-IoT Medium Range BS

| Channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB* | -53 | ±340 | CW |
|  |  | -53 | ±880 | 5MHz NR signal, 1 RB** |
| 3520 | PREFSENS + 6dB* | -53 | ±270 | CW |
|  |  | -53 | ±780 | 5MHz NR signal, 1 RB** |
| Note*: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1. Note**: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |  |  |

- References

[1] RP-252745, TS 38.194 v1.0.0 NR; Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception, Huawei, RAN#109, Sept., 2025

[2] R4-2511800 WF on A-IoT BS and CW requirements, Moderator (Huawei), RAN4#116, August, 2025