3GPP TSG-RAN WG4 Meeting #116bis	R4-2513382

Prague, Czech, 13th -17th August, 2025

Source: 	Huawei, HiSilicon

Title:  	Discussion on A-IoT BS RF perf requirements

Agenda Item:	6.14.2

Document for:	Approval

- Introduction

In RAN#106 the new work item Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. This WI has been revised during RAN#107[2]. According to the work item description, the objective is to specify RF requirements for Ambient-IoT BS, device 1, and CW. The core part of WI on solutions for Ambient IoT in NR has been completed at RAN#109 and TS38.194 has been released during last RAN#109 meeting. The objective of performance part WI includes specify the necessary conformance testing for Ambient-IoT BS.

In this contribution we discuss some issues for testing A-IoT including measurement uncertainties, manufacturer declaration, new test model and test configurations that would be needed to specify A-IoT requirements tests.

- Discussion

  - general

The following tables from Table 1 to 4 summarized requirements defined for A-IoT BS and CW inTS38.194[3].

Table 1: System parameters for A-IoT

| A-IoT system parameters |  | Value | Note |
| --- | --- | --- | --- |
| Operating band |  | n8 | Refer to TS38.141-1 |
| Channel BW | R2D Channel BW | 200/400/600/800 kHz | New |
|  | D2R Channel BW | By equations and tables | New |
| R2D channel raster |  | 10 kHz | Refer to TS38.141-1 |

Table 2: A-IoT BS TX requirements

| RF Requirement for A-IoT BS- TX requirements |  | Value | Note |
| --- | --- | --- | --- |
| Base station output power |  | Prated,c,AC≤ 38 dBm | Refer to TS38.141-1 |
| Transmit ON/OFF power | Transmitter OFF power | -85dBm/MHz | Adaptation is needed |
|  | Transmitter transient period | 10µs | Adaptation is needed |
| Transmit signal quality | Frequency error | ±0.1 ppm | Refer to TS38.141-1 |
|  | Modulation quality | Refer to TS38194 | new |
| Unwanted emissions | Occupied bandwidth | less than the A-IoT R2D CBW | Adaptation is needed |
|  | ACLR | 100kHz offset, 40/45dB | Adaptation is needed |
|  | Operating band unwanted emissions | Refer to TS38194 | Adaptation is needed |
|  | Transmitter spurious emissions | Reuse the FR1 NR BS transmitter spurious emission requirement Reuse the co-existence and co-location requirements of NR Micro BS | Refer to TS38.141-1 |

Table 3: A-IoT BS RX requirements

| A-IoT BS- RX RF Requirement |  | Value | Note |
| --- | --- | --- | --- |
| Reference sensitivity level |  | CW input level to BS Antenna: -38dBm, Desens target: 30dB | new |
| Adjacent Channel Selectivity |  | Refer to 38194 | Adaptation is needed |
| In-band blocking | general blocking | Refer to 38194 | Adaptation is needed |
| Out-of-band blocking |  | Reuse from NB-IoT standalone Medium Range BS | Refer to TS38.141-1 |
| Receiver spurious emission |  | reuse the NR BS requirements | Refer to TS38.141-1 |
| Narrowband intermodulation |  | Refer to 38194 | Adaptation is needed |

Table 4: A-IoT CW requirements

| RF Requirement for CW | Value | Note |
| --- | --- | --- |
| Output power | 33dBm | Refer to TS38.141-1 |
| OFF power | -50dBm/MHz | Adaptation is needed |
| Frequency error | ±0.1 ppm | Refer to TS38.141-1 |
| Phase noise | Refer to 38194 | New |
| Unwanted emission | Refer to 38194 | New |
| Spurious emissions | Refer to 38194, ΔF = 10 MHz | Adaptation is needed |

These A-IoT BS and CW requirements in relation to NR can be categorized into three groups:

Group 1: Directly Referenced from NR:
These items share same requirements with NR and can directly reference TS 38.141-1.

- R2D Channel Raster

- BS Output Power

- Frequency Error

- Transmitter Spurious Emissions

- Receiver Spurious Emissions

Group 2: Modified from NR (Same Item, Different Value):
These items exist in NR but have different values for A-IoT due to its specific physical layer design.

- Channel BW

- ACLR

- Blocking

- OBUE

- Reference Sensitivity Level

- ACS

- General Blocking

- Narrowband Intermodulation

Group 3: New A-IoT-Specific Requirements:
These are new items not defined for NR.

- Modulation Quality

- CW Phase Noise

Observation 1: A-IoT BS and CW requirements in relation to NR can be categorized into three groups: Group 1：Directly Referenced from NR; Group 2: Modified from NR (Same Item, Different Value); Group 3: New A-IoT-Specific Requirements

  - Measurement uncertainties and test requirements

A-IoT measurement uncertainties and test requirements can refer to that of NB-IoT and NR with adaption if necessary.

Table 5: Maximum Test System uncertainty for transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Base Station output power | ±0.7 dB, f ≤ 3 GHz |  |
| 6.4.1 Transmit OFF power | ±2.0 dB, f ≤ 3 GHz |  |
| 6.4.2 Transmitter transient period | N/A |  |
| 6.5.2 Frequency error | ± 12 Hz |  |
| 6.5.3 Modulation quality | [Modulation depth：± 5% Ripple： Rise/fall time: Pulse width:] |  |
| 6.6.2 Occupied bandwidth | ±[2]kHz |  |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | ACLR ±0.8 dB |  |
| 6.6.4 Operating band unwanted emissions | ±1.5 dB, f ≤ 3 GHz |  |
| 6.6.5.5.1.1 Transmitter spurious emissions, Mandatory Requirements | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 19 GHz: ±4.0 dB 19 GHz < f ≤ 26 GHz: ±4.5 dB |  |
| 6.6.5.5.1.2 Transmitter spurious emissions, Protection of BS receiver | ±3.0 dB |  |
| 6.6.5.5.1.3 Transmitter spurious emissions, Additional spurious emission requirements | ±2.0 dB for > -60 dBm, f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 4.2 GHz ±3.0 dB, 4.2 GHz < f ≤ 7.125 GHz (Note) ±3.0 dB for ≤ -60 dBm, f ≤ 3 GHz ±3.5 dB, 3 GHz < f ≤ 4.2 GHz ±4.0 dB, 4.2 GHz < f ≤ 7.125 GHz (Note) ±4.0 dB, for bands n46 and n96 |  |
| 6.6.5.2.4 Transmitter spurious emissions, Co-location | ±3.0 dB |  |
| NOTE: Test system uncertainty values for 3 GHz < f ≤ 7.125 GHz apply for BS operates in licensed spectrum only. |  |  |

Table 6 Maximum Test System Uncertainty for receiver tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.2 Reference sensitivity level | ±0.7 dB, f ≤ 3 GHz |  |
| 7.4.1 Adjacent channel selectivity | ±1.4 dB, f ≤ 3 GHz | Overall system uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer leakage Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer leakage effect is systematic, and is added arithmetically. Test System uncertainty = SQRT (wanted_level_error2 + interferer_level_error2) + leakage effect. f ≤ 3 GHz Wanted signal level ±0.7 dB Interferer signal level ±0.7 dB |
| 7.4.2.4.2 In-band blocking (General blocking) | ±1.6 dB, f ≤ 3 GHz |  |
| 7.5.5.1 Out-of-band blocking (General requirements) | fwanted ≤ 3GHz 1MHz < finterferer ≤ 3 GHz: ±1.3 dB 3.0GHz < finterferer ≤ 4.2 GHz: ±1.5 dB 4.2GHz < finterferer ≤ 12.75 GHz: ±3.2 dB | Overall system uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = SQRT (wanted_level_error2 + interferer_level_error2) + Broadband noise effect. Out of band blocking, using CW interferer: Wanted signal level: ±0.7 dB up to 3 GHz ±1.0 dB up to 4.2 GHz ±1.22 dB up to 6 GHz ±1.5 dB up to 7.125 GHz Interferer signal level: ±1.0 dB up to 3 GHz ±1.2 dB up to 4.2 GHz ±3.0 dB up to 12.75 GHz Impact of interferer Broadband noise 0.1 dB |
| 7.5.5.2 Out-of-band blocking (Co-location requirements) | Co-location blocking, using CW interferer: ±2.5 dB, f ≤ 3.0 GHz | Co-location blocking, using CW interferer: f ≤ 3.0 GHz Wanted signal level ± 0.7 dB 3.0 GHz < f ≤ 4.2 GHz Wanted signal level ± 1.0dB 4.2 GHz < f ≤ 6.0 GHz (Note 2) Wanted signal level ± 1.22 dB 6 GHz < f ≤ 7.125 GHz Wanted signal level ± 1.5 dB For bands n46, n96 and n102 Wanted signal level ± 1.5 dB f ≤ 7.125 GHz Interferer signal level: ± 2.0 dB Interferer signal level for band n46, n96 and n102: ± 2.0 dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.4 dB |
| 7.6 Receiver spurious emissions | 30 MHz ≤ f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 19 GHz: ±4.0 dB 19 GHz < f ≤ 26 GHz: ±4.5 dB |  |
| 7.7 Receiver intermodulation | ±1.8 dB, f ≤ 3.0 GHz ±2.4 dB, 3.0 GHz < f ≤ 4.2 GHz ±3.0 dB, 4.2 GHz < f ≤ 6.0 GHz (Note 2) ±3.3 dB, 6 GHz < f ≤ 7.125 GHz ±3.3 dB, for bands n46, n96 and n102 | Overall system uncertainty comprises four quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error 4. Impact of interferer ACLR The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] + ACLR effect. f ≤ 3.0 GHz Wanted signal level ± 0.7dB CW interferer level ± 0.5 dB Mod interferer level ± 0.7 dB 3.0 GHz < f ≤ 4.2 GHz Wanted signal level ± 1.0 dB CW Interferer level ± 0.7 dB Mod Interferer level ± 1.0 dB 4.2 GHz < f ≤ 6 GHz (Note 2) Wanted signal level ± 1.22 dB CW Interferer level ± 0.98 dB Mod Interferer level ± 1.22 dB 6 GHz < f ≤ 7.125 GHz Wanted signal level ± 1.5dB CW Interferer level ± 1.0dB Mod Interferer level ± 1.5dB For bands n46, n96 and n102 Wanted signal level ± 1.5dB CW Interferer level ± 1.0dB Mod Interferer level ± 1.5dB f ≤ 7.125 GHz Impact of interferer ACLR 0.4 dB |
| NOTE 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. NOTE 2: Test system uncertainty values for 4.2 GHz < f ≤7.125 GHz apply for BS operates in licensed spectrum only. |  |  |

Proposal 1: Adopt the measurement uncertainties and test requirements in Table 5 and 6.

  - Manufacturer declaration

The following BS declarations listed in table x, when applicable to the BS under test, are required to be provided by the manufacturer for the conducted requirements testing of the A-IoT BS.

Table 7 Manufacturer declarations for A-IoT BS test requirements

| Declaration | Description | Applicability |
| --- | --- | --- |
| BS requirements set | Declaration of one of the NR base station requirement's set as defined for BS type 1-C. | x |
| Operating bands and frequency ranges | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Declarations shall be made per antenna connector for BS type 1-C | x |
| Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | x |
| Additional operating band unwanted emissions | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.6.4.5.6 apply. (Note 3, Note 6). | x |
| Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. | x |
| Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. | x |
| Maximum Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band for single-band operation. Declared per supported operating band, per antenna connector for BS type 1-C | x |
| A-IoT supported channel bandwidths and SCS | NR supported SCS and channel bandwidths per supported SCS. Declared per supported operating band, per antenna connector for BS type 1-C | x |
| Rated carrier output power (Prated,c,AC, or Prated,c,TABC) | Conducted rated carrier output power, per single band connector or multi-band connector. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. (Note 1, 2, 7) | x |
| Rated total output power (Prated,t,AC, or Prated,t,TABC) | Conducted total rated output power. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. For multi-band connectors declared for each supported operating band in each supported band combination. (Note 1, 2, 7) | x |
| Intra-system interfering signal declaration list | List of single band connector(s) or multi-band connector(s) for which an intra-system interfering signal level is required to be declared. Declaration is required if the intra-system interfering signal level is larger than the co-location interfering signal level. |  |
| Intra-system interfering signal level | The interfering signal level in dBm. Declared per supported operating band, per TAB connector for BS type 1-H covered by D.29. |  |
| Equivalent connectors | List of antenna connectors of BS type 1-C, or TAB connector of BS type 1-H, which have been declared equivalent. Equivalent connectors imply that the antenna connector of BS type 1-C, or TAB connector of BS type 1-H, are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the antenna connector of BS type 1-C, or TAB connector of BS type 1-H are identical and the transmitter unit and/or receiver unit driving the antenna connector of BS type 1-C or TAB connector of BS type 1-H are of identical design. | x |
| TAB connector RX min cell group | Declared as a group of TAB connectors to which RX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for receiving a cell when the BS type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |  |
| TAB connector TX min cell group | Declared group of TAB connectors to which TX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for transmitting a cell when the BS type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |  |
| Connecting network loss range for BS testing with ancillary RF amplifiers | Declaration of the range of connecting network losses (in dB) for BS type 1-C testing with ancillary Tx RF amplifier only, or with Rx RF amplifier only, or with combined Tx/Rx RF amplifiers. (Note 4) | x |
| TAB connectors used for performance requirement testing | To reduce test complexity, declaration of a representative (sub)set of TAB connectors to be used for performance requirement test purposes. At least one TAB connector mapped to each demodulation branch is declared. |  |

Set the power of each carrier to the same level so that the sum of the carrier powers equals the rated total output power Prated,t according to the manufacturer’s declaration in subclause 4.6.8.

Proposal 2: Adopt the manufacturer declarations for A-IoT BS test requirements in Table 7.

  - A-IoT test mode for TX

Table 1 summarized test modes for NR and NB-IoT in TS 36.141 and TS 38.141 used for Tx tests.

Table 8: Test modes for NR and NB-IoT

| TS | test mode | Used for tests on | note |
| --- | --- | --- | --- |
| 38.141-1 | FR1 test model 1.1 (NR-FR1-TM1.1) | This model shall be used for tests on: - BS output power - Transmit ON/OFF power - TAE - Unwanted emissions - Occupied bandwidth - ACLR - Operating band unwanted emissions - Transmitter spurious emissions - Transmitter intermodulation - Receiver spurious emissions | QPSK |
|  | FR1 test model 1.2 (NR-FR1-TM1.2) | This model shall be used for tests on: - Unwanted emissions - ACLR - Operating band unwanted emissions |  |
|  | FR1 test model 2 (NR-FR1-TM2) | This model shall be used for tests on: - Total power dynamic range (lower OFDM symbol TX power limit (OSTP) at min power) - EVM of single 64QAM PRB allocation (at min power) - Frequency error (at min power) | 64QAM |
|  | FR1 test model 2a (NR-FR1-TM2a) |  | 256QAM |
|  | FR1 test model 2b (NR-FR1-TM2b) |  | 1024QAM |
|  | FR1 test model 3.1 (NR-FR1-TM3.1) | This model shall be used for tests on: - Output power dynamics - Total power dynamic range (upper OFDM symbol TX power limit (OSTP) at max power with all 64QAM PRBs allocated) - Transmitted signal quality - Frequency error - EVM for 64QAM modulation (at max power) | 64QAM |
|  | FR1 test model 3.1a (NR-FR1-TM3.1a) |  | 256QAM |
|  | FR1 test model 3.1b (NR-FR1-TM3.1b) |  | 1024QAM |
|  | FR1 test model 3.2 (NR-FR1-TM3.2) | This model shall be used for tests on: - Transmitted signal quality - Frequency error - EVM for 16QAM modulation | 16QAM |
|  | FR1 test model 3.3 (NR-FR1-TM3.3) | This model shall be used for tests on: - Transmitted signal quality - Frequency error - EVM for QPSK modulation | QPSK |
|  | NB-IoT operation in NR in-band test model (NR-N-TM) |  |  |
| 36141 | NB-IoT Test Model (N-TM) |  |  |

R19 support only single band and single carrier A-IoT operation. Compared with NR, test mode can be simplified.

There are 4 kinds of R2D CBWs. All of their TX requirements need to be specified. To reduce the test cases, modulation quality can be tested for the 200kHz only.

For A-IoT, only OOK modulation is considered in R2D. R2D CBW is 200/400/600/800kHz. We would suggest defining one new test model with different RB numbers for A-IoT. This test model would be used for all tests.

Proposal 3: Defining one set test model with different RB numbers for A-IoT TX. This test model would be used for all TX tests.

  - A-IoT test mode for RX

Among the total 36 supported D2R CBWs, requirements are currently defined for 200kHz and 3520kHz D2R CBW.

The 200 kHz requirements apply for D2R CBW ≤ 200 kHz. The 3520 kHz requirements apply for 200 kHz < D2R CBW ≤ 3520 kHz. Specifications for other D2R channel bandwidths are reserved for future definition upon request.

Proposal 4: Defining one set test model with different D2R CBW for A-IoT RX. This test model would be used for all RX tests.

  - A-IoT test configuration for TX

A-IoT support only single carrier operation. Test configuration can be as follows:

The purpose of the TC1 is to test A-IoT R2D carrier aspects.

TC1 generation

TC1 is constructed using the following method:

-	The Base Station RF Bandwidth shall be the declared maximum Base Station RF Bandwidth.

-	Place a A-IoT carrier within the Base Station RF Bandwidth.

-	Set the power of A-IoT carrier to the rated output power Prated,t according to the manufacturer’s declaration.

Proposal 5: Define single carrier A-IoT test configuration for TX.

- Summary

In this contribution, we discuss some maintenance issues on Ambient-IoT BS we give our opinions of A-IoT BS requirements, and has the following observations and proposals:

Observation 1: A-IoT BS and CW requirements in relation to NR can be categorized into three groups: Group 1：Directly Referenced from NR; Group 2: Modified from NR (Same Item, Different Value); Group 3: New A-IoT-Specific Requirements.

Proposal 1: Adopt the measurement uncertainties and test requirements in Table 5 and 6.

Proposal 2: Adopt the manufacturer declarations for A-IoT BS test requirements in Table 7.

Proposal 3: Defining one set test model with different RB numbers for A-IoT TX. This test model would be used for all TX tests.

Proposal 4: Defining one set test model with different D2R CBW for A-IoT RX. This test model would be used for all RX tests.

Proposal 5: Define single carrier A-IoT test configuration for TX.

- References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc, Dec.,2024

[2]RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc, March, 2025

[3] RP-252745, TS 38.194 v1.0.0 NR; Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception, Huawei, RAN#109, Sept., 2025