3GPP TSG-RAN WG4 Meeting #116bis           	R4-2513395

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.21.4

Source:	Samsung

Title:	Discussion and simulation results on demodulation requirement for IoT NTN

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test scope for IoT NTN phase 2 was discussed, the related agreement was captured into the WF in [1] as following

- General - Requirements applying under standalone and in-band - The same requirement for IOT NTN NPUSCH applies for both standalone and in-band operation. - SAN demodulation requirements - DMRS pattern for NPUSCH 3.75kHz SCS - Sequential mapping of samples of the original DMRS sequence to active DMRS slots - Whether needs to configure the co-schedule UE - Configure the co-scheduled UE for OCC2 NPUSCH format 1 demodulation requirements. - Number of Repetition - Reuse the last meeting agreement: 2 for single tone with 3.75kHz and 8 for single tone with 15kHz. - And discuss how to capture the number of repetitions into specification during CR drafting stage. - Power assumption for two-schedule UEs - Assuming the same transmission power for two UEs. - Assuming the same receiving power for two UEs at gNB side - Test metric - Measure the NPUSCH format 1 throughput per UE. - UE 1 uses OCC sequence index 0 - UE 2 uses OCC sequence index 1 - Set the same SNR for NPUSCH format 1 from two UEs if there is no performance difference - FFS how to write the specification, define NPUSCH 1 demodulation requirement for each UE - Test Duration - Tx duration= 256ms for 15khZ - Frequency offset - 128Hz for 15khZ

In this contribution, the view on  remainig issue of test setup on  NPUSCH1 demodualtion requirement is provided. Meanwhile, the initial simulation results are provided for alignment purpose.

# 2	Discussion

In this section, the view on remaining issue of test setup for NPUSCH format 1 demodulation requirement

Number of Repetition

Regarding number of repetitions, due to the OCC operation, the actual number of transmissions will be extended.  Based on agreement in the last meeting, RAN4 will maintain the assumption of number of repetitions for requirement definition.

Agreement - 2 for single tone with 3.75kHz and 8 for single tone with 15kHz

RAN4 will further discuss how to capture the number of repetitions into the specification. Since the number of repetitions agreed in RAN4 means the number of transmissions before OCC operation, we suggest add one more parameter to indicate the length of OCC operation. Considering RAN1 has already agreed the total number of slots in NPUSCH transmission after OCC is following

Agreement The total number of slots in the NPUSCH transmission after OCC is applied is MrepNPUSCHNslotsULNRU , where the parameters have the legacy definitions in TS36.211 and MrepNPUSCH≥2.

To align with RAN1 agreement with the total number of slots in NPUSCH transmission, we think some additional clarification or note is needed as following

- Note: The total number of slots in NPUSCH transmission after transmission after OCC is applied Repetition number* OCC length *NslotsULNRU,  where NslotsUL is number of UL slot and NRU is the number of RU.

| Case | UE | Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | OCC length | Fraction of maximum throughput | SNR [dB] (ideal) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 | 2 | 70% | TBD |
|  | 1 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 | 2 | 70% | TBD |
| - Note: The total number of slots in NPUSCH transmission after transmission after OCC is applied Repetition number* OCC length *NslotsULNRU, where NslotsUL is number of UL slot and NRU is the number of RU. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1: RAN4 could configure the length of OCC operation, and add the note following when specifying the NPUSCH 1 requirement into specification

| Case | UE | Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | OCC length | Fraction of maximum throughput | SNR [dB] (ideal) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 | 2 | 70% | TBD |
|  | 1 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 | 2 | 70% | TBD |
| - Note: The total number of slots in NPUSCH transmission after transmission after OCC is applied Repetition number* OCC length *NslotsULNRU, where NslotsUL is number of UL slot and NRU is the number of RU. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 3	Simulation Results

In this section, based on agreed test setup as following table 1 and table 2 for 3.75KHz SCS and 15KHz SCS, the initial simulation results are provided for alignment purpose.

Table 1: FRC parameters for NB-IoT NPUSCH format 1, 3.75KHz SCS

| Reference channel | A7-1 |
| --- | --- |
| Subcarrier spacing (kHz) | 3.75 |
| Number of allocated subcarriers | 1 |
| Diversity | No |
| Modulation | BPSK |
| ITBS / IRU | 0 / 1 |
| Payload size (bits) | 32 |
| Allocated resource unit | 2 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.29 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 96 |
| Total symbols per resource unit | 96 |
| Channel estimation length (ms) Note 1 | 16 |

Table 2: FRC parameters for NB-IoT NPUSCH format 1, 15KHz SCS

| Reference channel | A16-2 |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Number of allocated subcarriers | 1 |
| Diversity | No |
| Modulation | BPSK |
| ITBS / IRU | 0 / 1 |
| Payload size (bits) | 32 |
| Allocated resource unit | 2 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.29 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 96 |
| Total symbols per resource unit | 96 |
| Channel estimation length (ms) Note 1 | 4 |
| Note 1: Channel estimation lengths are included in the table for information only |  |

The initial simulation results are provided for alignment purpose in table 3 and table 4 for 3.75KHz and 15KHz SCS, respectively.

Table 3:  Initial simulation results for 3.75KHz with supporting inter-symbol OCC, single tone

| Case | UE | Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput | SNR [dB] (ideal) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 4 without OCC | 70% | -3.77 |
|  | 0 | 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 4 without OCC | 70% | -7.11 |
| 1 | 0 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -2.6 |
|  | 1 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -2.58 |
| 2 | 0 | 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -5.62 |
|  | 1 | 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -5.58 |

Table 4: Initial simulation results for 15KHz with supporting inter-slot OCC, single tone

| Case | UE | Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput | SNR [dB] (ideal) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 128Hz | 256 | 16 without OCC | 70% | -8.21 |
|  | 0 | 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 128Hz | 256 | 16 without OCC | 70% | -11.36 |
| 1 | 0 | 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 128HZ | 256 | 8 before OCC | 70% | -8.48 |
|  | 1 | 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 128HZ | 256 | 8 before OCC | 70% | -8.52 |
| 2 | 0 | 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 128HZ | 256 | 8 before OCC | 70% | -11.53 |
|  | 1 | 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 128HZ | 256 | 8 before OCC | 70% | -11.54 |

For 3.75KHz SCS, there is around 1-2dB performance degradation compared with single UE without OCC operation, considering the DMRS for each UE is TDM allocation, it results in the performance degradation of channel estimation.

For 15KHz SCS, the performance degradation due to OCC operation is minor compared with single UE without OCC operation.

Meanwhile, compared with 2Rx configuration, around 3dB performance degradation was observed for 1Rx configuration.

Observation 1: For 3.75KHz SCS, with OCC operation, around 1-2dB performance degradation compared with single UE without OCC operation.

Observation 2: For 15KHz SCS, with OCC operation, minor performance degradation compared with single UE without OCC operation

Observation 3: Compared with 2Rx configuration, around 3dB performance degradation was observed for 1Rx configuration

# 4	Conclusion

In this contribution, the view on remaining issue of NPUSCH demodulation requirement with supporting inter-slot OCC. Meanwhile, the initial simulation results are provided for alignment purpose.

Proposal 1: RAN4 could configure the length of OCC operation, and add the note following when specifying the NPUSCH 1 requirement into specification

| Case | UE | Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | OCC length | Fraction of maximum throughput | SNR [dB] (ideal) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 | 2 | 70% | TBD |
|  | 1 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 | 2 | 70% | TBD |
| - Note: The total number of slots in NPUSCH transmission after transmission after OCC is applied Repetition number* OCC length *NslotsULNRU, where NslotsUL is number of UL slot and NRU is the number of RU. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation 1: For 3.75KHz SCS, with OCC operation, around 1-2dB performance degradation compared with single UE without OCC operation.

Observation 2: For 15KHz SCS, with OCC operation, minor performance degradation compared with single UE without OCC operation

Observation 3: Compared with 2Rx configuration, around 3dB performance degradation was observed for 1Rx configuration

# References

RP-250472, Revised WID on NTN for internet of Things (IoT) phase 3, MTK

R4-2512645, Way Forward for [116][332] IoT_NTN_Ph3_demod