3GPP TSG-RAN WG4 Meeting #116bis           	R4-2513397

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.15.5

Source:	Samsung

Title:	View on demodulation requirement for Ambient-IoT BS

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test scope of test setup for performance part of Ambient IoT (Internet of Things) in NR was under discussion. The related agreement was captured into WF in [1] as following

- Device requirement - General - Deployment and topology - The assumed deployment and topology D1T1-B for A-IoT demodulation requirements discussion in R19 - No need to capture it in RAN4 specification - Operation mode - Standalone and in-band operation for Device 1 - Same demodulation performance requirements for standalone and in-band operation. - How to capture it in the specification can be discussed during the CR drafting if necessary - Standalone operation for BS - Demodulation requirements - BS demodulation requirements - Whether to define PDRCH demodulation requirement for Ambient-IoT - Option 1: Define PDRCH demodulation requirement - Option 2: Not define PDRCH - D2R message type for testing - Option 1: Define PDRCH demodulation requirements with Message 1 reception - Option 2: Define PDRCH demodulation requirements with Message 3 reception - D2R message structure for testing - Option 1: Short D2R transmission without midamble with 31 bits Preamble - Option 2: Long D2R transmission with midamble inserted with 31 bits Preamble and 31 bits Midamble - Reader detection assumption - Option 1: Coherent detection - Other options are not precluded - Antenna configuration - 1T1R - Numerology - 15KHz SCS - Modulation - Option 1: Both BPSK and OOK modulation scheme - Option 2: OOK modulation - Waveform - Option 1: backscattering a carrier wave (CW) - Option 2: Unmodulated single tone (CW) - Channel coding and FEC - Channel coding with 1/3 Convolutional Coding - Block level Repetition - Option 1: 1 - Option 2: 2 - SFO assumption - Option 1: 104 ppm~105 ppm - Other options - Device sampling rate - Option 1: 1.92MHz device sampling rate - Other options - TBS - Option 1: 96 bis - Other options - CBW - Option 1: further discuss how to select the proper CBW - Option 2: postpone the channel bandwidth and (Tb, Tc, R) discussion until related core part is stable - Channel model - Option 1: TDA30-10 - Other options are not precluded - Test methodology - 10% BLER with BLER=1- Number of responsed Message 2Number of transmitted Message 1 for testing - During countering, the number of responded Message 2, the TE should check if the responded Message 2 is the correct response of Message 1. - Specification structure - Option 1: Specify the Ambient IoT BS demodulation requirements in “Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception”an“Ambient IoT Base Station (BS) and Carrier-Wave (CW) node conformance testing”if RAN4 agreed to define Ambient IoT BS demodulation requirements - Option 2: The demodulation requirements can be captured in clause 10 of TS 38.194/195.

As stated in the WID, from RAN4 perspective, RAN4 needs to specify the necessay demodualtion requirements for device 1 and Ambient-IoT BS.

In this contribution, the view on the remainning issue of test scope and test setup for demodualtion requirment for Ambient-IoT BS is provided.

# 2	General Scope

In this section, the view on remaining issue of scope of demodulation requirement for Ambient-IoT BS is provided.

FFS to define PDRCH requirement

Based on RAN1 design for A-IoT, only one physical channel is available for D2R link, as PDRCH (Physical Device to reader channel (PDRCH)), which carriers the data and control information. Due to the simple design of the device, dedicated control channel (e.g., 5G PUCCH like channel), and random-access channel (e.g,5G PRACH like channel) are not considered for A-IoT. Reference signals such as DM-RS, PTRS, SRS are considered.

For PDRCH transmission, RAN1 has designed the waveform, modulation, coding, repetition, FEC schemes to support A-IoT. The following is the processing procedure which includes CRC attachment, coding, and modulation/waveform generation where the coding including FEC, TB repetition, and line coding

Figure 1: PDRCH transmission structure

In general, considering it is a new physical layer design, RAN4 should define the related demodulation requirement for PRDCH transmission, considering it has the baseband impact.

In the last meeting, RAN4 has discussed whether to define PDRCH demodulation requirement. The main discussion is that whether it is necessary to define PDRCH demodulation requirement considering the PDRCH requirement can be verified by RF REFSENS requirement.

Based on the RF REFSENS discussion in RF session, the following agreement was made in previous meeting

Agreement - LLS assumptions and parameters for SNR Parameters; Assumptions. Carrier frequency; 900 MHz. SCS; 15 kHz as baseline. Block structure; Depend on RAN1. Channel model; AWGN. Number of Tx/Rx chains for Ambient IoT device; 1. Number of BS antenna elements; 1. Reference data rate; 1 kbps (M). Message size; Option 1： {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size Option 2： 96 bits. BLER target; 10%. Sampling frequency（SFO）; Option 1: Randomly select a value from the range of [104 ppm~105 ppm] for device 1 Option 2: Use Residual SFO based on companies report. [OOK/BPSK chip rate]; 7.5 kcps for 2SB 15 kHz Transmission bandwidth. Receiver bandwidth; Option1: the same as transmission bandwidth. Option 2：18 kHz (consider SFO and BS filter) for 2SB 15 kHz transmission bandwidth Option 3: first sidelobes as receiver bandwidth Companies can report. Waveform (CW); unmodulated single tone. Modulation; Option1：OOK Option 2：BPSK Companies can report. Line code; Manchester encoding. FEC; 1/3 Convolutional Code. ADC bit width; Ideal. D2R receiver; coherent receiver. Repetition; None. Preamble configuration; Companies can report. Midamble configuration; companies can report.

From RF REFSENS test perspective, the current test is proceed based on CFA procedure. The D2R message type with message 1 will be transmitted for testing. And no repetition is configured.

Observation 1:  From the test procedure perspective, CFA based random-access procedure was considered for testing with D2R message type as message 1

Observation 2:  From the test configuration perspective, no repetition was configured.

For demodulation test point of view, to verify the proper baseband processing, especially for repetition transmission, we think it is necessary to define PDRCH demodulation requirement. While we are also open to further discuss if companies think there is no need to define the related PRDCH demodulation requirement.

Proposal 1:  RAN4 could consider to define the PRDCH demodulation requirement

Specification structure

According to core specification, the RF requirement is specified into the 38.194 for ambient IoT base station (BS) and Carrier-Wave (CW) note radio transmission and reception and into 38.195 for ambient IoT base station (BS) and Carrier -Wave (CW) node conformance testing, since ambient is a new feature with new physical layer design, it is straightforward to specify the corresponding Ambient IoT BS demodulation requirement into 38.194 and Ambient IoT BS conformance testing into 38.195, separately. As for clause number, base on current latest specification structure, considering the RF requirement for A-IoT CW receiver characteristics is not specified, which can be voided, then demodulation requirement can be specified into clause 10 of 38.194 and 38.195, respectively.

Proposal 2:  RAN4 specifies the corresponding Ambient IoT BS demodulation requirement into 38.194 and Ambient IoT BS conformance testing into 38.195, separately. And captured the demodulation requirements into the clause 10 of TS 38.194/195

# 3	Test Setup

In this section, the view on the test setup for PDRCH demodulation requirement.

Modulation

Based on RAN1 design, both OOK and BPSK modulation can be supported for D2R transmission,

Agreement - For D2R BPSK modulation - After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {-1} in the baseband. - For D2R OOK modulation - After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {0} in the baseband.

According to device procedure for modulation scheme determination as following, it is up to device implementation to use OOK and BPSK modulation.

- If the PDRCH is for transmitting Msg1 or corresponds to a contention-free random-access procedure - determine according to its implementation to use either OOK modulation or BPSK modulation - Otherwise - Use the same modulation as determined for transmitting the preceding Msg 1 .

Meanwhile, based on RF reference sensitivity testing, both OOK and BPSK were considered. To align with RF testing, we think both of them can be considered when defining PDRCH requirement.

Proposal 3: RAN4 should cover both BPSK and OOK modulation scheme when defining PDRCH requirement

Waveform

For the D2R transmission by backscattering, the waveform is provided by the carrier wave. The D2R baseband signal is not -OFDM.

Based on the WID, only device 1 was included, where the device is passive device without an amplified, relies on backscattering a carrier wave. Following the WID, we prefer to consider use the backscattering a carrier wave when defining PDRCH requirement. While based on RF REFSENS testing, the unmodulated single tone CW was considered.  Therefore, RAN4 should clarify whether unmodulated single tone CW should be considered for PDRCH requirement following the RF testing.

Proposal 4: Following the WID, RAN4 should consider the waveform with backscattering a carrier wave for defining PDRCH requirement, FFS on consider the unmodulated single tone CW for PDRCH requirement.

Block level Repetition

For the D2R transmission, the block level repetition is supported as following

Figure 4: PDRCH transmission with block-level repetition

The number of block-level repetitions, {1, 2} are supported.  From demodulation requirement, block-level repetition requires additional processing. Meanwhile, based on the random-access procedure, for PDRCH, either message 1 or message 3 can support.  Message 3 will include the scheduling information includes whether to apply convolutional coding or not an apply the repetition or not. Since our preference is to consider message 3 for PDRCH testing, we think at least block-level repetition as 2 should be considered for defining PDRCH requirement. We are also open to discuss whether block-level repletion as 1 will also be considered for requirement.

Proposal 5: RAN4 can consider block-level repetition at least as 2 when defining PDRCH requirement, FFS on considering block-level repetition as 1 for requirement

D2R bandwidth

Regarding the D2R bandwidth, the following was agreed in RAN4 previous meeting as

Agreement D2R bandwidth - Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1. - Assumed 10% SFO for device - For Reader, [90%] filter spectrum utility ([10%] guard band) is considered. - The value of transmission BW and channel BW will be explicitly listed. - Using following equation for BS as starting point: - D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9 =(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9 =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9 - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS - Using following equation for device as starting point: - D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift =(2000*(1+R)/Tb)* (1+∣SFO∣) =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣) - The following are for information: Figure 1-1: Small frequency shift diagram of FDMA without SFO Figure 1-1: SFO impact of transmission bandwidth and small frequency shift of FDMA - As the figures above indicate, SFO not only affects the D2R transmission bandwidth but also scale the small frequency shift value - The 2SB transmission BW and small frequency shift both measured in kHz- with and without SFO are as follows - Note: BW and small frequency shift are in kHz, Tc and Tb are in us: 2SB Transmission BW (kHz); Small frequency shift (kHz). Without SFO; With SFO; Without SFO; With SFO. Btx,D2R_ without SFO= 4*1000/Tb=2*1000/(R*Tc); Btx,D2R_ with SFO =Btx,D2R_ without SFO×(1±∣SFO∣); SFSwithout SFO=±1000*R/Tb= ±1000/(2*Tc); SFSwith SFO =SFSwithout SFO×(1±∣SFO∣).

Based on RAN4 agreement, SFO not only affects the D2R transmission bandwidth, but also scale the small frequency shift value. And the assuming 10% SFO for device.

Based on the RF discussion, the BS D2R CBW can be derived as following

- For BS D2R CBW - D2R CBW for BS (kHz) = Ceiling ((2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO)/0.9) = Ceiling ((2+2R)/Tb × (1+∣SFO∣)/0.9) = Ceiling ((1+R)/ (Tc ×R) × (1+∣SFO∣)/0.9) - The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band)

Observation 3: the D2R channel bandwidth is decided based on {Tb, Tc, R} and SFO assumption

As for {Tb, Tc, R }, the following was agreed in RAN1#121

Agreement For D2R transmission, - The minimum Tb is 1.39μs. - Support at least the following values of Tb, Tchip, and R from the table agreed in RAN1#120bis. Tchip (μs). Tb (μs). 133.33; 66.67; 33.33; 16.67; 11.11; 8.33; 5.56; 4.17; 2.78; 2.08; 1.39; 1.04; 0.69; 0.52. 266.67; R=1; R=2; R=4; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128; R=256. 133.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128. 66.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64. 33.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32. 22.22; R=1; R=2; R=4; R=8; R=16. 16.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16. 11.11; R=1; R=2; R=4; R=8. 8.33; R=1; R=2; R=4; R=6; R=8. 5.56; R=1; R=2; R=4. 4.17; R=1; R=2; R=4. 2.78; R=1; R=2. 2.08; R=1; R=2. 1.39; R=1. 1.04; R=1. Note: This does not imply the device to pre-store the table.

Pending on the collusion of SFO assumption, RAN4 should further discuss how to select the proper CBW when defining PDRCH requirements

Proposal 6: Pending on the collusion of SFO assumption, RAN4 should further discuss how to select the proper CBW when defining PDRCH requirements

SFO assumption

Based on RAN4 agreements, SFO not only affects the D2R transmission bandwidth, but also scales the small frequency shift value. And 10% SFO is assumed for device.

To align with RF testing, the SFO with randomly selected a value from the range of [104 ppm~105 ppm] for device 1 can be considered as starting point for defining PDRCH requirement.

Proposal 7: RAN4 can consider the SFO assumption with randomly selected a value from the range of [104 ppm~105 ppm] when defining PDRCH requirement

Device sampling rate

Regarding the device sampling rate, the duration for one chip is 1/(15Kz*M*2) with OOK modulation and mancheester coding.  With larger number of UL sampling, it can be benefit to improve the SFO estimation accuracy.  In general, the device sampling rate is up to implementation. To align with RAN1 assumption, we are fine to consider the device sampling rate as 1.92MHz for alignment, while it should be noted that there is no any limitation for A-IoT device limitation

Proposal 8: RAN4 could consider the device sampling rate as 1.92MHz for alignment, adding  a note there is no restriction on AIoT device  implementation.

TBS

For the D2R transmission, the maximum of TBS equals to the maximum message size of 1000bits for A-IoT. Pending on the size of the PDRCH data transmission, the number of midamble will be different. Considering the 96 bits was selected in the RF testing, we can configure the value of TBS as 96 as starting point

Proposal 9: RAN4 can consider the value of TBS as 96 as starting point when defining PDRCH requirement.

Channel Model

For A-IoT device, the main deployment should be targeting for indoor scenario with low speed, similar as eMTC and NB-IoT. Therefore, the channel varying is slow. Based on RAN1 evaluation, AWGN and TDL-A channel were selected for feasibility evaluation. In existing NR BS requirement, both conducted and OTA test are supported in FR1, and TDL channel was considered for requirement. Therefore, for Ambient IoT BS, we think there is no issue for OTA testing with TDL channel condition. So, we propose to apply TDA30-10 channel for specifying the PDRCH requirement if agreed to introduce.

Proposal 10: RAN4 consider TDLA30-10 channel for specifying PDRCH requirement.

Reader detection assumption

The main motivation for preamble/midamble transmission is to achieve the good synchronous performance, and channel estimation. With coherent detection, it is benefit to improve the PDRCH detection performance. To align with RF REFSENS testing, the coherent receiver is assumed.

Proposal 11: RAN4 consider the coherent receiver when specifying PDRCH requirement

D2R message type for testing

For D2R message type to test, according to the random-access procedure, message 1 or message 3 can be transmitted by PDRCH.  For message 3 transmission, it will include the scheduling information includes whether to apply convolutional coding or not and apply the repetition or not. In that sense. It that sense, it is better to focus on the message 3 for testing.

Proposal 12: RAN4 should consider the D2R message type as Message 3 for testing when defining the PDRCH requirement

D2R message structure for testing

Regarding the D2R message structure, pending on the datarates of PDRCH transmission, both preamble with midamble or without midamble can be supported. And the length of preamble can be categorized as shore preamble and long preamble.

From the D2R preamble signal generation, it is based on m-sequence with the following agreement in RAN1

Agreement - For D2R preamble/midamble, base sequence is generated from m-sequence, where the length of the sequence is 2n-1 - Value(s) of n - Long preamble/midamble is generated based on n = 5 - Working assumption: Short preamble/midamble is generated based on n=3 - Only 1-part preamble/midamble are supported for D2R - Preamble immediately precedes the PDRCH without any gap - Both long and short preamble and midamble are supported based on the working assumption on n - when midamble is present at least the following cases are supported and reader explicitly indicates one of the following cases for PDRCH: - Short preamble and short midamble - Long preamble and long midamble Note: the case of short preamble and long midamble will not be supported - When midamble is not present the reader explicitly indicates short or long preamble for PDRCH For D2R preambles, - For n = 3, adopt m-sequence with following: - Polynomial: x³ + x² + 1 - Initial State: Down-select between 010 or 100 - Resulting Sequence: Down-select between 0 1 0 0 1 1 1 (for 010 initial state) or 1 0 0 1 1 1 0 (for 100 initial state) - For n = 5, adopt m-sequence with following: - Polynomial: x⁵ + x³ + 1 - Initial State: 01001 - Resulting Sequence: 0 1 0 0 1 0 0 0 0 1 0 1 0 1 1 1 0 1 1 0 0 0 1 1 1 1 1 0 0 1 1 - For m-sequence with n =3 for D2R preambles, adopt initial State of 100 and resulting sequence of 1 0 0 1 1 1 0

According to RAN1, for long preamble/midamble, the length of bits is 31 bits, and for short preamble/ midamble,  the length of bits is 7bits.

Due to the asynchronous nature of A-IoT communication, timing signals should be transmitted from the device to the reader. A preamble-based D2R timing acquisition signal (D-TAS) is transmitted before the PDRCH to indicate the starting point of the PDRCH. This signal can also be used for SFO, CFO and channel estimation. The D-TAS signal is a binary signal that has good auto-correlation and cross-correlation properties. For D2R channel estimation, CFO estimation, time tracking, etc., a midamble signal is used. Since the SFO is considered in current test, we think the good synchronous performance can be achieved with long preamble. Therefore, our preference is to consider long preamble/midamble configuration

Proposal 13: RAN4 could consider long D2R transmission with midamble inserted with 31 bits Preamble and 31 bits Midamble when defining PDRCH requirements

Number of interval bits for D2R midamble insertion

The number of midambles used depends on the size of the PDRCH data, but there is a trade-off between the number of midambles and achievable datarates. The following is the device procedure for D2R generation

A device shall generate the D2R transmission using the following parameters provided by higher layers: - the duration in microseconds of each D2R bit, TbitD2R - the block repetition number, Rblock - the small frequency shift factor to be used, RSFS - the interval in bits for D2R midamble insertion, Ibit - sequence length indicator for D2R-ambles, Lamble - the additional D2R midamble insertion indicator, Iadd

According to the procedure, the proper value of interval in bits for D2R midamble insertion should be configured, As agreed in RAN1#121 meeting as following, the value can be configured as {48,96, 168, 240}

Agreement - For D2R, for indicating the interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information, following interval values are adopted: - For bit duration of 266.67μs - I = 48 bits, 96 bits, 168 bits, 240 bits - For other supported bit durations of 266.67μs/Y - I = Y * {48, 96, 168, 240} bits - Values of Y: 2, 4, 8, 16, 32, 64, 192 - For signaling via R2D control information, following is adopted: - 1-bit length codepoint is used to indicate whether long or short preamble/midamble is applied at the device, where “0” indicates short preamble/midamble and “1” indicates long preamble/midamble - Midamble interval is indicated by a 2-bit length codepoint - Lowest to highest codepoint value indicates lowest to highest interval value - 1-bit length codepoint is used to indicate whether the midamble is present at the end or not, where “0” indicates no midamble present at the end and “1” indicates midamble present at the end - Note: if the indicated interval is longer than the number of bits after FEC (if FEC is applied) and repetition (if repetition is applied), and 1-bit length codepoint does not indicate midamble present at the end, then there is no midamble.

Considering multiple midambles is used for D2R channel estimation and CFO and timing tracking, based on our preferred TBS value, RAN4 could take the value as 48 bits as starting point.

Proposal 14: RAN4 should discuss the value of interval bits for D2R midamble insertion. The value with 48 bits can be taken as starting point

# 4	Conclusion

In this contribution, the overview on test scope and test setup for Ambient-IoT BS demodulation requirement was provided.

FFS to define PDRCH requirement

Observation 1:  From the test procedure perspective, CFA based random-access procedure was considered for testing with D2R message type as message 1

Observation 2:  From the test configuration perspective, no repetition was configured.

Proposal 1:  RAN4 could consider to define the PDRCH demodulation requirement

Specification structure

Proposal 2:  RAN4 specifies the corresponding Ambient IoT BS demodulation requirement into 38.194 and Ambient IoT BS conformance testing into 38.195, separately. And captured the demodulation requirements into the clause of TS 38.194/195

Modulation

Proposal 3: RAN4 should cover both BPSK and OOK modulation scheme when defining PDRCH requirement

Waveform

Proposal 4: Following the WID, RAN4 should consider the waveform with backscattering a carrier wave for defining PDRCH requirement, FFS on consider the unmodulated single tone CW for PDRCH requirement.

Block level Repetition

Proposal 5: RAN4 can consider block-level repetition at least as 2 when defining PDRCH requirement, FFS on considering block-level repetition as 1 for requirement

D2R bandwidth

Observation 3: the D2R channel bandwidth is decided based on {Tb, Tc, R} and SFO assumption

Proposal 6: Pending on the collusion of SFO assumption, RAN4 should further discuss how to select the proper CBW when defining PDRCH requirements

SFO assumption

Proposal 7: RAN4 can consider the SFO assumption with randomly selected a value from the range of [104 ppm~105 ppm] when defining PDRCH requirement

Device sampling rate

Proposal 8: RAN4 could consider the device sampling rate as 1.92MHz for alignment, adding  a note there is no restriction on AIoT device implementation.

TBS

Proposal 9: RAN4 can consider the value of TBS as 96 as starting point when defining PDRCH requirement.

Channel Model

Proposal 10: RAN4 consider TDLA30-10 channel for specifying PDRCH requirement.

Reader detection assumption

Proposal 11: RAN4 consider the coherent receiver when specifying PDRCH requirement

D2R message type for testing

Proposal 12: RAN4 should consider the D2R message type as Message 3 for testing when defining the PDRCH requirement

D2R message structure for testing

Proposal 13: RAN4 could consider long D2R transmission with midamble inserted with 31 bits Preamble and 31 bits Midamble when defining PDRCH requirements

Number of interval bits for D2R midamble insertion

Proposal 14: RAN4 should discuss the value of interval bits for D2R midamble insertion. The value with 48 bits can be taken as starting point

# References

R4-2512603, WF for A-IoT demod_RAN4#116_final