3GPP TSG-RAN WG4 Meeting #116bis            	R4-2513405

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.12.7

Source:	Samsung

Title:	Discussion and simulation results for AI based CSI prediction

Document for:	Discussion

- Introduction

In the last RAN4#116 meeting, CSI reporting requirement framework for CSI prediction was under discussed. The related agreed agreements are captured in the WF [1] as following

- CSI reporting requirement framework for CSI prediction - Scheduling delay - The delay from when UE prediction is sent until the time the TE applied is n+4 (slots) for FDD - Performance monitoring - RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction - FFS which requirements are to be introduced - Requirement baseline for monitoring - Introduce the following requirements for CSI performance monitoring - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS whether accuracy requirement can be defined/checked - Mapping-table-To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this - Reporting delay requirement - Wait for RAN1 decision

Meanwhile, in order to facilitate the AI/ML model evaluation and alignment, the updated simulation assumption is agreed and captured in the [2]. In this contribution, the view on the test setup discussion is provided. Meanwhile, the simulation results for AI/ML based CSI prediction calibration and comparison are provided.

# 2	Test setup for requirement

In this section, we provide our view on the general test setup for specifying the PMI requirement based on AI/ML CSI prediction related with Aperiodic CSI-RS configuration and periodic CSI-RS configuration with TDD

Aperiodic CSI-RS configuration

According RAN1 agreement, both periodic and aperiodic CSI-RS configuration are considered as following

Agreement in RAN1#116 For the evaluation of the AI/ML based CSI prediction, consider the following CSI-RS configuration - Periodic: 5ms periodicity (baseline), 20 ms periodicity (encouraged) - Aperiodic: Optional, CSI-RS burst with K resources and time interval m slots (based on R18 MIMO eType -II ) - Note: Companies to report observation window (number/distance) and prediction window Agreement in RAN1#116bis For the AI/ML based CSI prediction, adopt the following assumption as a baseline for evaluation purpose - UE speed: 30km/h, 60km/h - Others can be additionally submitted, e.g., 10km/h, 120km/h - Observation Window (number/distance): 5/5ms, 10/5ms - Others can be additionally submitted, e.g., 4/5ms, 15/5ms - Prediction window (number/distance between prediction instance/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms, 4/5ms/5ms - Others can be additionally submitted, e.g., 2/5ms/5ms, 3/5ms/5ms, 1/5ms/10ms - For other assumption, reuse Rel-18 baseline Agreement in RAN1#116bis For the evaluation of the AI/ML based CSI prediction, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted - Note: The same selected parameter combination shall be applied for benchmarks - CSI report periodicity: 5ms, 20ms (encouraged) - Other can be additionally submitted, e.g., 10ms

RAN4 has already discussed the periodic CSI-RS configuration based on RAN1 agreement in the previous meeting, and agreed to apply the test setup in [3] as starting point for performing step-3 throughput simulation.

Regarding the aperiodic CSI-RS configuration, the number of CSI-RS burst with K resource and time interval m slots can be configured based on Rel-18 MIMO eType II codebook. And for Rel-18 eType II doppler codebook test, only 1 time distance for prediction was considered, and K as 4 for both FDD and TDD, as following

Agreement in RAN4#110bis Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook - For FR1 FDD 15khZ SCS case, test setup is as below descriptions - Non-CSI slots: scheduled in mod(i, 10) = {1,3,5,7,8,9}, i={0,1,2,…,19} - CSI-RS is scheduled in mod(i, 10) = {0,2,4,6}, i={0,1,2,…,19} - SSB slots: i=0 - TRS slots: i=10 and 11 - CSI report is transmitted in slot#10n+10 to estimate precoder in slot#10n+11 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 17, 18, 19, 21, 23} Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook - For FR1 TDD 30kHz SCS case, test setup is as below descriptions for the simulation of following Rel18 Doppler codebook vs random Type I single panel codebook: - Scheduling pattern for TDD SCS=30kHz with the periodicity of 20ms (40 slots): - Non-CSI slots: scheduled in mod(i, 10) = {5,6}, i={0,1,2,…,39} - CSI slots: scheduled in mod(i, 10) = {0,1,2,3}, i={0,1,2,…,39} - SSB slots: i=0 - TRS slots: i=20 and 21 - CSI report is transmitted in slot#10n+9 to estimate precoder in slot#10n+10 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 16}

The existing test setup for aperiodic CSI-RS configuration can be considered as starting point for AI/ML-based CSI prediction. Considering the alignment of AI/ML based CSI under periodic CSI-RS configuration are still under conducted, meanwhile, the aperiodic CSI-RS configuration for CSI prediction is optional, we suggest the aperiodic CSI-RS configuration can be deprioritized. After the alignment work of periodic CSI-RS configuration is stable, we can proceed the related alignment and requirement discussion for aperiodic CSI-RS configuration.

Proposal 1:  Existing test configuration with aperiodic CSI-RS configuration can be considered as starting point for AI/ML based CSI prediction.

Proposal 2: Focus on periodic CSI-RS configuration for AI/ML based CSI prediction firstly, after the alignment work of periodic CSI-RS Configuration is stable, RAN4 can proceed the related alignment and requirement discussion for Aperiodic CSI-RS configuration

TDD configuration

In legacy PMI test, the requirements with both TDD and FDD are specified. In current evaluation for AI/ML based CSI prediction, it is mainly based on the FDD scenario.  For TDD configuration, the following configuration can be considered as starting point for evaluation.

For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note:If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3

Considering the limited time schedule for this WI, only 3 meeting is left based on the original plan. If the TDD requirement with AI/ML based CSI prediction, also need to be specified in Rel-19, it is better to discuss the related test configuration and prepare the AI/ML model alignment.

Proposal 3: Considering only 3 meeting is left based on the original plan,  if TDD requirement   with AI/ML based on CSI prediction also need to be specified in Rel-19,  it is better to discuss the related test configuration and prepare the AI/ML model alignment.

MCS

In the last previous meeting, MCS 13 was selected for initial performance evaluation. Based on companies results, the target SNR related with 90% of maximum throughput is lower and close to 0dB for AI/ML based CSI prediction, which is not a typical SNR value for PMI considering the robust SNR testing. As agreed, MCS 17 and MCS 19 were also included for evaluation and check the feasibility of AI/ML CSI prediction. Based on agreement, UE performance at more than one MCS should be tested. Based on results, companies can discuss the MCS selected for requirement if it is feasible for testing.

Proposal 4: Pending on the results, companies can discuss the MCS selected for requirement if it is feasible for testing.

Scheduling for PMI requirement

Regarding the reporting delay between CSI-RS and UCI containing the report with the predicted PMI, in current evaluation, 4 ms separation between CSI-RS and predicted PMI report was considered.

Meanwhile, RAN1 is discussing the CSI processing timeline with following agreement in the previous meeting

Agreement For CSI prediction using UE-side model, for inference, in addition to legacy Z/Z’ for doppler codebook, UE may report the value of t per SCS - Detailed value of t can be discussed in UE feature

The negative values of t shall be supported to shorted inference latency compared to non-AI based scheme if using dedicated AI hardware and high degree of parallelization for AI based CSI processing (e.g., matrix operations)

Based on UE feature discussion, RAN1 supported value of t for the relaxation of Z and Z’ timeline, and the value is pending on UE reported capability. The value of t can be large than 0 or small than 0 pending on different SCS.

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp 12. supported value of t for the relaxation of Z and Z’ timeline 13. supported number of occupied CPU 14. supported number of occupied APU 15. Index of the occupied APU pool | 2-35 58-0-1 | yes | n/a | CSI prediction for N4=1 for inference is not supported | [Per band and Per BC] | n/a | n/a | n/a | Component 3 candidate values: a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Based on UE capability, we think RAN4 should update the schedule timeline for requirement according to the value of t pending on UE capability report

Proposal 5: RAN4 will update the schedule timeline for requirement according to the value of t pending on UE capability report

# 3	Simulation Results

According to the WF in [2], companies are encouraged to provide the CSI prediction simulation results for next meeting, including both SGSC and throughput based on agreed simulation assumption. The simulation assumption can be referenced in Appendix.

For performing the throughput simulation, the test setup for FDD with Periodic CSI-RS configuration (based on R.PDSCH.1-24.1 FDD) was applied

Test setup for FDD with Periodic CSI-RS configuration (based on R.PDSCH.1-24.1 FDD)

In the updated simulation assumptions for AI/ML CSI prediction [2] there is a proposal for test setup for FDD with Periodic CSI-RS configuration based on R.PDSCH.1-24.1 FDD fixed reference channel. We see this proposal as a good and simple starting point for the next meeting Step-3 simulations where PDSCH throughput will be evaluated with codebook quantization.

3.1	SGCS

According to updated simulation, companies are also encouraged to provide the following two SGCS metrics as

- SGCS 2:  Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channels’ SVD

- SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType-II-Doppler CB with the model output against the PMI-derived based on Rel-18 eType-II doppler CB with the ground truth.

Meanwhile, under practical channel estimation, the additional SCS -1 and SGC-3 as KPIs for step-3 are also encouraged to provide for better aligning and understanding companies’ simulation results

Fig 1. Illustration of SGCS2 and SGCS 3 in step 2

For this part, we have provided the updated results of SGCS in step 2 as summaries in table 1 and table 2

Table 1:  SGCS result in Step 2, 20Hz Doppler (TO be updated)

| Metric | Number of Tx | Train | 20Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank 2-average | Rank2-layer1 | Rank2-layer2 | Non-AI (sample and hold) |
| SGCS2 | 16 | Non-mix | 0.5211 | 0.4826 | 0.5211 | 0.4441 | 0.4425 for Rank1 0.4425 for Rank2-L1 0.3243 for Rank2-L2 |
|  |  | Mix (20/50) | 0. 0.52083 | 0.4815 | 0.5208 | 0.4421 |  |
| SGCS3 | 16 | Non-mix | 0.9900 | 0.9853 | 0.9900 | 0.9807 | 0.4664 for Rank1 0.4464 for Rank2-L1 0.3507 for Rank2-L2 |
|  |  | Mix (20/50) | 0.9724 | 0.9631 | 0.9724 | 0.9538 |  |

Table 2:  SGCS result in Step 2, 50Hz Doppler (To be updated)

| Metric | Number of Tx | Train | 50Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank 2-average | Rank2-layer1 | Rank2-layer2 | Non-AI (sample and hold) |
| SGCS2 | 16 | Non-mix | 0.4846 | 0.4444 | 0.4846 | 0.4042 | 0.2255 for Rank1 0.2255 for Rank2-L1 0.1406 for Rank2-L2 |
|  |  | Mix (20/50) | 0. 4848 | 0.4455 | 0.4848 | 0.4062 |  |
| SGCS3 | 16 | Non-mix | 0.6475 | 0.5990 | 0.6475 | 0.5506 | 0.2033 for Rank1 0.2033 for Rank2-L1 0.1428 for Rank2-L2 |
|  |  | Mix (20/50) | 0.6468 | 0.5900 | 0.6468 | 0.5512 |  |

And also, the SGCS in step 3 under practical channel estimation were also provided and summarized in table 3 and 4

Table 3:  SGCS result in Step 3, 20Hz Doppler (TO be updated)

| Metric | Number of Tx | Train | 20Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank 2-average | Rank2-layer1 | Rank2-layer2 | Non-AI (sample and hold) |
| SGCS2 | 16 | Non-mix |  |  |  |  |  |
|  |  | Mix (20/50) |  |  |  |  |  |
| SGCS3 | 16 | Non-mix |  |  |  |  |  |
|  |  | Mix (20/50) |  |  |  |  |  |

Table 4:  SGCS result in Step 2, 50Hz Doppler (To be updated)

| Metric | Number of Tx | Train | 50Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank 2-average | Rank2-layer1 | Rank2-layer2 | Non-AI (sample and hold) |
| SGCS2 | 16 | Non-mix |  |  |  |  |  |
|  |  | Mix (20/50) |  |  |  |  |  |
| SGCS3 | 16 | Non-mix |  |  |  |  |  |

Observation 1:  With mix dataset training, the performance has minor performance degradation. Good generalization performance can be achieved with mix dataset training

3.2	Throughput

In the performance evaluation, the raw channel matrices estimated based on CSI-RS signals are feed into the AI model as input data per SNR, where the SNR range in our simulation is from -20dB to 20dB.The CSI-RS interval is defined as 5ms. The AI model predicted the channel information in the next one CSI-RS slot, based on the historical 5 CSI-RS channel information, i.e., the observation window is 5/5ms and the prediction window is 1/5ms/5ms. The following shows the initial throughput performance of AI/ML-based CSI prediction schemes under various UE moving speed scenarios.  As benchmark schemes, the throughput achieved by random PMI with type -I single panel codebook, achieved by achieved non-AI scheme (sample and hold) scheme based on Rel-18 eType II Doppler codebook are provided for comparison separately.

In the last meeting, as agreed, in addition to Tput ratio of AI/ML based CSI prediction compared with Rel-15 Type I random PMI, the Tput ratio of AI/ML based CSI prediction compared with Rel-16 eType II CB(sample and hold) is also encouraged to provide. Beside, for comparison, we also output the Tput ratio of Rel-16 eTypen II CB (sample and hold ) compared with Rel-15 Type I random PMI.

- γ1_AI/ML：AI/ML based CSI prediction is compared with Rel-15 Type I random PMI

- γ2_AI/ML：AI/ML based CSI prediction is compared with Rel-16 eType II CB(sample and hold)

- γ3_AI/ML：Rel-16 eType II CB (sample and hold) is compared with Rel-16 eType II CB(sample and hold)

Figure 2.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI,  16Tx/4Rx, MCS 13, 20Hz

Figure 3.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI,  16Tx/4Rx, MCS 13, 50Hz

Figure 4.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI,  16Tx/4Rx, MCS 17, 20Hz

Figure 5.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI,  16Tx/4Rx, MCS 17, 50Hz

Figure 6.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI,  16Tx/4Rx, MCS 19, 20Hz

Figure 7.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI, 16Tx/4Rx, MCS 19, 50Hz

As observed from figure 2 and figure 3 for MCS 13, compared with Rel-15 Type I random PMI, both AI and non-AI schemes can achieve large performance gain for different doppler scenario. Compared with non-AI scheme Rel-18 eType II Doppler with K=1, the gain of AI scheme is minor, especially under the scenario with doppler as 50Hz. Although AI can achieve better performance of channel prediction, the gain can be not reflected the throughput improvement directly.

As observed from figure 4 and figure 5 for MCS 17, the Tput ratio corresponding to 90% of maximum throughput is reduced as the doppler increasing. The similar Phenomenon can be observed in figure 6 and figure 7 for MCS 19

In table 3, the SNR value corresponding to 90% of maximum throughput of AI/ML based-CSI prediction, Rel-18 eType II Doppler Codebook (with K=1) compared with Rel-15 random Type 1 PMI, as well as the related Tput ratio is summarized

Table 3 SNR value corresponding to 90% of maximum throughput of AI /ML based CSI prediction Comparison (To be updated)

| Doppler | MCS | Prediction SNR [dB] | Sample&Hold SNR [dB] | Prediction vs random γ1_AI/M | Prediction vs Sample&Hold γ2_AI/M | Sample&Hold vs Random γ3_AI/M |
| --- | --- | --- | --- | --- | --- | --- |
| 20Hz | 13 | -1.691 | -0.869 | 7 | 1.1 | 6.69 |
|  | 17 |  | 2.6 |  |  | 3.8 |
|  | 19 |  | 3.8 |  |  | 2.44 |
| 50Hz | 13 | 1.1778 | 1.323 | 3 | 1 | 3 |
|  | 17 |  | 5.38 |  |  | 3.65 |
|  | 19 |  | 7.26 |  |  | 2.27 |

Observation 2:  Compared with Rel-15 Type I random PMI, both AI and non-AI schemes can achieve large performance gain for different doppler scenario. Compared with non-AI scheme Rel-18 eType II Doppler with K=1, the gain of AI scheme is minor, especially under the scenario with doppler as 50Hz

Observation 3: With doppler increasing, all the schemes show the performance degradation

3.3	Generalization

In this part, we further check the generalization of AI/ML model. In our current results, the model training is per SNR value.  The following is our initial results related the model generalization with SNR,

Figure 8. Throughput comparison with AI based scheme, with different SNR, 16Tx.4Rx, 20Hz, test SNR=-5dB,

Figure 9.  Throughput comparison with AI based scheme, with different SNR, 16Tx.4Rx, 50Hz, testing SNR =0dB

According to our results, we observe with the AI/ML model training under high SNR condition, can achieve better performance compared with the AI/ML model training with the test SNR. With the minor noise impact, AI/ML can learn better channel correlation property under high SNR condition compared with low SNR condition. For one specific testing SNR, with different model training by different dataset, the performance will be different. Then, it may be difficult to align the result and specific the proper requirement. Therefore, RAN4 should clarify the SNR range of dataset for AI/ML model training if the final results among companies have large gap.

Observation 4: With the AI/ML model training under high SNR condition, better performance can be achieved compared with the AI/ML model training with the test SNR

Proposal 6: RAN4 should further check the generalization related with SNR for AI/ML based CSI prediction

Proposal 7: RAN4 should clarify the SNR range of dataset for AI/ML model training if the final results among companies have large gap.

# 4	Conclusion

In this contribution, the view on test setup of demodulation requirement for AI/ML based CSI prediction is provided. Meanwhile, the simulation results including the SGCS in Step 2 and Throughput performance in Step 3 are provided for AI/ML model alignment.

Proposal 1:  Existing test configuration with aperiodic CSI-RS configuration can be considered as starting point for AI/ML based CSI prediction.

Proposal 2: Focus on periodic CSI-RS configuration for AI/ML based CSI prediction firstly, after the alignment work of periodic CSI-RS Configuration is stable, RAN4 can proceed the related alignment and requirement discussion for Aperiodic CSI-RS configuration

Proposal 3: Considering only 3 meeting is left based on the original plan,  if TDD requirement   with AI/ML based on CSI prediction also need to be specified in Rel-19,  it is better to discuss the related test configuration and prepare the AI/ML model alignment.

Proposal 4: Pending on the results, companies can discuss the MCS selected for requirement if it is feasible for testing.

Proposal 5: RAN4 will update the schedule timeline for requirement according to the value of t pending on UE capability report

Observation 1:  With mix dataset training, the performance has minor performance degradation. Good generalization performance can be achieved with mix dataset training

Observation 2:  Compared with Rel-15 Type I random PMI, both AI and non-AI schemes can achieve large performance gain for different doppler scenario. Compared with non-AI scheme Rel-18 eType II Doppler with K=1, the gain of AI scheme is minor, especially under the scenario with doppler as 50Hz

Observation 3: With doppler increasing, all the schemes show the performance degradation

Observation 4: With the AI/ML model training under high SNR condition, better performance can be achieved compared with the AI/ML model training with the test SNR

Proposal 6: RAN4 should further check the generalization related with SNR for AI/ML based CSI prediction

Proposal 7: RAN4 should clarify the SNR range of dataset for AI/ML model training if the final results among companies have large gap.

# References

R4-2502856, WF on AI/ML for NR air interface

R4-2505105, WF on AI/ML for NR Air Interface

R4-2508102, WF on updated simulation assumptions for AI based CSI prediction

# Appendix

# 1	Simulation Assumption

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD |
| Subcarrier spacing | 15kHz for FDD |
| Bandwidth | 10MHz for FDD |
| Symbol | 1 symbol for step-1 and step-2 simulation |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| gNB TX antennas | Baseline: 16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements Optional: 32(N1,N2)=(4,4) as defined in Rel-15 type1 |
| UE RX antennas | 4 |
| Channel model | TDLA30 with XP medium correlation as baseline |
| Doppler spread | 20Hz, 50Hz |
| Channel estimation for CSI prediction | Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| CSI-RS configuration | For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). |
| Rank per UE | Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline |
| CSI feedback assumption | Step-1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account |
|  | Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed |
| Test setup(timeline) | Only for step-3 For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 FFS test setup for aperiodic CSI-RS configuration |
| SNR assumption | Only for step-3 Claimed by companies for step-3 first round simulation (TP as KPI) FFS: other suggestions |
| CSI feedback overhead | PC7 as baseline |
| Baseline | - Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-3 or performance requirements) |
| KPI | Step-1: - check SGCS before CSI feedback - SGCS is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-2: - check SGCS after CSI feedback - Encouraged companies to provide the following two SGCS metrics: - SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. - SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-3: - check TPs after CSI feedback for both AI based CSI prediction and the baseline(Random PMI with Rel-15 Type I single panel codebook), as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction. - Companies are also encouraged to provide the TP curve with corresponding SNRs. - The test parameters for TPs simulation can be referred as 38.101-4 section of 6.3 - FDD with 16Tx4Rx: Table 6.3.3.1.8-1 |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training data and test data | Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz,); 20Hz Doppler spreads. 50Hz Doppler spreads. |
| Companies can provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models. |  |