3GPP TSG-RAN WG4 Meeting #116bis	R4-2513419

Prague, Czech Republic, October 13 – 17, 2025

Agenda Item:	6.11.2

Source:	Huawei, HiSilicon

Title:	Discussion on general aspects on AI/ML for NR air interface

Document for:	Discussion and Decision

- Introduction

According to WF in RAN4 #114bis [1], we discuss the remaining issues related to the general aspects on AI/ML for NR air interface.

- LCM discussion

## RAN4 #114bis WF ## Delay requirements for reporting / monitoring Agreement: - Delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) in case-by-case manner - FFS on which use cases the requirement will be defined for - FFS on the details for each use case in WID

According to RAN4 #114bis WF, delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) will be discussed in case-by-case manner.

For Type 1 Option2 UE-assisted monitoring for BM, RAN1 discussed multiple types of resource for the set for monitoring. It is observed that periodic, aperiodic and semi-persistent monitoring report types are all agreed.

RAN1 #120bis Chairman Notes: Agreement For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB Agreement For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type: Monitoring report type Inference report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support. Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling Agreement For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model: - Support the size of a set for monitoring is the same as the size of Set A, - The n-th resource in the set for monitoring is linked to the n-th resource in Set A. - Support the size of a set for monitoring is smaller than the size of Set A Working Assumption At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

For periodic monitoring reporting, delay requirement is related to the transmission occasion of the CSI-RS/SSB resources for monitoring. For aperiodic and semi-persistent monitoring reporting, delay requirement is not only related to the transmission occasion of the CSI-RS/SSB resources for monitoring, but also related to the trigger signaling of the monitoring reporting.

Observation 1: Different delay requirements are identified for different performance monitoring reporting types.

Proposal 1: RAN4 defines different delay requirements for periodic, aperiodic and semi-persistent monitoring reporting.

RAN4 #114bis WF [1] ## Delay requirements for activation/switching/fallback Agreement: - RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2: - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference - functionality deactivation refers to the process of network deactivating an active functionality Agreement: - FFS on whether the deactivation requirements will be introduced - The starting point is when the UE receives the signalling to commence deactivation - FFS when the endpoint for deactivation is - FFS whether fallback delay is the same as deactivation delay or has some other components Agreement: - For beam management use case - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistent reporting based on AI/ML - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML - FFS on the definition of first reporting

If UE-side AI/ML functionality is deactivated by NW, UE is not expected to report AI inference results once the deactivation signalling is received at UE. The uplink transmission resource for AI-based reporting will be released. There is no need to introduce deactivation requirements, since NW can control when the uplink transmission resource for AI-based reporting is not available.

Observation 2: NW controls when the uplink transmission resource for AI-based reporting is not available.

Proposal 2: RAN4 not to define deactivation requirements.

- Generalization

RAN4 #114bis WF [1] ## Relation between generalization testing and additional conditions signalling Agreements - There shall be consistency between applicable conditions signalling and testing - No test will be defined that implies a change of network condition or associated ID during the test. ## Non-static scenarios Agreement: - Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing. - FFS whether a mixed datset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario.

It is observed that static scenarios are assumed by default according to RAN4 #114bis WF. For generalization testing, the mixed dataset should also be static. Since for non-static scenarios, performance monitoring and management procedures, such as model switching, may be involved. There is no need to mix multiple tests together.

Proposal 3: If a mixed dataset is created for testing generalization, the mixed dataset should be static.

- Conclusions

According to the discussion, following proposals and observations are provided:

Observation 1: Different delay requirements are identified for different performance monitoring reporting types.

Proposal 1: RAN4 defines different delay requirements for periodic, aperiodic and semi-persistent monitoring reporting.

Observation 2: NW controls when the uplink transmission resource for AI-based reporting is not available.

Proposal 2: RAN4 not to define deactivation requirements.

Proposal 3: If a mixed dataset is created for testing generalization, the mixed dataset should be static.

References

- R4-2508080, “WF on AI ML for air interface study”, 3GPP RAN4 #115, 2025.