3GPP TSG-RAN WG4 Meeting #116bis	R4-25xxxxx

# Bengaluru, India, August 25th – 29th, 2025

Title: 	On demodulation and CSI requirements for R19 MIMO evolution

Source: 	Huawei, HiSilicon

Agenda item:	6.13.3

Document for:	Discussion

Background

In last meeting, a WF [1] for MIMO demod was agreed, where many open issues were remained for discussion. This contribution provides our views on these open issues.

- UE performance requirements

Single panel Type I codebook

Scaling factors for MIMO correlation matrices

One issue is scaling factors for MIMO correlation matrices, legacy scaling factor was introduced to ensure the MIMO correlation matrices is positive semi-definite after round-off to four digit precision. However, it’s hard to specify the 64x2 and 64x4 matrix in the table, so we propose to not specify the concrete MIMO correlation matrices for 64Tx in the spec.

Proposal 1: Not to specify the concrete MIMO correlation matrices for 64Tx in the spec.

Another issue is whether round-off to four digit precision is needed if we don’t specify the concrete MIMO matrices. Our measurement in Table 2.1-1 and 2.1-2 show the smallest scaling factor for MIMO correlation matrix with and without round-off to four digit precision operation respectively:

Table 2.1-1: Summary of smallest scaling factor with round-off to four digit precision operation

| MIMO correlation | 64x2 | 64x4 |
| --- | --- | --- |
| Medium | 0.00041 | 0.00062 |
| High | 0.00032 | 0.00065 |

Table 2.1-2: Summary of smallest scaling factor without round-off to four digit precision operation

| MIMO correlation | 64x2 | 64x4 |
| --- | --- | --- |
| Medium | 0.00000 | 0.00000 |
| High | 0.00001 | 0.00001 |

It can be shown that the smallest scaling factor for MIMO correlation with and without round-off to four digit precision operation is different.

Observation 1: The smallest scaling factor for MIMO correlation with and without round-off to four digit precision operation is different.

We prefer to include round-off to four digit precision operation, hence, scaling factors in Table 2.1-1 are preferred.

Proposal 2: Include round-off to four digit precision operation and use scaling factors in Table 2.1-1

MCS

One issue is MCS value, our simulation results show that both values guarantee the reasonable SNR range. We are OK with either one, slightly prefer MCS 20.

Proposal 3: Slightly prefer MCS 20.

Correlation configuration

One issue is correlation configuration:

Agreement: - XP-medium as starting point for simulation results alignment.

Our simulation results show XP-medium can guarantee the sufficient gap between follow PMI and random PMI, hence XP-medium is preferred.

Proposal 4: Confirm XP-medium.

Test metric

The legacy PMI metric is defined as the TP ratio of follow PMI to random PMI when SNR point is corresponding to  90% maximum TP of follow PMI. However, the problem is that throughput of random PMI is 0 when SNR point is corresponding to 90% maximum TP of follow PMI, which leads to the infinite Gama value. To address this issue, we propose to set different SNR points for follow PMI and random PMI, denoted as SNR1 and SNR2 respectively, where SNR1 is corresponding to 90% of maximum TP of follow PMI and SNR2 is SNR1 plus XdB (X>0 and FFS for X), X could be specified to make the Gama value in the reasonable range.  Follow text summarize the test metric:

is 90 % of the maximum throughput obtained at using the precoders configured according to the UE reports, and is the throughput measured at +[XdB] with random precoding.

Proposal 5: Modify the legacy test metric to following for 64Port test:

is 90 % of the maximum throughput obtained at using the precoders configured according to the UE reports, and is the throughput measured at +[XdB] with random precoding. X could be specified to make the γvalue in the reasonable range.

CQI/RI/PMI delay for FDD 64CSI-RS ports

One issue is CQI/RI/PMI delay for FDD 64CSI-RS ports:

Agreement: - Apply slot n+4, PMI delay (from the PMI estimation slot to the PMI apply slot) 11ms, Report slot offset = [7], for FDD SCS=15kHz - Interested companies are welcomed to provide results when the reported PMI is applied at slot n+7 Issue 1-2-9: CQI/RI/PMI delay for TDD 64CSI-RS ports Agreement: - Apply slot n+6, PMI delay (from the PMI estimation slot to the PMI apply slot) 11.5ms, Report slot offset = 17, for TDD SCS=30kHz

Capability 2 has been agreed, which means the CSI computation time shall be scaled. According to RAN1 agreement, at least 80 symbols and 144 symbols are needed for FDD/15kHz and TDD/30kHz respectively.

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, the UE shall provide a valid CSI report for the n-th triggered report, - - if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts no earlier than at symbol Zref, and - - if the first uplink symbol to carry the n-th CSI report including the effect of the timing advance, starts no earlier than at symbol Z'ref(n), where Zref is defined as the next uplink symbol with its CP starting after the end of the last symbol of the PDCCH triggering the CSI report(s), and where Z'ref(n), is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of: aperiodic CSI-RS resource for channel measurements, aperiodic CSI-IM used for interference measurements, and aperiodic NZP CSI-RS for interference measurement for a CSI-ReportConfig, or for all triggered sub-configurations if CSI-ReportConfig contains multiple sub-configurations, when aperiodic CSI-RS is used for channel measurement for the n-th triggered CSI report, and where Tswitch is defined in clause 6.4 and is applied only if Z1 of table 5.4-1 is applied. Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as - (Z2,Z2') or Z2⋅P32,Z2'⋅P32, according to UE reported capability 1 or 2, respectively, with (Z2,Z2') of table 5.4-2, if codebookType is set to 'typeI-SinglePanel-r19', 'typeI-MultiPanel-r19', 'eTypeII-r19' or 'typeII-FePortSelection-r19', where P is the total number of CSI-RS ports across the K configured CSI-RS resources for channel measurement, or Table 5.4-2: CSI computation delay requirement 2 Z1 [symbols]; Z2 [symbols]; Z3 [symbols]. Z1; Z'1; Z2; Z'2; Z3; Z'3. 0; 22; 16; 40; 37; 22; X0. 1; 33; 30; 72; 69; 33; X1. 2; 44; 42; 141; 140; min(44, X3X2+ KB1); X2. 3; 97; 85; 152; 140; min(97, X3+ KB2); X3. 5; 388; 340; 608; 560; min(388, X5+ KB3); X5. 6; 776; 680; 1216; 1120; min(776, X6+ KB4); X6.

For report slot offset, for FDD, report slot offset=7 gives UE 84 symbols for CSI computation, for TDD, report slot offset=17 gives UE 238 symbols for CSI computation. That configuration can satisfy the RAN1’s capability. For CSI apply time, we don’t see any technical issues for n+4 for FDD and n+6 for TDD.

Proposal 6: Confirm following CSI pattern:

- Configure report slot offset=7 for FDD and 17 for TDD.

- Configure CSI apply time n+4 for FDD and n+6 for TDD

Other codebook types

R19 eType II and doppler eType II codebook are just simple extension from up to 32 ports to 128 ports, our understanding is that supporting 64 ports has been verified in Type I codebook test, which means the test purpose has been satisfied. However, we see the potential application scenario for eType II codebook in the real deployment, we are open to eType II-r19 codebook. We don’t think type II-Doppler-r19 is needed since it would be sufficient to cover type 1 and eType II-r19 codebook(if agreed).

Proposal 7: Not to introduce PMI requirements for type II-Doppler-r19 codebook.

CJT calibration offset reporting requirements

There is an open issue whether to introduce CJTC reporting requirements. Based on our understanding, like RSRP/RSRQ/TDCP reporting, this feature has more impact on RRM part, where UE report the measurement results of frequency, time and phase offset between TRPs to BS.

RRM has agreed to define measurement period requirements for delay and frequency offset reporting but FFS for defining accuracy requirements. If RRM decides to define accuracy requirements, the feature has been verified and demodulation requirements just to covert measurement errors to performance degradation, which is redundant test. Furthermore, we need to specify new TE behavior that how to utilize the reported results such as time, frequency and phase compensation, which will bring additional TE implementation complexity.

Therefore, we don’t see the need to introduce demodulation and CSI requirements for this feature.

Proposal 8: Not to introduce CJTC offset reporting requirements.

SRS grouping

Now we present our concrete test setup as below. We prefer to only consider 8Rx with 8 layers case.

Approach 1:

Step 1: For simplicity, map each layer to one Tx with same index, i,e, map layer1 to Tx1, layer2 to Tx2, … , map Layer N to Tx N, which can be done by applying identity matrix precoding.

Step 2: In each TDD pattern n, SRS is transmitted in last two UL slots, the SRS configuration shall cover all UE antenna switching capabilities:

- For 1T8R, configure one SRS set having 8 SRS resources transmitted in different symbols, each SRS resource has a single SRS port.

- For 2T8R, configure one SRS set having 4 SRS resources transmitted in different symbols, each SRS resource has 2 SRS ports.

- For 4T8R, configure one SRS set having 2 SRS resources transmitted in different symbols, each SRS resource has 4 SRS ports

TE identifies the mapping between SRS and UE antenna index by measuring the power of each antenna.  The mapping index is denoted as [a0,a1,a2,…,a7], which means UE antenna index a0,a1,a2,…,a7 is related to SRS port index 0,1,2,…,7 respectively.

Step 3: In TDD pattern n+1, TE updates the channel matrix to following according to the measurement of TDD pattern n

Hi,j=1,  if i  equal to SRS port index which UE antenna j maps to 0,  else

With above test setup, each individual codeword can map to each 4Rx which used for MIMO detection jointly. It is noted that TE doesn’t need to calculate precoder and SRS based channel estimation, which is very easy for implementation.

Approach 2:

Another approach is TE use static channel matrix and update precoding per TDD pattern. The concrete test setup is shown as follows:

Step 1: Select static channel H defined in B.1 in TS 38.101-4 as testing propagation channel model

Step 2: In each TDD pattern n, SRS is transmitted in last two UL slots, the SRS configuration shall cover all UE antenna switching capabilities:

- For 1T8R, configure one SRS set having 8 SRS resources transmitted in different symbols, each SRS resource has a single SRS port.

- For 2T8R, configure one SRS set having 4 SRS resources transmitted in different symbols, each SRS resource has 2 SRS ports.

- For 4T8R, configure one SRS set having 2 SRS resources transmitted in different symbols, each SRS resource has 4 SRS ports

TE identifies the mapping between SRS and UE antenna index by measuring the power of each antenna.  The mapping index is denoted as [a0,a1,a2,…,a7], which means UE antenna index a0,a1,a2,…,a7 is related to SRS port index 0,1,2,…,7 respectively.

Step 3: In TDD pattern n+1, TE updates the precoder to following with zero forcing algorithm to cancel the interference between two 4Rx.

W=HHHH-1

Where H=H1,H2, H1 (8x4) is channel matrix from TE to UE antennas with index a0,a1,a2,a4, i,e, SRS port index 0,1,2,3, H2 (8x4) is channel matrix from TE to UE antennas with index a5,a6,a7,a8, i,e, SRS port index 4,5,6,7. The columns of H1 and H2 are columns a0,a1,a2,a4 and a5,a6,a7,a8 of propagation conditions H respectively.

According to RAN1 feature list, supporting “SRS grouping” is optional feature and simplified receiver is RAN4 optional feature, it’s proposed to define the applicability rules as the test is applicable to UE supporting both “Simplified 8Rx receiver” and “SRS grouping”.

Proposal 9: Only define 8Rx with 8 layers for SRS grouping and use two approaches for test setup as starting point:

Approach 1: Step 1: For simplicity, map each layer to one Tx with same index, i,e, map layer1 to Tx1, layer2 to Tx2, … , map Layer N to Tx N, which can be done by applying identity matrix precoding. Step 2: In each TDD pattern n, SRS is transmitted in last two UL slots, the SRS configuration shall cover all UE antenna switching capabilities: - For 1T8R, configure one SRS set having 8 SRS resources transmitted in different symbols, each SRS resource has a single SRS port. - For 2T8R, configure one SRS set having 4 SRS resources transmitted in different symbols, each SRS resource has 2 SRS ports. - For 4T8R, configure one SRS set having 2 SRS resources transmitted in different symbols, each SRS resource has 4 SRS ports TE identifies the mapping between SRS and UE antenna index by measuring the power of each antenna. The mapping index is denoted as [a0,a1,a2,…,a7], which means UE antenna index a0,a1,a2,…,a7 is related to SRS port index 0,1,2,…,7 respectively. Step 3: In TDD pattern n+1, TE updates the channel matrix to following according to the measurement of TDD pattern n Hi,j=1, if i equal to SRS port index which UE antenna j maps to 0, else Approach 2: Another approach is TE use static channel matrix and update precoding per TDD pattern. The concrete test setup is shown as follows: Step 1: Select static channel H defined in B.1 in TS 38.101-4 as testing propagation channel model Step 2: In each TDD pattern n, SRS is transmitted in last two UL slots, the SRS configuration shall cover all UE antenna switching capabilities: - For 1T8R, configure one SRS set having 8 SRS resources transmitted in different symbols, each SRS resource has a single SRS port. - For 2T8R, configure one SRS set having 4 SRS resources transmitted in different symbols, each SRS resource has 2 SRS ports. - For 4T8R, configure one SRS set having 2 SRS resources transmitted in different symbols, each SRS resource has 4 SRS ports TE identifies the mapping between SRS and UE antenna index by measuring the power of each antenna. The mapping index is denoted as [a0,a1,a2,…,a7], which means UE antenna index a0,a1,a2,…,a7 is related to SRS port index 0,1,2,…,7 respectively. Step 3: In TDD pattern n+1, TE updates the precoder to following with zero forcing algorithm to cancel the interference between two 4Rx. W=HHHH-1 Where H=H1,H2, H1 (8x4) is channel matrix from TE to UE antennas with index a0,a1,a2,a4, i,e, SRS port index 0,1,2,3, H2 (8x4) is channel matrix from TE to UE antennas with index a5,a6,a7,a8, i,e, SRS port index 4,5,6,7. The columns of H1 and H2 are columns a0,a1,a2,a4 and a5,a6,a7,a8 of propagation conditions H respectively.

Proposal 10: define the applicability rules as the test is applicable to UE supporting both “Simplified 8Rx receiver” and “SRS grouping”.

- BS performance requirements

There is an open issue whether to define PUSCH demodulation requirements for 3-antenna-port-based UL transmission. Based on our understanding, the feature is introducing new 3-Tx UL codebook. Considering PUSCH test is outer-loop test with fixed precoder rather than PMI test, BS only aware of number of layers (DMRS port) but not the number Tx for PUSCH demodulation. Furthermore, RAN4 has already defined requirements for 4 MIMO layers, we don’t see the need to introduce easier case with 3 MIMO layers. Therefore, we don’t support defining PUSCH requirements for R19 MIMO.

Proposal 11: Not to introduce BS performance requirements for R19 MIMO.

- Simulation results

Figure 4-1 shows our initial simulation results for single panel type 1 codebook, where both MCS20 and MCS 22, XPL medium  are simulated.

Figure 4-1: Simulation results for single panel type 1 codebook.

Table 4-1: Summary of results for single panel type 1 codebook

| Case | SNR(dB)@90% of maxTP |
| --- | --- |
| MCS20, 64T2R, Follow | 13.2 |
| MCS20, 64T2R, Random | N/A |
| MCS20, 64T4R, Follow | 8.0 |
| MCS20, 64T4R, Random | N/A |
| MCS22, 64T2R, Follow | 16.2 |
| MCS22, 64T2R, Random | N/A |
| MCS22, 64T4R, Follow | 10.0 |
| MCS22, 64T4R, Random | N/A |

- Conclusion

In this contribution we provide our views on demodulation and CSI requirements for R19 MIMO. The proposals and observations are:

Proposal 1: Not to specify the concrete MIMO correlation matrices for 64Tx in the spec.

Observation 1: The smallest scaling factor for MIMO correlation with and without round-off to four digit precision operation is different.

Proposal 2: Include round-off to four digit precision operation and use scaling factors in Table 2.1-1

Proposal 3: Slightly prefer MCS 20.

Proposal 4: Confirm XP-medium.

Proposal 5: Modify the legacy test metric to following for 64Port test:

is 90 % of the maximum throughput obtained at using the precoders configured according to the UE reports, and is the throughput measured at +[XdB] with random precoding. X could be specified to make the γvalue in the reasonable range.

Proposal 6: Confirm following CSI pattern:

- Configure report slot offset=7 for FDD and 17 for TDD.

- Configure CSI apply time n+4 for FDD and n+6 for TDD

Proposal 7: Not to introduce PMI requirements for type II-Doppler-r19 codebook.

Proposal 8: Not to introduce CJTC offset reporting requirements.

Proposal 9: Only define 8Rx with 8 layers for SRS grouping and use two approaches for test setup as starting point:

Approach 1: Step 1: For simplicity, map each layer to one Tx with same index, i,e, map layer1 to Tx1, layer2 to Tx2, … , map Layer N to Tx N, which can be done by applying identity matrix precoding. Step 2: In each TDD pattern n, SRS is transmitted in last two UL slots, the SRS configuration shall cover all UE antenna switching capabilities: - For 1T8R, configure one SRS set having 8 SRS resources transmitted in different symbols, each SRS resource has a single SRS port. - For 2T8R, configure one SRS set having 4 SRS resources transmitted in different symbols, each SRS resource has 2 SRS ports. - For 4T8R, configure one SRS set having 2 SRS resources transmitted in different symbols, each SRS resource has 4 SRS ports TE identifies the mapping between SRS and UE antenna index by measuring the power of each antenna. The mapping index is denoted as [a0,a1,a2,…,a7], which means UE antenna index a0,a1,a2,…,a7 is related to SRS port index 0,1,2,…,7 respectively. Step 3: In TDD pattern n+1, TE updates the channel matrix to following according to the measurement of TDD pattern n Hi,j=1, if i equal to SRS port index which UE antenna j maps to 0, else Approach 2: Another approach is TE use static channel matrix and update precoding per TDD pattern. The concrete test setup is shown as follows: Step 1: Select static channel H defined in B.1 in TS 38.101-4 as testing propagation channel model Step 2: In each TDD pattern n, SRS is transmitted in last two UL slots, the SRS configuration shall cover all UE antenna switching capabilities: - For 1T8R, configure one SRS set having 8 SRS resources transmitted in different symbols, each SRS resource has a single SRS port. - For 2T8R, configure one SRS set having 4 SRS resources transmitted in different symbols, each SRS resource has 2 SRS ports. - For 4T8R, configure one SRS set having 2 SRS resources transmitted in different symbols, each SRS resource has 4 SRS ports TE identifies the mapping between SRS and UE antenna index by measuring the power of each antenna. The mapping index is denoted as [a0,a1,a2,…,a7], which means UE antenna index a0,a1,a2,…,a7 is related to SRS port index 0,1,2,…,7 respectively. Step 3: In TDD pattern n+1, TE updates the precoder to following with zero forcing algorithm to cancel the interference between two 4Rx. W=HHHH-1 Where H=H1,H2, H1 (8x4) is channel matrix from TE to UE antennas with index a0,a1,a2,a4, i,e, SRS port index 0,1,2,3, H2 (8x4) is channel matrix from TE to UE antennas with index a5,a6,a7,a8, i,e, SRS port index 4,5,6,7. The columns of H1 and H2 are columns a0,a1,a2,a4 and a5,a6,a7,a8 of propagation conditions H respectively.

Proposal 10: define the applicability rules as the test is applicable to UE supporting both “Simplified 8Rx receiver” and “SRS grouping”.

Proposal 11: Not to introduce BS performance requirements for R19 MIMO.

- Reference

[1] R4-2512527 Way Forward for [116][327] NR_MIMO_demod. Samsung