3GPP TSG-RAN WG4 Meeting #116bis	R4-2513435

Prague, Czech Republic, October 13 – 17, 2025

Title: 	Discussion on demodulation requirements for Ambient-IoT device 1

Source: 	Huawei, HiSilicon

Agenda item:	6.14.4

Document for:	Discussion

Background

A WF [1] was agreed in last meeting, where many open issues are left. This contribution provides our views.

- Discussions

One issue is whether to define PRDCH requirements for Ambient-IoT:

Whether to define PRDCH demodulation requirements for Ambient-IoT - Way forward - Option 1: Define PRDCH demodulation requirement. - Option 2: Not define PRDCH demodulation requirements for Ambient-IoT considering that it can be verified by the RF REFSENS requirement

The difference between RF REFSENS and demodulation test is channel model, which may impact the performance but don’t change the UE processing. Furthermore, Ambient -Iot device is a cheap device, it may not be worth to define costly OTA tests. Therefore it’s proposed to not define demodulation requirements for Ambient-Iot.

Proposal 1: RAN4 to not define PRDCH demodulation requirements for Ambient-IoT.

If RAN4 agrees to define the related PRDCH demodulation requirements for device, we have the following considerations on the test setup.

Test methodology

One issue is test methodology:

- Agreement - 10% BLER with BLER=1- Number of responsed Message 1Number of transmitted Message 0 or BLER=1- Number of responsed Message 3Number of transmitted Message 2 for testing - During countering the number of responded Message 1 or 3, the TE should check if the responded Message 1 or Message 3 is the correct response of Message 0 or Message 2 by checking the device ID - Further discuss the message type to be tested

Message 0 is used to indicate device to initial access the network and message 1 is the response of corresponding message 0, which work in idle mode. Demod test conducted in idle mode will take long test time and high complexity since UE will switch frequently between idle mode and connect mode. We think combination of Message 2 and message 3 is more suitable for demodulation test since message 2 and 3 are applied in connected mode.

Proposal 2: RAN4 to consider message 2 and 3 and use following formular for test methodology

BLER=1- Number of responsed Message 3Number of transmitted Message 2

TBS

One issue is TBS, two options are listed as below:

- Way forward - Option 1: 20 bits - Option 2: 96 bits - Other options are not precluded

According to [2], RAN2 specifies the details of R2D messages:

The fields in this message are defined as follows: - R2D Message Type: This field indicates the message type. See the Table 6.1-1. The length of the field is 3 bits. - AS ID: This field provides/indicates the value of AS ID. The length of the field is 16 bits. - D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. - Choice Indication (CI): This field indicates either Data SDU field is included (when set to 1) or Received Data Size field is included (when set to 0). The length of the field is 1 bit. - Data SDU: This field contains the upper layer data. This field is of variable size. - Received Data Size: This field is to indicate the number of bytes successfully received by the reader. This field is 7 bits.

Following table shows R2D scheduling information:

| Field name | Length | Value range | Description | Indicated L1 parameter in TS 38.291 [2] |
| --- | --- | --- | --- | --- |
| Time Resource Indication | 1 bit | {1, 2} | The number of time domain resource of access occasions triggered by A-IoT Paging message or one Access Trigger message, i.e., X. | N/A |
| Bit Duration | 3 bits | {2, 1, 1/2, 1/4, 1/8, 1/16, 1/32, 1/96} ×τ, where τ=2×106/15000 | The duration in microseconds of each D2R bit. | TbitD2R |
| Frequency Resource Indication | 8 bits | An 8-bit bitmap. The values of small frequency shift factor are {1, 2, 4, 8, 16, 32, 64, 128}. In the bitmap, the first/leftmost bit of the bitmap corresponds to the first value of small frequency shift factor, the second bit corresponds to the second value of small frequency shift factor, and so on. For each bit, value 0 indicates that the corresponding value is not allowed, while value 1 indicates that the corresponding value can be used. | This field indicates: - the set of NSFS potential small frequency shift factors when present in A-IoT Paging message for CBRA. Each small frequency shift factor corresponding to X access occasion(s). NSFS is the number of frequency domain resource of access occasions triggered by A-IoT Paging message or one Access Trigger message, i.e., the number of bits set to value 1. Or - one value of small frequency shift factor when present in A-IoT Paging message for CFA, R2D Upper Layer Data Transfer message. Or - one or multiple values of small frequency shift factor when present in Random ID Response message. A device determines its small frequency shift factor value for the following D2R transmission based on its order of Echoed Random ID field in the Random ID Response message. Regarding different Bit Duration, only the following values can be indicated to 1 in the bitmap: - {1, 2, 4, 8, 16, 32, 64, 128}, when Bit Duration is configured to 2τμs; - {1, 2, 4, 8, 16, 32, 64}, when Bit Duration is configured to τμs; - {1, 2, 4, 8, 16, 32}, when Bit Duration is configured to τ/2 μs; - {1, 2, 4, 8, 16}, when Bit Duration is configured to τ/4 μs; - {1, 2, 4, 8}, when Bit Duration is configured to τ/8 μs; - {1, 2, 4}, when Bit Duration is configured to τ/16 μs; - {1, 2}, when Bit Duration is configured to τ/32 μs; - {1}, when Bit Duration is configured to τ/96 μs. | RSFS associated to the selected access occasion or configured resource for D2R transmission |
| Block Repetition number | 1 bit | {1, 2} | The block repetition number. | Rblock |
| Channel Coding Indicator | 1 bit | {FEC, no FEC} | The channel coding indicator. | Rcode |
| Interval Bits | 2 bits | {S*48, S*96, S*168, S*240}, S is a scale factor, and equals to: - 1, when Bit Duration is configured to 2τ μs; - 2, when Bit Duration is configured to τ μs; - 4, when Bit Duration is configured to τ/2 μs; - 8, when Bit Duration is configured to τ/4 μs; - 16, when Bit Duration is configured to τ/8 μs; - 32, when Bit Duration is configured to τ/16 μs; - 64, when Bit Duration is configured to τ/32 μs; - 192, when Bit Duration is configured to τ/96 μs. | The interval in bits for D2R midamble insertion. | Ibit |
| Sequence Length Indicator | 1 bit | {short, long} | Sequence length indicator for D2R preamble/midamble. | Lamble |
| Additional Midamble Indicator | 1 bit | {absent, present} | Additional D2R midamble insertion indicator. | Iadd |
| D2R TBS | 7 bits | {1, 2, …, 124, 125}, i.e. integers from 1 to 125. | The D2R transport block size in bytes. | NTBSD2R |

Except for upper layer data, R2D message need at least 3(R2D Message Type)+16 (AS ID)+1(Choice Indication)+7(Received Data Size)+25(D2R information)=52 bits, hence option 2(96 bits) is more reasonable.

Proposal 3: RAN4 to consider 96 bits TBS

M-chips for OOK

One issue is M value for OOK , the options are listed as below:

- Way forward - Option 1: 6 - Option 2: 24 - Other options are not precluded

If 96bits TBS is confirmed, we slightly prefer option 2(M=24) to ensure the whole date package transmitted within one slot.

Proposal 4: Use M=24

Channel model

One issue is channel model:

- Way forward - Option 1: TDLA30-10 - Other options are not precluded - TE vendors are encouraged to check the feasibility to emulate TDL channel in the chamber for Device 1 testing

We prefer option 1, which is widely used for demodulation test.

Proposal 5: Use TDLA30-10

Channel Bandwidth

One issue is channel bandwidth, the options are listed as below:

- Way forward - Option 1: 2 PRBs - Option 2: 3 PRBs - Other options are not precluded

According to TR 38.769 [3], the minimum number of PRBs for M=24 is 3:

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 3 |
| 32 | 4 |

Hence, we prefer option 2.

Proposal 6: RAN4 to choose 3PRBs.

- Conclusion

In this contribution we provide our views on demodulation requirements for Ambient-IoT device 1. The proposals are:

Proposal 1: RAN4 to not define PRDCH demodulation requirements for Ambient-IoT.

If RAN4 agrees to define the related PRDCH demodulation requirements for device, we have the following considerations on the test setup:

Proposal 2: RAN4 to consider message 2 and 3 and use following formular for test methodology

BLER=1- Number of responsed Message 3Number of transmitted Message 2

Proposal 3: RAN4 to consider 96 bits TBS

Proposal 4: Use M=24

Proposal 5: Use TDLA30-10

Proposal 6: RAN4 to choose 3PRBs.

- Reference

[1] R4-2512603 Way Forward for [116][335] A-IoT_demod

[2] R4-2505520

[3] TR 38.769