# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2513462

# Prague, CZ, 13-17 Oct, 2025

Source: 	Xiaomi

Title: 	Discussion on RRM core requirements of HD-FDD for Ku band with FR1 numerology

Agenda Item:	7.6.4.2

Document for:	Discussion

# 1Introduction

The new WI of enhanced requirements for NR NTN and IoT NTN Phase 2 [1] has been approved in RANP#109. Based on the arrangement of the agenda for RAN4#116bis, RRM core discussion for HD-FDD can start from this October meeting. The RRM related scope of HD-FDD is:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

According to the WID, phase 1 work of the HD-FDD objective should commence firstly. In this contribution, we discuss the RRM working scope and some preliminary technical parts for the HD-FDD part of this new WI.

- Discussion

Based on the conclusion from previous release discussion, RAN4 introduced several NTN VSAT types:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 64 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. |  |  |

NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247. NOTE 4: The NTN VSAT type 6 is only applicable to bands n509, n508 and n248, n247.

During the WI of NR_NTN_Ku_bands, RAN4 has introduced complete RRM requirements for VSAT UE in FR1-NTN, including:

-	RRC Idle and Inactive mobility

-	Satellite switching with re-synchronization

-	RRC Re-establishment

-	UE transmit timing

-	L3 measurements

-	L1 measurements

-	RLM

-	UE Measurement capability

-	Scheduling and Measurement Restrictions

Based on the guidance in the WI, RAN4 should leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable. Here below, we also list all the RRM agreements achieved for RedCap NTN HD-FDD:

-	Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

-	The HD-FDD UE shall meet the requirements provided that at least 1 SSB is available at the UE in the serving cell during the last 160 ms duration

-	RRC Connection Release with Redirection

-	The HD-FDD UE shall meet the RRC connection release with redirection requirements provided that SSB is available at the UE once every SMTC period during Tsearch.

-	Minimum requirement for L1 indication for RLM and LR

-	The HD-FDD UE shall meet the requirements provided that for each RS configuration, at least one RS sample must fall with DL occasion within an indication period.

-	Scheduling availability of UE during L1-RSRP measurement

-	When NW indicates UL overriding DL for collision handling,

-	RAN4 to clarify that the measurement requirements for CSI-RS and PRS apply provided that no collision between CSI-RS & PRS and UL transmission

-	RAN4 to clarify that measurement requirements for CSI-RS, scheduling restriction of PDSCH, and PRACH apply provided that no collision between CSI-RS/PDSCH and PRACH

  - Otherwise,

  - CSI-RS based L1 measurement and PRS based measurement is prioritized over the UL transmission.

  - The scheduling restriction requirements for PUCCH/PUSCH/SRS will apply.

From our perspective, RAN4 should discuss the HD-FDD related RRM requirement for NTN HD-FDD VSAT, taken the principle in NTN HD-FDD RedCap UE as baseline

Proposal 1: RAN4 to discuss the following HD-FDD related RRM requirement for NTN HD-FDD VSAT, taken the principle in NTN HD-FDD RedCap UE as baseline:

  - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

  - RRC Connection Release with Redirection

  - Minimum requirement for L1 indication for RLM and LR

  - Scheduling availability of UE during L1-RSRP measurement

For the second task in phase 1, RAN4 is requested to introduce requirements for a new VSAT type of HD-FDD Mobile VSAT with small antennas. In our understanding, UE with small sizeo basically would have deteriorated performance compared with the normal size. However, we think RF should start the work at first. RRM can commence the requirement discussion later after the definition of new VSAT type stable.

Proposal 2: The discuss on RRM requirements of HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) is pending on RF progress.

- Conclusion

Proposal 1: RAN4 to discuss the following HD-FDD related RRM requirement for NTN HD-FDD VSAT, taken the principle in NTN HD-FDD RedCap UE as baseline:

  - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

  - RRC Connection Release with Redirection

  - Minimum requirement for L1 indication for RLM and LR

  - Scheduling availability of UE during L1-RSRP measurement

Proposal 2: The discuss on RRM requirements of HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) is pending on RF progress.

- Reference

- RP-252962, Enhanced requirements for NR NTN and IoT NTN Phase 2, MTK, LGE