3GPP TSG-RAN WG4 Meeting #116bis	R4-2513490

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on performance requirements of CSI-RS based L1 measurement

Source: 	Huawei, HiSilicon

Agenda item:	6.17.2.2

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

The contribution provides discussion on the performance requirements for R19 mobility.

Discussion

Accuracy of CSI-RS based L1 RSRP on candidate cells

It is agreed in RAN4#114bis [2] that SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline.

In addition, the conditions of CSI-RS L1-RSRP measurement on candidate cells shall be:

- The bandwidth of CSI-RS is 48 PRBs and the density is 3.

- The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

- The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

Proposal 1: The SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline. The conditions of CSI-RS L1-RSRP measurement on candidate cells are:

- The bandwidth of CSI-RS is 48 PRBs and the density is 3.

- The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

- The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

It is agreed that Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. And as per the intra-frequency definition of CSI-RS based measurement, gap is not needed in intra-frequency measurement.  Then the following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified as well:

- Absolute and relative CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 and FR2

Proposal 2: The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified:

- FR1 Absolute and relative CSI-RS based intra-frequency L1-RSRP measurement

- FR2 Absolute and relative CSI-RS based intra-frequency L1-RSRP measurement

In addition, it is straight forward that the legacy SS/CSI-RSRP measurement L1 report mapping for serving cell specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

Proposal 3: Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

Test case

The accuracy requirements on CSI-RS RSRP L1 measurement on neighbor cell shall be verified. As per the intra-frequency definition of CSI-RS based L1 measurement, gap is not needed. For inter-frequency CSI-RS based L1 measurement, gap is needed. It is also agreed that NOT to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell, therefore it means that no requirements for inter-frequency CSI-RS based L1 measurement.

As the baseline CSI-RS L1 measurement on neighbor cell is “3-step” (i.e., L3 measurement, SSB based L1 measurement, CSI-RS based L1 measurement). Skipping step2 is optional capability. To reduce test number and effort, we suggest to verify 3-step CSI-RS L1-RSRP measurement only.

Proposal 4: The following test cases are specified for CSI-RS based L1 measurement:

- FR1 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP measurement test cases without gap

- FR2 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP measurement test cases without gap

Proposal 5: The following accuracy test cases are specified for CSI-RS L1-RSRP neighbour cell measurement:

- FR1 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases

- FR2 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases

As per UE capability agreed in RAN1 and RAN2, CSI-RS based L1 measurement and LTM event trigger reporting are sperate UE capability. For UE which supports both capabilities, it is only required to pass CSI-RS based L1 measurement event trigger test cases, and skip the test sets of CSI-RS based L1 measurement without reporting.

Proposal 6: For UE which supports both capabilities of CSI-RS based L1 measurement and LTM event trigger reporting, it is only required to pass CSI-RS based L1 measurement event trigger test cases, and skip the test sets of CSI-RS based L1 measurement without reporting.

Conclusion

The contribution provides analysis on performance part. The following proposals are proposed:

Proposal 1: The SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline. The conditions of CSI-RS L1-RSRP measurement on candidate cells are:

- The bandwidth of CSI-RS is 48 PRBs and the density is 3.

- The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

- The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

Proposal 2: The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified:

- FR1 Absolute and relative CSI-RS based intra-frequency L1-RSRP measurement

- FR2 Absolute and relative CSI-RS based intra-frequency L1-RSRP measurement

Proposal 3: Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

Proposal 4: The following test cases are specified for CSI-RS based L1 measurement:

- FR1 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP measurement test cases without gap

- FR2 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP measurement test cases without gap

Proposal 5: The following accuracy test cases are specified for CSI-RS L1-RSRP neighbour cell measurement:

- FR1 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases

- FR2 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases

Proposal 6: For UE which supports both capabilities of CSI-RS based L1 measurement and LTM event trigger reporting, it is only required to pass CSI-RS based L1 measurement event trigger test cases, and skip the test sets of CSI-RS based L1 measurement without reporting.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2508461, WF on RRM requirements for NR_Mob_Ph4_Part1), Apple