3GPP TSG RAN4 Meeting #116bis	R4-2513505

Prague, Czech Republic, October 13 - 17, 2025

Agenda item:		5.5.3

Source:		Union Inter. Chemins de Fer, Nokia

Title:		TP to TR 38.796: HPUE PC2 in FDD band n100

Release:		19

Work Item:	HPUE_NR_FR1_bands_R19-Core

Document for:		Discussion and approval

# Introduction

In RAN4#109 the WID RP-252606 has been approved, allowing completion of the TR 38.795 and TR 38.796 at TSG#110. The objectives of this WID are:

- Objective 1.  For TDD band: PC1.5 and PC2 for handheld UE and FWA, and PC2, PC1.5 and PC1 for FWVM

- Objective 2.  For FDD band: PC2 for handheld UE and FWA, and PC2 and PC1 for FWVM

This enables the introduction of Power Class 2 in FDD band n100 for railway FRMCS cab-radios (i.e. FWVM UE). It is noted that based on EU regulation, railway FRMCS cab-radios for PC2 must meet an ACLR requirement of 37 dB minimum. It is also noted that in TS 38.101-1 PC1 has already been defined for band n100.

The implementation of FRMCS PC2 is assumed to be based on FRMCS PC1 devices making use of high power PAs external to the chipset. The implication of this is that for FRMCS cab-radios at PC2 power levels the existing PC1 requirements apply as described in section 2 below.

For information the following pictures of currently developed FRMCS PC1 cab-radio devices are provided:

This contribution provides a text proposal for TR 38.796 to introduce PC2 for FRMCS cab-radios in band n100.

# Text Proposal

<Start of change>

6	High Power (Power Class 1) FWVM for FDD single band

7	High Power (Power Class 2) FWVM for FDD single band

7.1 	NR band n100

7.1.1 	UE maximum output power

Table 7.1.1-1: UE output power for FRMCS PC2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n100 |  |  |  |  | 261 | +2/-3 | 23 | ±2 |
| NOTE 1: PC2 in Band n100 is allowed only for FRMCS cab-radios, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground. |  |  |  |  |  |  |  |  |

7.1.2 	MPR requirements

For FRMCS PC2, the MPR is defined as per TS 38.101-1 Table 6.2.2-4b: Maximum power reduction (MPR) for power class 1 for bands other than Band n14.

7.1.3 	A-MPR requirements

For FRMCS PC2 no A-MPR values are defined as there are no additional spurious emission requirements for band n100.

7.1.4	Reference sensitivity requirements

For FRMCS PC2 no reference sensitivity degradation is defined as none is defined for PC1 in band n100.

7.1.5 	ACLR requirements

As FRMCS cab-radios are required by EU regulation to have an ACLR of minimum 37dB, a new ACLR table for PC 2 in band n100 is defined:

Table 7.1.5-1: NR ACLR requirement for FRMCS PC2 in band n100

|  | Power class 2 |
| --- | --- |
| NR ACLR | 37 dB |
| NOTE: PC2 in Band n100 is allowed only for FRMCS cab-radios, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground, that also support PC1. |  |

<End of change>

# References

[1]	RP-252435 TR 38.796 V1.0.0

[2]	RP-252606 NR_bands_xFeature_R19-Core

[3]	TS 38.101-1