3GPP TSG-RAN WG4 Meeting #116bis	R4-2513539

Prague, Cezh, 13th – 17th Oct, 2025

Agenda Item:	6.11.3

Source:	CAICT

Title:	Discussion on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

1	Introduction

In this contribution, we provide our views on left FFS in following issues,

- Issue 1-2: Requirement baseline for monitoring

2	Discussion

2.1 Issue 1-2: Requirement baseline for monitoring

In last RAN4 meeting, following agreements had been achieved for this issue,

Issue 1-2: Requirement baseline for monitoring Agreement: Introduce the following requirements for CSI performance monitoring: - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this

Regarding the first FFS point, according to RAN1’s progress in following agreement

Agreement For CSI prediction using UE-side model, for UE assisted performance monitoring, - Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - For monitoring report type, semi-persistent and aperiodic are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported.

SP/AP type are supported monitoring report and P/SP/AP type are supported for monitoring measurement resource. In our understanding, following combination for monitoring report and measurement type should be considered.

| Monitoring report type Monitoring measurement type | SP report | AP report |
| --- | --- | --- |
| Periodic | Support | Support |
| Semi-persistent | Support | Support |
| Aperiodic | Not support | Support |

For SP and AP report, to define the monitoring report delay, the starting point could be the reception of triggering command (DCI or MAC-CE) for monitoring report, and the ending point could be the transmission of first monitoring report. In this way, the overall monitoring report delay includes the monitoring measurement delay, monitoring processing delay and reporting delay, which align with the framework of prediction report delay agreed for beam management. The uncertainty point is the measurement delay that relates to the length of observation window for CSI prediction and is up to the implementation of specific model.

Therefore, an alternative is to take the last RS occasion associated with corresponding monitoring report as the starting point. In this way, the measurement delay is ignored, and only processing delay and reporting delay is considered.

Observation 1: To define the monitoring report delay, there are two possible alternatives,

- Alt 1: take the reception of triggering command as the stating point, and the transmission of first monitoring report as the ending point.

- Alt 2: take the last RS occasion associated with corresponding monitoring report as the start point and the transmission of first monitoring report as the ending point.

Proposal 1: Suggest to discuss whether to consider monitoring measurement delay when defining monitoring report delay.

Regarding to the second FFS point, “FFS on whether accuracy requirement can be defined/checked”. In our understanding, it seems unnecessary to define accuracy requirement for monitoring report. First, from the perspective of definition, if we want to define the accuracy requirement, a reference value for comparison is needed. However, the report content is based prediction and/or measurement, what shall we compare to is unclear if accuracy requirement was to be defined. Second, from the perspective of report content, according to RAN1 agreement, there are two SGCS values are include,

- SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI,

- SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance

In our understanding, the first value could be covered by prediction accuracy requirement and the second could be covered by measurement accuracy requirement.

Observation 2: The accuracy requirement of monitoring report including SGCS, could be reflected by measurement and prediction accuracy.

Proposal 2: The definition and necessity of defining accuracy requirement for monitoring report requires clarification.

Conclusion

In this contribution, Requirement for monitoring of CSI prediction are discussed with following proposals and observations.

Observation 1: To define the monitoring report delay, there are two possible alternatives,

- Alt 1: take the reception of triggering command as the stating point, and the transmission of first monitoring report as the ending point.

- Alt 2: take the last RS occasion associated with corresponding monitoring report as the start point and the transmission of first monitoring report as the ending point.

Proposal 1: Suggest to discuss whether to consider monitoring measurement delay when defining monitoring report delay.

Observation 2: The accuracy requirement of monitoring report including SGCS, could be reflected by measurement and prediction accuracy.

Proposal 2: The definition and necessity of defining accuracy requirement for monitoring report requires clarification.