3GPP TSG-RAN WG4 #116bis	R4-2513548

Prague, Czech Republic, October 13-17, 2025

Agenda item:	6.15.2.1		On-demand SSB SCell operation

Source:	Qualcomm Incorporated

Title:	On performance requirements for on-demand SSB SCell operation

Document for:	Discussion

# Introduction

In RAN4 #116 the work on the core requirements has ended and the work on the performance requirements was started. First agreements for the performance requirements have been made for on-demand SSB, and additional open issues have been identified in [1].

In this paper we provide our view for the open issues identified in the way forward in [1] in RAN4 #116.

# Discussion

First discussions on the performance aspects took place in RAN4 #116 [1]. Several questions remained open for which scenarios test cases should be defined. In the following subsections we address these open topics.

The first open issue (Issue 3-1-1) deals with the question for which scenarios #2, 2#A and #3B performance requirements should be defined

Issue 3-1-1: Scenario - General

- RAN4 to further discuss the test case scope for scenario #2, scenario #2A and scenario #3B as follow.

  - Option 1:

    - In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

    - In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

    - The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

  - Option 2: RAN4 not to define test cases for scenario #3B.

  - Option 3: RAN4 to define the OD-SSB based test cases for

    - Case #1 and Scenario #2

    - Case #2 and Scenario #2A

Scenario #2 refers to a deactivated SCell, scenario #2A means that SCell activation and OD-SBB activation is triggered jointly in different MAC CE messages, and finally scenario #3B refers to the activated SCell.

In scenario #3B the MAC CE is only used for updating the transmission parameters of OD-SSB, but the activation happened earlier. This is a very similar behavior of SSB adaptation. We feel that it is not needed to have a separate performance test for scenario #3B.

Proposal 1: In our view it is not necessary to define performance test cases for scenario #3B.

For scenarios #2 and 2A we think it is not necessary that both scenarios are tested both for Case 1 and Case 2. We do not see a loss in test coverage if scenario #2 is only tested for Case 1 and scenario #2A is only tested for Case 2.

Proposal 2: RAN4 to define the OD-SSB based test cases for scenario #2 in case #1 for scenarios # 2A in Case 2.

The next open issue was whether performance requirements should be defined for Alt Time-C1, Alt Time-C2 and scenario 3 (Issue 3-1-1).

Issue 3-1-1: Scenario - General

- RAN4 to further discuss the test case scope for Case 2 as follow.

- FFS: RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1, Alt. Time C2 and scenario 3 if RAN4 agrees to define the RRM requirements for all these scenarios.

In our understanding Alt Time-C1 (periodic effective time domain pattern of OD-SSB and always-on SSB) is the most natural use case for Case 2. Hence, performance requirements should be defined for Alt Time-C1. The effective periodicity between OD-SSB and always-on SSB should not be less than 10 ms.

Observation 1: In our understanding Alt Time-C1 (periodic effective time domain pattern of OD-SSB and always-on SSB) is the most natural use case for Case 2.

Proposal 3: We propose to define performance requirements for Alt Time-C1 (periodic effective time domain pattern of OD-SSB and always-on SSB). The effective periodicity between OD-SSB and always-on SSB should not be less than 10 ms.

We still have not seen the use case for Alt Time-C2. It can result in an effective SSB burst periodicity less than 5 ms if no additional restrictions are applied. A periodicity less than 5 ms is not a periodicity supported by SSB in the 5G NR specification. Hence, we prefer not to specify requirements for Alt Time-C2.

Observation 2: No clear use case for Alt Time-C2 has been outlined.

Proposal 4: We propose postponing Alt Time-C2 (non-periodic effective time domain pattern of OD-SSB and always-on SSB) and not defining requirements in Rel-19.

In our understanding the use case of scenario 3 (OD-SSB and AO-SSB on different frequencies) is not very clear, and it will become rather complicated to define requirements in the remaining time of Rel-19 since no discussion has taken place so far in RAN4. Hence, we propose postponing this scenario and do not define requirements in Rel-19.

Observation 3: The use case of scenario 3 (OD-SSB and AO-SSB on different frequencies) is not clear.

Proposal 5: We propose postponing scenario 3 (OD-SSB and always-on SSB are within different frequencies) and not defining requirements in Rel-19.

The next open issue is for which SCell activation scenarios to define performance test cases (Issue 3-1-2).

Issue 3-1-2: Scenario – Scell activation scope

- Proposals

  - Proposal 1: RAN4 should focus on SCell activation for case 1 and case 2 Alt Time-C1 and multiple SCells activation. The other SCell activation scenarios should have lower priority.

  - Proposal 2: RAN4 only define the test cases for single SCell activation.

In our understanding only single SCell activation and multiple SCell activation are deployed in the field. All other scenarios for SCell activation are not applied in the field although they have been in TS 38.133 for many years. Hence, we think that activation of multiple SCells should have higher priority, and other SCell activation procedures can have lower priority.

Observation 4: Only single SCell activation and multiple SCell activation are deployed in the field. All other scenarios for SCell activation are not applied in the field although they have been in TS 38.133 for many years.

Proposal 6: RAN4 should focus on SCell activation for single SCell and multiple SCell activation. The other SCell activation test cases should have lower priority.

The next open issue is whether performance test cases for SCell activation for known and/or unknown SCell should be defined (Issue 3-1-3).

In particular for Case 2 we think that the scenario should always be assumed to be a known scenario since always-on SSB is always present. In case of Case 1 when only OD-SSB is present we also think that assuming that the SCell is known is justified since getting L3 measurements for deactivated SCell was the main purpose of the work item. Hence, we propose that test cases are only defined for the case that the SCell is known

Issue 3-1-3: Scenario – known/unknown Scell activation

- Proposal 1: RAN4 to define the OD-SSB based SCell activation test case for both known SCell and unknown SCell.

- Proposal 2: RAN4 to define the OD-SSB based SCell activation test case for known SCell only.

- Proposal 3: RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. One in known scenario and the other in unknown scenario such as:

  - Case 1 in unknown scenario

  - Case 2 in known scenario

.

Proposal 7: RAN4 to define the OD-SSB based SCell activation test case for both known SCell and unknown SCell.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #116. Our observations and proposals are summarized as follows:

Proposal 1: In our view it is not necessary to define performance test cases for scenario #3B.

Proposal 2: RAN4 to define the OD-SSB based test cases for scenario #2 in case #1 for scenarios # 2A in Case 2.

Observation 1: In our understanding Alt Time-C1 (periodic effective time domain pattern of OD-SSB and always-on SSB) is the most natural use case for Case 2.

Proposal 3: We propose to define performance requirements for Alt Time-C1 (periodic effective time domain pattern of OD-SSB and always-on SSB). The effective periodicity between OD-SSB and always-on SSB should not be less than 10 ms.

Observation 2: No clear use case for Alt Time-C2 has been outlined.

Proposal 4: We propose postponing Alt Time-C2 (non-periodic effective time domain pattern of OD-SSB and always-on SSB) and not defining requirements in Rel-19.

Observation 3: The use case of scenario 3 (OD-SSB and AO-SSB on different frequencies) is not clear.

Proposal 5: We propose postponing scenario 3 (OD-SSB and always-on SSB are within different frequencies) and not defining requirements in Rel-19.

Observation 4: Only single SCell activation and multiple SCell activation are deployed in the field. All other scenarios for SCell activation are not applied in the field although they have been in TS 38.133 for many years.

Proposal 6: RAN4 should focus on SCell activation for single SCell and multiple SCell activation. The other SCell activation test cases should have lower priority.

Proposal 7: RAN4 to define the OD-SSB based SCell activation test case for both known SCell and unknown SCell.

# References

- R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1 v1, RAN4 #116, August 2025