3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513554

Prague, Czech, 13th – 17th Oct., 2025

Source:	LG Electronics

Title:	Discussion on remaining issue of RRM requirements for Rel-19 NES

Agenda item:	4.21.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on remaining issue of RRM core requirements for Rel-19 NES.

Discussion

Remaining issue of On-demand SSB requirements

OD-SSB based L3 reporting

Issue 1-4-4: OD-SSB based L3 reporting Agreement The discussion of the issue shall not impact the RAN4 core part completion of the WI.

Regarding OD-SSB based L3 reporting, the intention is to quickly provide report of the measurements results after FMW. In legacy specification, periodic, and event-triggered reporting are specified. From our perspective, legacy reporting mechanism is workable, and no need optimization only for FMW. If faster reporting is needed, NW can configure lower threshold of event-triggered reporting or periodicity of periodic reporting. Besides, if OD-SSB based L3 reporting is introduced, a UE needs to meet the additional reporting delay requirement, which increases complexity without clear benefit.

Proposal 1: RAN4 to consider not to introduce OD-SSB based L3 reporting

Remaining issue of On-demand SIB1 requirements

Impact on Idle measurement

Sub-topic 2-1 RRM requirements impacts for OD-SIB1 Issue 2-1-1: RRM requirements impacts for OD-SIB1 RAN4 agreements in R4-2508441 Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption may be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.. Agreement on Ad hoc: - Keep current agreement and if companies think additional requirements are needed discuss in maintenance stage.

In previous meeting, some company proposed to consider the longer delay in cell-reselection requirement in RRC_IDLE/INACTIVE mode (intra-, inter-frequency, inter-RAT) due to procedure of OD-SIB1. Although RAN4 has already agreed that paging interruption may be longer when the target Cell is an NES Cell, it is unclear whether cell-reselection is affected under the same condition.

The main point is whether a collision can occur when the UE performs measurement in RRC_IDLE/INACTIVE. In paging interruption requirement, paging message of serving cell may be collided when the UE intitaes OD-SIB1 procedure for NES neighbor cell. This is because the serving cell and neighbor cell may not be coordinated. On the other hand, in cell-reselction requirement, usually the requirement is applied for the cell being identified. In our understanding, if the OD-SIB1 procedure is added, the scenario is limited to cases where the target cell is an NES cell, and the UE performs both measurements and the OD-SIB1 procedure on the same NES cell. It can be assumed that proper NES cell would avoid collision between OD-SIB1 and measurements by scheduling. Therefore, it is not necessary to consider longer delay for cell-reselction requirement when the target cell is NES cell.

Proposal 2: RAN4 to consider not to consider longer delay in cell-reselection requirement

Conclusion

In this contribution, we provide views on RRM requireiments for on-demand SSB SCell operation, and we propose

Proposal 1: RAN4 to consider not to introduce OD-SSB based L3 reporting

Proposal 2: RAN4 to consider not to consider longer delay in cell-reselection requirement

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2508277, “WF on RRM requirements for Netw_Energy_NR_enh_Part1,” Ericsson