# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513569

# Prague, CZ, 13th – 17th Octobor, 2025

Source: 	vivo

Title: 	Initial views on RF requirements for support of less than 5 MHz channel bandwidth for (e)RedCap UE

Agenda Item:	7.4.3.1

Document for:	Approval

Introduction

In RAN#109, WID for NR_RRM_Ph6 [1] was approved [1]. One objective of this WID is to specify RF requirements for support of less than 5 MHz channel bandwidth, considering example bands: n100, n106, n26, n28 and n85. In this contribution, we provide our initial views on RF requirements impact for support of 3MHz for (e)RedCap UE.

Discussion

The objective for support of less than 5MHz channel bandwidth for (e)RedCap UE is captured as follows. In this section, we focus on the RF requirements part.

- Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

From our perspective, there isn’t much work to do for the RF requirements part for introduction of 3MHz for (e)RedCap UEs. Practically most of requirements defined for 3MHz non-RedCap in band n100, n106, n26, n28 and n85 can be inherited for (e)RedCap UEs.

For system parameters for 3MHz, we didn’t see the difference for non-RedCap UE and (e)RedCap UE. Currently maximum transmission bandwidth configuration 15PRB was defined for 3MHz and 12 PRB SSB was considered and separate GSCN parameters for 3MHz channel bandwidth were defined. The only change we need to make in TS 38.101-1 is to clarify 3MHz is applicable for (e)RedCap UE in R20:

- 5.3I Channel bandwidth for (e)RedCap The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 with maximum channel bandwidth of 20MHz. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths up to 20 MHz. When UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 the requirements in this specification apply with maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths. 3MHz channel bandwidth is not applicable for (e)RedCap UE in the current release.

For Tx requirements, we didn’t see any specification impact. For maximum output power, we suggest to adopt the same power class for non-RedCap and (e)RedCap. For MPR requirement, it is channel bandwidth agnostic and there is no specification impact. For other RF requirements, there is no difference between RedCap and non-RedCap.

For (e)RedCap UE, HD-FDD RedCap UE with 1Rx and 2Rx were defined, however, there is no REFSENS value for 3MHz. It is suggested to define 3MHz REFSENS for HD-FDD (e)RedCap UE with 1Rx and 2Rx.

After going through all the RF requirements, the following RF impacts are summarized:

Table 1: Summary of RF impacts for support of 3MHz for (e)RedCap

| Requirements for (e)RedCap in TS 38.101-1 | Proposal for NR 3 MHz |
| --- | --- |
| 5.3I Channel bandwidth for (e)RedCap | The following sentence needs to be changed: ’3MHz channel bandwidth is not applicable for (e)RedCap UE in the current release.’ |
| 5.4I Channel arrangement for (e)RedCap | No specification impact |
| 6.2I Transmitter power for (e)RedCap | No specification impact |
| 7.3I Reference sensitivity for (e)RedCap | REFSENS needs to be defined for 3MHz considering 1Rx and 2Rx in HD-FDD mode |

Observation 1: For support of less than 5MHz channel bandwidth for (e)RedCap UEs, the work for RF requirements is small considering most of the requirements defined for non-RedCap UE can be reused for (e)RedCap UE.

Proposal 1: The spec impact for support of 3MHz for (e)RedCap is summarized as shown in Table 1.

Conclusion

This contribution discusses RF impacts for support of 3MHz for (e)RedCap. The following observations and proposals are made:

Observation 1: For support of less than 5MHz channel bandwidth for (e)RedCap UEs, the work for RF requirements is small considering most of the requirements defined for non-RedCap UE can be reused for (e)RedCap UE.

Proposal 1: The spec impact for support of 3MHz for (e)RedCap is summarized as shown in Table 1.

Table 1: Summary of RF impacts for support of 3MHz for (e)RedCap

| Requirements for (e)RedCap in TS 38.101-1 | RF requirements impact |
| --- | --- |
| 5.3I Channel bandwidth for (e)RedCap | The following sentence needs to be changed: ’3MHz channel bandwidth is not applicable for (e)RedCap UE in the current release.’ |
| 5.4I Channel arrangement for (e)RedCap | No specification impact |
| 6.2I Transmitter power for (e)RedCap | No specification impact |
| 7.3I Reference sensitivity for (e)RedCap | REFSENS needs to be defined for 3MHz considering 1Rx and 2Rx in HD-FDD mode |

- References

[1] RP-252955, New WID: NR RRM enh Phase 6, moderator (Apple), RAN#109.