3GPP TSG-RAN WG4 Meeting #116bis	R4-2513570  
Prague, Czech, Oct. 13-17, 2025

Agenda item:	7.3.2

Source:	vivo, CAICT

Title:	Workplan of Rel-20 OTA WI

Document for:	Approval

1	Introduction

Objectives for this core part work item are as follows

- Define enhanced test methodologies to cover NTN Ka and Ku bands

  - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing

    - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority

    - FR1 OTA test method extension is not precluded, if also applicable

  - Study whether and how to support circular polarization measurements

    - Linear polarization is the baseline

  - Specify the required quiet zone to cover VSAT device type based on industry needs

    - 60cm QZ as a starting point

  - Specify preliminary MU assessment for test methodologies

During the course of this work item, ongoing communication with 3GPP RAN WG5, CTIA OTA Working Group, CCSA TC9, GCF, GSMA TSG-AP, ETSI MSG TFES, and PTCRB shall be maintained to ensure industry coordination on this topic.

Objectives for this performance part work item are as follows

- Specify FR1 TRP and TRS requirements for new bands

  - Handheld UE is the first priority.

  - Bands n20, n38, n79 for 1Tx, n41 and n78 for 2Tx non-coherent UE

    - Maximum 4 bands are targeted for introducing requirements depending on measurement data availability

    - Handheld UEs width >=56mm and <=92mm

Note: Check in RAN #112, whether to include other UE types as second priority if a sufficient number of commercial devices and new phantom (if needed) become available.

- Specify FR1 MIMO OTA requirements for new bands based on static test method

  - Applicable number of layers for targeting bands

    - 2 layers: band n20

    - 4 layers: band n38, n79

  - n20, n38, n79 are first priority

    - Maximum 3 bands are targeted for introducing requirements depending on measurement data availability

  - Handheld UE is the first priority

Objectives for this core part work item are as follows

- Define enhanced test methodologies to cover NTN Ka and Ku bands

  - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing

    - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority

    - FR1 OTA test method extension is not precluded, if also applicable

  - Study whether and how to support circular polarization measurements

    - Linear polarization is the baseline

  - Specify the required quiet zone to cover VSAT device type based on industry needs

    - 60cm QZ as a starting point

  - Specify preliminary MU assessment for test methodologies

During the course of this work item, ongoing communication with 3GPP RAN WG5, CTIA OTA Working Group, CCSA TC9, GCF, GSMA TSG-AP, ETSI MSG TFES, and PTCRB shall be maintained to ensure industry coordination on this topic.

Objectives for this performance part work item are as follows

- Specify FR1 TRP and TRS requirements for new bands

  - Handheld UE is the first priority.

  - Bands n20, n38, n79 for 1Tx, n41 and n78 for 2Tx non-coherent UE

    - Maximum 4 bands are targeted for introducing requirements depending on measurement data availability

    - Handheld UEs width >=56mm and <=92mm

Note: Check in RAN #112, whether to include other UE types as second priority if a sufficient number of commercial devices and new phantom (if needed) become available.

- Specify FR1 MIMO OTA requirements for new bands based on static test method

  - Applicable number of layers for targeting bands

    - 2 layers: band n20

    - 4 layers: band n38, n79

  - n20, n38, n79 are first priority

    - Maximum 3 bands are targeted for introducing requirements depending on measurement data availability

  - Handheld UE is the first priority

3GPP RAN Plenary approved the new Rel-20 OTA WI [1] in the most recent RAN#109 meeting held in Beijing, Sep 2025. To ensure smooth progress of the Work Item, it is therefore important to produce a work plan to facilitate the discussions in RAN4.

In this paper, we share the workplan of Rel-20 OTA WI to guide the detailed work targets for each meeting.

2	Workplan for Rel-20 OTA WI

We propose the workplan for Rel-20 OTA WI as following:

- RAN4 #116bis (Oct 2025)

- Approve the workplan

- Endorse initial skeleton for new TR

- Core part started

- Initial views on NTN test methodology

- RAN4 #117 (Nov 2025)

- Agree skeleton for TR

- Core part

- NTN test methodology

  - Make progress on extending existing test methodologies for NR NTN Ka and Ku bands

  - Make progress on supporting circular polarization

  - Make progress on QZ and min measurement distance

  - Start discussion of preliminary MU

- RAN4 #118 (Feb 2026)

- Make progress on TR drafting

- Core part

- NTN test methodology

  - Make progress on extending existing test methodologies for NR NTN Ka and Ku bands

  - Make progress on supporting circular polarization

  - Make progress on QZ and min measurement distance

  - Make progress on preliminary MU

- RAN4 #118-bis (Apr 2026)

- Make progress on TR drafting

- Core part

- NTN test methodology

  - Make progress on extending existing test methodologies for NR NTN Ka and Ku bands

  - Make progress on supporting circular polarization

  - Make progress on QZ and min measurement distance

  - Make progress on preliminary MU

- Performance part

- TRP TRS requirements

  - Initial views on Rel-20 FR1 TRP TRS requirement

- MIMO OTA requirements

  - Initial views on Rel-20 MIMO OTA requirement

- RAN4 #119 (May 2026)

- Make progress on TR drafting

- Core part

- NTN test methodology

  - Make progress on extending existing test methodologies for NR NTN Ka and Ku bands

  - Make progress on supporting circular polarization

  - Make progress on required QZ and min measurement distance

  - Make progress on preliminary MU

- Performance part

- TRP TRS requirements

  - Make progress on framework and prioritized bands for Handheld UE FR1 TRP TRS requirement

- MIMO OTA requirements

  - Make progress on framework and prioritized bands for Handheld UE FR1 MIMO OTA requirement

RAN-Plenary #112 (June 2026)

- Check in RAN plenary whether to include other UE types as second priority if a sufficient number of commercial devices and new phantom (if needed) become available.

- RAN4 #120 (Aug 2026)

- Make progress on TR drafting

- Core part

- NTN test methodology

  - Make conclusions on feasible test methodologies for NR NTN Ka and Ku bands. Work on details of the concluded test methods, e.g., test setup, procedure, configurations, and others

  - Conclude on supporting circular polarization or not

  - Conclude on required QZ and min measurement distance

  - Make progress on preliminary MU

- Performance part

- TRP TRS requirements

  - Make progress on framework and prioritized bands for Handheld UE FR1 TRP TRS requirement

  - Start discussion of framework and prioritized bands for other UE types (if concluded in RAN, also follow-up work for future meetings)

- MIMO OTA requirements

  - Make progress on framework and prioritized bands for Handheld UE FR1 MIMO OTA requirement

- RAN4 #120bis (Oct 2026)

- Make progress on TR drafting

- Core part

- NTN test methodology

  - Make further progress and discuss TPs on details of the concluded test methods, e.g., test setup, procedure, configurations, and others

  - Discuss TPs for circular polarization (if supported), required QZ, min measurement distance

  - Conclude and discuss TPs on preliminary MU assessment

- Performance part

- TRP TRS requirements

  - Conclude framework and prioritized bands for Handheld UE FR1 TRP TRS requirement

  - Start the measurement campaign of handheld after this meeting

  - Make progress of framework and prioritized bands for other UE types

- MIMO OTA requirements

  - Conclude framework and prioritized bands for Handheld UE FR1 MIMO OTA requirement

  - Start the measurement campaign of handheld after this meeting

- RAN4 #121 (Nov 2026)

- Conclude main part of TR drafting

- Core part

- NTN test methodology

  - Conclude main part of TPs on test methodologies including e.g., test setup, configuration, procedure, applicability, supported UE types

  - Conclude main part of TPs for supporting circular polarization (if supported), required QZ, min measurement distance

  - Conclude main part of TPs on preliminary MU assessment

- Performance part

- TRP TRS requirements

  - Make progress on measurement campaign and discuss Handheld UE requirements

  - Conclude framework and prioritized bands for other UE types

- MIMO OTA requirements

  - Make progress on measurement campaign and discuss Handheld UE requirements

- RAN4 #122 (Feb 2027)

- Finalize the TR drafting

- Conclude Core part

- Conclude all NTN Ka and Ku bands test methodology related issues, including all TP/CRs

- Performance part

- TRP TRS requirements

  - Make progress on measurement campaign and discuss Handheld UE requirements

  - Start measurement campaign for other UE types

- MIMO OTA requirements

  - Make progress on measurement campaign and discuss Handheld UE requirements

RAN-Plenary #115 (Mar 2027)

- Release the TR for NR NTN above 10GHz

- Core part closed

- Continue performance part

- RAN4 #122bis (Apr 2027)

- Performance part

- TRP TRS requirements

  - Make progress on measurement campaign and discuss Handheld UE requirements

  - Make progress on measurement campaign for other UE types

- MIMO OTA requirements

  - Make progress on measurement campaign and discuss Handheld UE requirements

- RAN4 #123 (May 2027)

- Performance part

- TRP TRS requirements

  - Collect all measurement data of handheld UEs and specify the requirements

  - Make progress on measurement campaign for other UE types

- MIMO OTA requirements

  - Collect all measurement data of handheld UEs and specify the requirements

- RAN4 #124 (Aug 2027)

- Performance part

- Finalize Handheld UE TRP TRS requirements, including CRs

- Finalize Handheld UE MIMO OTA requirements, including CRs

- Collect all measurement data of other UEs, and finalize the requirements

- Conclude performance part

RAN-Plenary #116 (Sep 2027)

- Approve all Cat B CRs

- Performance part closed

- Close the WI

3	Conclusions

In this contribution, we provide workplan for Rel-20 OTA WI. Our proposal is as following:

Proposal: Approve the proposed workplan for Rel-20 OTA WI.

4	References

- RP-252903, WID: Enhancement of UE OTA test method and requirements for NR, RAN VC (vivo), RAN meeting #109, Sep 2025.