3GPP TSG-RAN WG4 Meeting #116bis                                             				R4-2513579

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	7.6.4.2

Source:		Eutelsat, THALES

Title:		HD-FDD implications for Ku and Ka band RRM

Document for:		Discussion/ Decision

# Introduction

At RAN#109 a new WID [1] with the following scope was agreed:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note : Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

# Overview

This contribution considers the impact on TS 38.133 [2] from the introduction of HD-FDD VSAT in Rel-20. The emphasis of this contribution is on Phase 1. Phase 2 aspects are also briefly considered and commonalities with Phase 1 work identified.

# Discussion

## HD-FDD

- At present the 3GPP specifications limit the use of FR1-NTN in bands above 10 GHz to the Ku band; Ka band only supports FR2-NTN numerology which is outside the scope of Phase 1 the WID agreed at RAN#109

- We fully support the Phase 2 work but recognise this will follow Phase 1 as this may have some impact on RAN1 specifications; in particular to add support for 120 kHz SCS operation.

Observation 1 : In Phase 1, work RAN4 should focus on Ku band using FR1-NTN numerology.

- Ku band using FR2-NTN numerology and Ka band (FR2-NTN numerology) will follow upon completion of Phase 1.

- It is expected that HD-FDD operation will enable smaller size flat panel antennas to be feasible.

## RRM implications

Key points:

- As is the case for presently specified FD-FDD VSAT, HD-FDD VSAT can either receive from the serving SAN or a transmit to the serving SAN at any one time.

- As for the presently specified FD VSAT UE, due to the directional antenna acting as a spatial filter, more than one SAN can illuminate the same area on the ground without requiring frequency reuse (as illustrated in Figure 1).

  - Note: As is the case for serving UE with omnidirectional antennas (e.g., handheld devices), SAN A and SAN B could also use different channel frequencies when serving VSAT UE.

Figure 1: Cell area illuminated by two SAN

- Additionally, as is the case for FD-FDD VSAT UE, a HD-FDD VSAT capable of fast receive beam switching [or dual beam capability – a possible implementation not presently specified by 3GPP] may be able to ‘measure’ signals from more than one SAN.

- As for FD-UE, the network can assist the VSAT UE measurement process by using Synchronization Signal and PBCH Block (SSB) based Measurement Timing Configuration “SMTC”. SMTC can be used by the network to insert a sufficient gap between the SSB from SAN A and the SSB from SAN B to avoid the SSB from two (or more) satellites overlapping in time as seen by all UE located anywhere in the cell area.

  - Note: Rel-19 specifications permits up to 4 SMTC to be configured [Reference?]

- The difference between HD-UE and FD-UE is that the HD VSAT UE will not be able to measure signals from the target SAN whilst it is transmitting to the serving SAN. This is a similar limitation as for eRedCap operating in NTN paired spectrum bands.

- RAN4 RF will specify the permitted T/R switching time as part of the work to update [3].

Observation 2: The HD-FDD VSAT can be assumed to have the same fast beam switching capability as a FD VSAT UE, however, RRM for HD-FDD VSAT needs to take the HD and Transmit/ Receive switching time into account.

Observation 3: HD-FDD VSAT operating in Ku band based on FR1-NTN numerology (15 kHz or 30 kHz) differ from eRedCap NTN devices as they have a directional antenna.

Proposal 1: The starting point for HD-FDD VSAT RRM should be to consider reusing eRedCap NTN and the Ku band RRM requirements related to directional antennas.

- RRM for mobile VSAT served by LEO (Type 6) will be the most demanding case to specify (moving VSAT platform, non-stationary satellite).

Observation 4 : As all types can benefit from the RRM work for Type 6 there appears to be no need to explicitly consider other VSAT types.

Proposal 2: Initial RRM work in Phase 1 should consider VSAT UE Type 6 (mobile VSAT served by LEO).

Proposal 3: RAN4 to discuss the RRM measurement accuracy (if any impact), due to reduced Rx time for HD-FDD VSAT.

Proposal 4: RAN4 to discuss maximum interruption in paging reception (if any impact), due to reduced Rx time for HD-FDD VSAT.

Proposal 5: RAN4 to discuss if any update required for handover requirements for HD-FDD VSAT, due to reduced Rx time, mainly with respect to SSB availability.

Proposal 6: RAN4 to discuss if any inter and/or intra-measurement requirement update for HD-FDD VSAT, due to Tx/Rx pattern.

Proposal 7: RAN4 to discuss if any specific update of requirement for HD-FDD VSAT with respect to initial access/random access, due to reduced Tx time.

Proposal 8: RAN4 to discuss if any timing error impact for HD-FDD VSAT, due to Tx/Rx pattern.

Proposal 9: RAN4 to discuss scheduling availability with respect to HD-FDD VSAT if any impact for measurement or other requirements.

Proposal 10: RAN4 to discuss specific test configurations for HD-FDD VSAT.

# Conclusions

An analysis of the impact of HD-FDD on the operation of VSAT UE above 10 GHz has been discussed and the following observations are made and proposals for future work are offered.

Observation 1 : In Phase 1, work RAN4 should focus on Ku band using FR1-NTN numerology.

Observation 2: The HD-FDD VSAT can be assumed to have the same fast beam switching capability as a FD VSAT UE, however, RRM for HD-FDD VSAT needs to take the HD and Transmit/ Receive switching time into account.

Observation 3: HD-FDD VSAT operating in Ku band based on FR1-NTN numerology (15 kHz or 30 kHz) differ from eRedCap NTN devices as they have a directional antenna.

Proposal 1: The starting point for HD-FDD VSAT RRM should be to consider reusing eRedCap NTN and the Ku band RRM requirements related to directional antennas.

Observation 4 : As all types can benefit from the RRM work for Type 6 there appears to be no need to explicitly consider other VSAT types.

Proposal 2: Initial RRM work in Phase 1 should consider VSAT UE Type 6 (mobile VSAT served by LEO).

Proposal 3: RAN4 to discuss the RRM measurement accuracy (if any impact), due to reduced Rx time for HD-FDD VSAT.

Proposal 4: RAN4 to discuss maximum interruption in paging reception (if any impact), due to reduced Rx time for HD-FDD VSAT.

Proposal 5: RAN4 to discuss if any update required for handover requirements for HD-FDD VSAT, due to reduced Rx time, mainly with respect to SSB availability.

Proposal 6: RAN4 to discuss if any inter and/or intra-measurement requirement update for HD-FDD VSAT, due to Tx/Rx pattern.

Proposal 7: RAN4 to discuss if any specific update of requirement for HD-FDD VSAT with respect to initial access/random access, due to reduced Tx time.

Proposal 8: RAN4 to discuss if any timing error impact for HD-FDD VSAT, due to Tx/Rx pattern.

Proposal 9: RAN4 to discuss scheduling availability with respect to HD-FDD VSAT if any impact for measurement or other requirements.

Proposal 10: RAN4 to discuss specific test configurations for HD-FDD VSAT.

# References

[1]  RP-252962 New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, RAN#109, Sept. 2025

[2] TS 38.133 NR; Requirements for support of radio resource management (release 19), v19.1.1, June 2025

[3] TS 38.101-5 User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 19) v19.1.1, June 2025