# 3GPP TSG-RAN WG4 Meeting #116bis                                                              R4-2513582

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.11.2

Source:	Apple

Title:	On general issues on AI/ML

Document for:	Discussion

# 1	Introduction

In this contribution, we further discuss the remaining issues related to:

- post-deployment pre/post activation functionality testing, and

- LCM requirements.

# 2	Post-deployment pre/post activation functionality testing and LCM

As AI/ML models become increasingly integrated into NR (New Radio) air interface functionalities, particularly on the UE side, ensuring their sustained performance post-deployment poses new challenges for 3GPP. After a model is deployed in the field especially when it has been updated or fine-tuned it may no longer be practical or even possible to subject the device to full RAN5 conformance testing again. This creates a need for alternative mechanisms to ensure that the updated model continues to perform reliably and does not degrade the UE's behavior or violate compliance standards.

The critical problem is how to validate AI/ML models after deployment in a way that provides confidence to vendors, operators, and test authorities without introducing unmanageable testing burdens, excessive signaling overhead, or compromising performance monitoring reliability.

During RAN4#114bis, there was not much discussion on the topic, apart from agreeing a naming update.

Agreement:

- Rename “option 1” as “Post-deployment pre-activation functionality/configuration update testing”

- Rename “option 2” as “Post-deployment post-activation functionality testing based on performance monitoring”

## Option 1: Post-Deployment Pre-Activation Functionality / Configuration Update Testing

Option 1 is a validation process performed before activating an AI/ML model update in deployed UEs. The intent is to ensure that any new or modified model still meets the performance expectations established during initial RAN5 conformance testing, taking into account the hardware characteristics of the target UE. This validation can occur in a controlled laboratory environment or within the model development facility, where realistic device behavior is emulated. By doing so, the testing environment can reproduce the hardware constraints, RF front-end behavior, and processing latency that the model will encounter in the field.

The approach depends on defining clear triggers for when pre-activation testing is required. It is important to distinguish between major and minor model changes. Major changes, such as a full retrain or architectural modification, are more likely to require this form of testing, whereas fine-tuning of weights with unchanged architecture might be exempt. However, the criteria for making this distinction remain an open standardization question. Determining whether such classification is standardized or left to the UE vendor has significant implications for enforceability and interoperability.

Another critical consideration in Option 1 is whether the testing process itself is executed on the UE hardware, on a lab replica, or via simulation. Laboratory testing can guarantee controlled conditions but may introduce long turnaround times and significant costs, especially if updates are frequent. On-device validation, by contrast, allows faster iteration but risks variability due to hardware differences and makes independent verification more difficult. These trade-offs are central to deciding the practicality and scalability of Option 1 in commercial deployments

In the post-deployment context, on-device validation refers to testing an updated AI/ML model directly inside the actual UE, either in the field or in a controlled test mode, before it is activated for live use. In this setup, the new model can operate in a “shadow” mode alongside the active model, receiving the same inputs but without influencing live network decisions. The UE collects performance metrics ,such as beam prediction accuracy or RSRP differences , using either pre-loaded test datasets or duplicated live measurements. These KPIs are then evaluated against pre-defined thresholds to decide whether the model can be activated. This method is fast, captures the true behavior of the UE’s RF and processing hardware, and is well-suited for minor updates or fine-tuning where quick validation is important. However, it is difficult for the network or certification bodies to independently verify results, and outcomes may vary between UE hardware versions. It also assumes the UE has enough compute and memory resources to run two models in parallel during validation.

Lab validation, by contrast, is performed in a controlled and reproducible environment , typically in a vendor’s AI/ML development facility or a certification lab, using either the target UE hardware or an accurate hardware emulator. The updated model is tested with standardized datasets and well-defined emulated conditions for channel fading, interference, Doppler, and measurement impairments, often matching the profiles used in RAN5 conformance testing. This allows direct comparison with baseline KPIs from the original conformance stage and ensures the results are verifiable by third parties. Lab validation avoids any risk of degrading live network performance during testing and is easier to standardize across vendors, since the environment and procedures are fixed.

While on-device validation is quicker and more representative of real-world device performance, it lacks the reproducibility and independent oversight of lab-based methods. Lab validation offers strong repeatability and regulatory trust but at the cost of slower turnaround, higher expense, and potentially missing device-specific quirks that only appear in the field.

## Option 2, Post-Deployment Post-Activation Functionality Testing Based on Performance Monitoring (LCM)

Option 2 moves the assurance process into the operational phase, monitoring the model’s performance in real-world conditions after activation. This approach leverages Life Cycle Management (LCM) procedures and relies on continuous or periodic collection of KPIs from the UE. The objective is to detect degradation early, identify root causes, and trigger recovery actions such as model switching, fallback to legacy algorithms, or retraining. The monitoring configuration, e.g., CSI-RS/SSB resource sets, measurement intervals, and thresholds, is signaled by the network and may align with RAN1-defined inference timing.

In beam management, RAN1 has already agreed on a Top-M vs. Top-K beam prediction accuracy metric for UE-assisted monitoring. In other use cases, like CSI prediction or positioning, intermediate KPIs or difference metrics are being considered. However, RAN4’s test metrics and RAN1’s monitoring metrics are not always aligned, which could complicate using monitoring results as a direct proxy for performance validation. Consistency across UE implementations is also a challenge, differences in analogue front-ends, filtering, or averaging methods could make KPI comparisons non-uniform unless RAN4 defines strict reporting requirements.

Option 2,  post-deployment monitoring via LCM, is attractive because it scales well to frequent model updates and evaluates performance in real network conditions without the delays of lab-based testing. However, running continuous monitoring in the field can create challenges: if KPI reporting thresholds are set too tightly, normal stochastic fluctuations in AI/ML outputs may trigger excessive reporting and even unnecessary fallback, increasing both signaling overhead and processing load on the UE and network.

To address this, different companies propose several mitigations: use relaxed thresholds during an initial “validation mode” and tighter thresholds in normal operation, apply multi-M evaluation (e.g., Top-1, Top-2, Top-4 beam accuracy) to enrich KPI data without extra signaling, and temporarily disable fallback in the early validation phase to avoid premature deactivation of a model that is still performing adequately. These measures aim to preserve Option 2’s scalability while reducing false triggers and minimizing overhead.

## Pros and Cons of Option 1 and Option 2

Option 1 offers the advantage of controlled, reproducible testing conditions and the ability to verify model behavior before it impacts live network performance. It avoids the risk of service degradation due to deploying an underperforming model and can be tied directly to conformance-like pass/fail criteria. However, it can be slow, costly, and operationally heavy, especially if frequent updates are expected. Standardizing major/minor change criteria is non-trivial, and on-device validation lacks the transparency and reproducibility of lab testing.

Option 2 enables real-time validation in the actual deployment environment, inherently capturing the effects of real-world conditions such as interference, mobility, and device variability. It scales well to frequent updates and can be integrated into existing LCM frameworks. At the same time, it depends heavily on robust metric design, interoperability of KPI reporting, and careful threshold tuning to avoid false triggers. Without alignment between performance testing and monitoring metrics, Option 2 risks ambiguity in determining whether a performance drop is due to the model update or external factors.

## Proposal Analysis: Hybrid Post-Deployment Validation

### 1. Problem Statement

Post-deployment AI/ML model updates present two competing needs:

- Assurance: Updated models must meet performance expectations comparable to those established in RAN5 conformance testing.

- Agility: The framework must support frequent updates without introducing excessive delay, cost, or complexity.

Option 1 (pre-activation testing) ensures assurance but can be slow, costly, and impractical for frequent updates. Option 2 (post-activation LCM monitoring) is scalable but risks false triggers, overhead, and delayed detection if not carefully designed.

Option 1 (pre-activation testing) ensures assurance but can be slow, costly, and impractical for frequent updates. Option 2 (post-activation LCM monitoring) is scalable but risks false triggers, overhead, and delayed detection if not carefully designed

### 2. Proposal Overview

The hybrid approach combines the strengths of both options:

- Stage 1 : Pre-Activation Testing (Option 1 Layer):
Applied only to major updates (full retrain, architecture change, high-impact fine-tuning, or critical KPI use cases). Testing occurs in either a lab or on-device with a standardized, verifiable procedure. This establishes a baseline KPI envelope for post-activation comparison.

- Stage 2 : Post-Activation Monitoring (Option 2 Layer):
Applied to all updates, including those that underwent Stage 1. Continuous KPI monitoring is configured through LCM, using metrics aligned with RAN1 definitions (e.g., Top-M vs Top-K beam prediction accuracy for beam mgmt).

In the hybrid approach, the post-deployment stage is the overall lifecycle stage after a model is activated in the live network , it covers everything that happens once the model leaves the lab or pre-activation test phase.

We provide an example below:

Example: Hybrid Post-Deployment Validation Process for Five Dedicated Models

### Step 1: Example of Five Dedicated Models

- Model A: Urban Macro, Low Doppler, CDL-D

- Model B: Urban Macro, High Doppler, CDL-D/E

- Model C: Urban Micro, Low Doppler, CDL-A

- Model D: Rural Macro, Low Doppler, CDL-F

- Model E: High-Speed Train, High Doppler, CDL-E

Each model is deployed in the UE with its own KPI envelope, derived from initial RAN5 conformance testing or controlled lab validation.

### Step 2: Major vs. Minor Updates for Dedicated Models

Major update (within-scenario change):

- Full retrain with new architecture or altered output format.

- Inclusion of previously unseen channel model or hardware front-end in that scenario.

- Large dataset expansion that significantly changes the statistical distribution.

- Algorithmic change to the inference pipeline (e.g., adding a new feature extraction block).

Minor update (within-scenario change):

- Fine-tuning with additional data from the same network/operator.

- Small parameter calibration or bug fix.

- Limited dataset refresh without altering distribution characteristics.

### Step 3: Example Update Classification

- Model A: Fine-tuned with new drive-test data in an existing network → Minor.

- Model B: Retrained with new CDL-E traces from a vendor RF front-end not in original dataset → Major.

- Model C: Adjusted normalization for updated power control in a known network → Minor.

- Model D: Expanded training set to include a new rural macro operator with different CDL delay spreads → Major.

- Model E: Architecture change from CNN to Transformer for high-Doppler handling → Major.

### Step 4: Hybrid Approach Execution

A. Pre-Activation Stage (Option 1 Layer)

- Major updates (B, D, E):

  - Lab validation preferred for architecture or RF profile changes (B, E).

  - On-device validation possible for dataset distribution changes without hardware changes (D, if test mode available).

  - Updated KPI envelopes generated after validation.

- Minor updates (A, C):

  - Skip lab stage and proceed directly to activation with immediate post-activation monitoring.

B. Post-Activation Stage (Option 2 Layer)

- All five models enter post-activation LCM monitoring after activation.

- Even major updates validated pre-activation still undergo post-activation monitoring to detect performance drift in real network conditions (channel variability, interference, hardware aging, data drift).

- KPI envelope from pre-activation becomes the baseline for live KPI comparisons, triggering fallback, retraining, or threshold adjustments when deviations occur.

Hybrid logic:

- Major update → Pre-activation validation → Post-activation monitoring (using KPI envelope from validation).

- Minor update → Skip pre-activation → Post-activation monitoring (using KPI envelope from previous deployment or initial conformance).

The technical rationale is clear: pre-activation testing is reserved for high-risk updates, reducing the volume of lab or vendor-facility testing while still protecting against degraded deployments.

Overall, this hybrid process reduces operational delays by avoiding mandatory lab testing for minor updates, maintains high assurance for major updates, captures real-world performance effects in the field, and minimizes signaling overhead. It also aligns with ongoing RAN1 work on metric definitions while preserving RAN4’s role in ensuring KPI interoperability.

Pre-activation validation acts as a gatekeeper. It stops obviously broken or high-risk models from ever reaching the live network. For major updates, such as architecture changes, new RF front-end profiles, or large distribution shifts , the potential for severe KPI degradation is high. If we skipped this step, those failures would surface only after activation, meaning the live network and end users would experience degraded performance until the monitoring system detected it and fallback was triggered. In a mobility or beam management context, that could mean seconds to minutes of mis-steering, dropped calls, or inefficient handovers before recovery.

Post-activation monitoring, in contrast, is about lifetime assurance under real network conditions. Even a model that passed pre-activation validation perfectly can degrade over time due to factors that were absent from the test dataset , e.g., new interference patterns, weather impacts, hardware aging, or seasonal usage shifts. Without ongoing monitoring, these long-term drifts would go unnoticed.

In short: pre-activation protects the network from immediate catastrophic failures at the moment of deployment, while post-activation protects against gradual or unforeseen degradation during the model’s operational life. Both are needed in the hybrid approach to cover different risks and time horizons.

Pre-activation protects the network from immediate catastrophic failures at the moment of deployment, while post-activation protects against gradual or unforeseen degradation during the model’s operational life. Both are needed in the hybrid approach to cover different risks and time horizons

However, in order for Option 2 to prove useful for post-deployment the UE should support shadow inference.

Shadow inference is when a model runs in parallel with the active, production model but its outputs are never used for live decision-making.

Here’s how it works in the RAN4 AI/ML context:

- The UE’s active model handles all real-time inference for beam prediction, mobility, etc.  its outputs drive network procedures.

- At the same time, one or more standby models (e.g., different adapters, updated versions) receive the exact same input data stream as the active model.

- These standby models produce predictions “in the shadows”   meaning the results are captured for KPI evaluation but never applied to the network.

- KPIs from these shadow models (e.g., beam ID accuracy, RSRP error) are compared to genie references or to the active model’s performance.

If shadow inference is not supported, Option 2 post-activation monitoring loses most of its value. This is because monitoring in Option 2 can only collect live KPI data from the model currently serving inference; any other models  stored on the UE remain untested during actual network operation. Without shadow inference, there is no mechanism to detect drift, degradation, or dataset mismatch in standby models until they are actively swapped in for inference  at which point a performance problem could directly affect service quality. In other words, Option 2’s “continuous safeguard” applies only to the active model, leaving all non-active models outside its protection. This effectively reduces the benefit of Option 2 to a single-model scenario, negating its intended role in multi-model LCM.

Shadow inference solves this gap by running one or more standby models in parallel with the active model, using the same input data stream. The outputs of these standby models are evaluated against genie values (when available) or compared to the active model’s KPIs, allowing their performance to be tracked without impacting live service. While shadow inference increases processing and memory load  especially in FR2 or high-throughput scenarios  it is the only way to achieve true post-activation monitoring for all stored models without service interruption. For efficiency, shadow inference can be limited to periodic sampling, reduced-resolution inputs, or selected KPIs, but some form of it is essential if Option 2 is to fulfill its purpose in a multi-model deployment.

Shadow inference is essential for Option 2 because post-activation monitoring only gathers KPIs from the model actively serving inference. Without shadow mode, standby models remain untested in live conditions, risking undetected drift or degradation until swapped

Based on our discussion we have the following proposals:

– Hybrid LCM for Post-Deployment AI/ML Models

- Adopt a hybrid life-cycle management (LCM) framework: pre-activation validation for major updates and post-activation monitoring for all updates

- Major updates (architecture change, new RF front-end, large dataset shift)

  - Require pre-activation validation in lab or on-device test mode.

  - Establish or refresh a KPI envelope as baseline.

- Minor updates (small fine-tune, parameter tweak)

  - May skip pre-activation.

  - Must retain prior KPI envelope and enter direct monitoring.

- Post-activation monitoring (all updates)

  - Continuous KPI checks under RAN4/RAN1 metrics (e.g., Top-M/Top-K beam accuracy).

  - Prefer shadow inference to track standby models.

- Rationale

  - Pre-activation blocks immediate large failures; post-activation catches drift and long-term degradation.

Multi-Model LCM and Shadow Inference

Investigate require shadow inference for all non-active models in multi-model deployments.
Every standby model must process the same live input stream as the active model, with its KPIs checked against genie references or active-model baselines.

- Applicability: Applies to any UE storing multiple AI/ML models under Option 2 post-activation monitoring.

- Functionality: Standby models run in parallel “shadow” mode, producing KPIs (e.g., beam accuracy, RSRP error) that never influence network decisions.

- Objective: Detect drift, dataset mismatch, or hidden faults in standby models before activation, ensuring they remain deployment-ready.

- Assurance: Guarantees that every stored model is continuously validated, not just the currently active one. Maintains service quality during frequent updates and dynamic model switching

Standardized Criteria for Major vs. Minor Model

RAN4 shall specify clear rules to classify AI/ML model updates as major or minor, determining whether pre-activation validation is required while ensuring all models undergo post-activation LCM monitoring.

- Major update triggers: full retraining, architecture change, new channel models or RF front-end profiles, significant dataset distribution shift, or inference-pipeline modification.

- Minor update triggers: limited fine-tuning, small parameter or dataset refresh without altering statistical characteristics.

- Purpose: provide consistent, enforceable criteria so vendors and operators know when lab or on-device pre-activation testing is mandatory.

- Assurance: guarantees that every update, regardless of classification, enters continuous post-activation monitoring to detect long-term drift.

Scenario- and Model-Specific KPI Envelopes

RAN4 shall require that every AI/ML model carries a scenario- and model-specific KPI envelope for post-deployment monitoring, providing the correct performance baseline for its intended environment.

- Scenario-specific: KPI thresholds reflect the radio conditions (e.g., urban macro, rural macro, high-Doppler) the model targets.

- Model-specific: Each model or adapter version maintains its own KPI envelope aligned with its architecture and training data.

- Major updates: Generate a new KPI envelope from pre-activation validation results.

- Minor updates: Inherit the validated KPI envelope from the prior deployment.

- Purpose: Ensure accurate and reliable live performance tracking, avoiding false alarms or missed degradations.

# 3     Conclusion

In the previous sections we made the following proposals:

- Option 1 (pre-activation testing) ensures assurance but can be slow, costly, and impractical for frequent updates. Option 2 (post-activation LCM monitoring) is scalable but risks false triggers, overhead, and delayed detection if not carefully designed

Pre-activation protects the network from immediate catastrophic failures at the moment of deployment, while post-activation protects against gradual or unforeseen degradation during the model’s operational life. Both are needed in the hybrid approach to cover different risks and time horizons

Shadow inference is essential for Option 2 because post-activation monitoring only gathers KPIs from the model actively serving inference. Without shadow mode, standby models remain untested in live conditions, risking undetected drift or degradation until swapped

- Hybrid LCM for Post-Deployment AI/ML Models

- Adopt a hybrid life-cycle management (LCM) framework: pre-activation validation for major updates and post-activation monitoring for all updates

- Major updates (architecture change, new RF front-end, large dataset shift)

  - Require pre-activation validation in lab or on-device test mode.

  - Establish or refresh a KPI envelope as baseline.

- Minor updates (small fine-tune, parameter tweak)

  - May skip pre-activation.

  - Must retain prior KPI envelope and enter direct monitoring.

- Post-activation monitoring (all updates)

  - Continuous KPI checks under RAN4/RAN1 metrics (e.g., Top-M/Top-K beam accuracy).

  - Prefer shadow inference to track standby models.

- Rationale

  - Pre-activation blocks immediate large failures; post-activation catches drift and long-term degradation.

Multi-Model LCM and Shadow Inference

Investigate require shadow inference for all non-active models in multi-model deployments.
Every standby model must process the same live input stream as the active model, with its KPIs checked against genie references or active-model baselines.

- Applicability: Applies to any UE storing multiple AI/ML models under Option 2 post-activation monitoring.

- Functionality: Standby models run in parallel “shadow” mode, producing KPIs (e.g., beam accuracy, RSRP error) that never influence network decisions.

- Objective: Detect drift, dataset mismatch, or hidden faults in standby models before activation, ensuring they remain deployment-ready.

- Assurance: Guarantees that every stored model is continuously validated, not just the currently active one. Maintains service quality during frequent updates and dynamic model switching

Standardized Criteria for Major vs. Minor Model

RAN4 shall specify clear rules to classify AI/ML model updates as major or minor, determining whether pre-activation validation is required while ensuring all models undergo post-activation LCM monitoring.

- Major update triggers: full retraining, architecture change, new channel models or RF front-end profiles, significant dataset distribution shift, or inference-pipeline modification.

- Minor update triggers: limited fine-tuning, small parameter or dataset refresh without altering statistical characteristics.

- Purpose: provide consistent, enforceable criteria so vendors and operators know when lab or on-device pre-activation testing is mandatory.

- Assurance: guarantees that every update, regardless of classification, enters continuous post-activation monitoring to detect long-term drift.

Scenario- and Model-Specific KPI Envelopes

RAN4 shall require that every AI/ML model carries a scenario- and model-specific KPI envelope for post-deployment monitoring, providing the correct performance baseline for its intended environment.

- Scenario-specific: KPI thresholds reflect the radio conditions (e.g., urban macro, rural macro, high-Doppler) the model targets.

- Model-specific: Each model or adapter version maintains its own KPI envelope aligned with its architecture and training data.

- Major updates: Generate a new KPI envelope from pre-activation validation results.

- Minor updates: Inherit the validated KPI envelope from the prior deployment.

- Purpose: Ensure accurate and reliable live performance tracking, avoiding false alarms or missed degradations.

# References

[1] R4-2505107, WF on AI ML for air interface study, Ericsson, Apr. 2025.