# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513589

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.9.3

Source:	Apple

Title:	Inter-vendor training collaboration and interoperability for two-sided AI/ML models

Document for:	Discussion

# 1	Introduction

RAN#108 has approved a new WI on AI/ML for NR air interface enhancements in Rel-20 [1], where the objective includes CSI feedback enhancement with two-sided model and inter-vendor training collaboration for two-sided model.

# 2	Number of Test Decoders for Option 3 and Generalization

The WF from RAN4#114bis was as follows:

Agreements

- The encoder that corresponds to the standardized test decoder is stored within 3GPP

  - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS)

  - FFS whether several encoders would be captured (e.g. due to different complexity)

For further discussion:

The following topics should be further discussed:

- For the encoder, the TBD and FFS above; i.e., where the encoder is stored/referred to and whether several encoders would be captured.

- Whether it is agreeable to assume that the training dataset for the standardized test decoder is stored within 3GPP, if available

  - For example in case UE or network vendors could find access to the dataset used for the test encoder useful for their developments

- Whether a test dataset is needed in the specification, or some other alternative (such as a channel model etc.)

For option 3 track 1, the endorsed observations are as follows:

- It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional

- The performance of the encoder is very dependent on the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testset

  - If the encoder is trained using a dataset of the UE vendor, the UE vendor dataset should contain the [features] captured in the dataset used to develop the test decoder.

- The variation in the performance depending on the training datasets and test datasets can be as much as 30-50% in SGCS. Further details are available in the results spreadsheet.

- For option 3, track 1 to be useful, more careful attention would be needed to the alignment of datasets

- Only one encoder structure was considered for option 3 track 1. Other lower complexity encoder structures were not examined.

Reference encoder and number of test decoders

Option 3 fixes a single neural-network test decoder in the TE and retains the encoder that was jointly trained with it as a reference model. What remains unresolved is the cardinality and scope of these models: do we need only one decoder (and thus one reference encoder) to cover all requirement scenarios and UE capability classes, or do fundamentally different propagation conditions and capability constraints force multiple frozen decoders? The current simulations trained one encoder–decoder pair and then derived new encoders, but they have not yet proven that a single decoder generalizes across all envisaged scenarios. If several decoders were ultimately required, each would demand its own reference encoder, increasing storage and maintenance overhead inside 3GPP.

A second, related issue is whether multiple reference encoders should be kept for a single decoder to reflect different encoder complexity levels. If low-complexity encoders can meet the same performance target as higher-complexity ones, it is unclear why vendors would not simply adopt the simplest solution and why 3GPP should store more than one reference encoder. Conversely, if different complexity levels deliver materially different, standardized performance tiers, several encoders might be justified but that would imply defining multiple performance levels for the same scenario, something RAN4 rarely does. Clear criteria are therefore needed to decide when additional decoders or reference encoders are warranted, how they are trained, and how their complexity is bounded, so that interoperability is guaranteed without proliferating models beyond what is strictly necessary.

We believe that RAN4 should keep one frozen test decoder and one associated reference encoder the encoder used to train that decoder stored in a 3GPP repository with versioning and checksums. Additional decoders are only added if objective evidence (e.g., an agreed NMSE/SGCS spread threshold across distinct requirement scenarios) shows the single decoder cannot meet all needs. The decoder is trained once with the reference encoder; retraining with others is unnecessary unless new, approved scenarios or tiers appear.

To determine the required cardinality of different test decoders to cover all applicable scenarios, we propose conducting a Test-Decoder Deployment Variability Assessment to draw conclusions on the appropriate granularity of the specified decoders.

In the worst-case, testing requirements for CSI compression could demand multiple distinct test decoders to cover fundamentally different scenarios, for example, varying frequency ranges, antenna configurations, mobility profiles, or channel models. If each scenario requires its own fully trained and frozen test decoder, RAN4 would need to manage, store, and maintain several large models in parallel. Any change, such as a dataset refresh, preprocessing adjustment, or bug fix would have to be replicated across all decoders, multiplying the verification, version control, and interoperability checks by the number of scenarios. This increases TE storage requirements, and complexity.

The scalability challenge intensifies over time. As new channel models or deployment scenarios are introduced in later releases, more frozen decoders would need to be added, each trained and validated independently. Older decoders can quickly become outdated while still being part of the certification scope, creating long-term maintenance burdens. This rigid “one full model per scenario” approach also limits flexibility, as each decoder is tied to its original training assumptions, making adaptation to future conditions cumbersome and inefficient.

Backbone + Adapter Concept for multiple test-decoders 
Instead of maintaining multiple full-size decoders, RAN4 could standardize a single backbone model, a shared, frozen core architecture and weight set that defines the common latent space for encoder–decoder interoperability. For scenario-specific requirements, lightweight adapters (e.g., small fine-tuned modules or layers) can be trained on a targeted dataset to specialize the backbone for that environment. The backbone remains unchanged across scenarios; only the adapter varies.

In this model, a new scenario (e.g., high-speed railway at mmWave) would not require creating and freezing an entirely new decoder. Instead, a scenario-specific adapter would be trained using a small, well-defined dataset. The adapter, combined with the fixed backbone, functions as the test decoder for that scenario. This approach decouples the common interoperability layer from the scenario-specific tuning, enabling RAN4 to maintain a single core model while allowing flexible, lightweight extensions.

This proposed approach reduces the number of large decoder models RAN4 must store and version, lowering maintenance costs and administrative effort. It also mitigates the risk of divergence between decoders, since all scenarios share the same backbone. Scenario-specific testing is more efficient because only the adapter needs validation, not the full decoder. Future-proofing is improved new scenarios can be supported by releasing a small dataset and fine the adapter without retraining or replacing the backbone.

If RAN4 standardizes a single frozen backbone and a common adapter architecture, there is no need to store multiple pre-trained adapters for different scenarios. Instead, each new scenario, whether defined by a specific channel model, environment, or frequency band, would only require an agreed scenario-specific dataset and simulation assumptions. At certification time, the adapter is trained on this dataset to produce the specialized decoder on the fly, while the backbone remains unchanged. This approach minimizes stored artifacts to just the backbone and adapter architecture, eliminates per-scenario versioning overhead and ensures that any future updates (e.g., changes to preprocessing rules) require only generating a new scenario-specific dataset and training the adapter, while the backbone remains frozen. This keeps the process scalable, reduces re-certification effort, and maintains consistency across all scenarios

Single Frozen Backbone Test Decoder for Interoperability

RAN4 should standardize one common backbone to simplify conformance and avoid unnecessary model proliferation:

- Single backbone and reference encoder – Define a frozen test decoder with one associated reference encoder, trained on a diverse dataset spanning agreed frequency bands, environments, antenna configurations, and SNR ranges.

- Evidence-based expansion – Allow additional full decoders only if objective metrics (e.g., agreed NMSE or SGCS spread thresholds across distinct requirement scenarios) show that one backbone cannot meet performance requirements.

- Goal – Ensure full interoperability while minimizing the number and size of standardized decoder models.

Backbone + Scenario-Specific Adapter Framework

RAN4 should enable targeted specialization while preserving a common interoperable core:

- Common frozen backbone – Defines a shared latent space for all scenarios, ensuring interoperability and stable conformance.

- Lightweight scenario-specific adapters – Trained on targeted datasets to meet performance needs when the single backbone alone is insufficient.

- Minimal overhead – Only adapters are trained and validated, reducing storage, update, and maintenance requirements while keeping the backbone unchanged.

On-Demand Adapter Generation Using Standardized Datasets

RAN4 should simplify scenario handling by creating adapters only when needed:

- Single main test decoder – Keep one frozen backbone as the base for all scenarios.

- Standardized per-scenario datasets – For each new scenario, only the dataset is captured; no new full decoder is defined.

- On-demand adapter training – At certification, fine-tune the backbone with the standardized dataset to create the scenario-specific adapter and final test decoder.

- Benefits – Eliminates per-scenario versioning, minimizes test-equipment storage, and scales smoothly to future channel models and deployments.

Test-Decoder Deployment Variability Assessment

RAN4 should evaluate when multiple test decoders are truly needed by comparing different training strategies:

- Model families to test – (i) universal model trained across all scenarios, (ii) standalone per-scenario models, and (iii) hybrid with shared backbone plus scenario-specific experts.

- Scenario diversity – Vary carrier frequency, antenna configuration, environment, and SNR.

- Evaluation method – Compare “diagonal” errors (model tested on its own scenario) with “off-diagonal” errors (tested on other scenarios).

- Goal – Identify when more than one decoder is required and whether compact expert heads can match standalone model accuracy while scaling more efficiently.

If multiple encoder complexity levels are explored, the reference encoder is simply the lowest-complexity one that still meets the target. Only if RAN4 explicitly defines different performance tiers tied to UE capabilities would multiple reference encoders be justified. Remaining FFS items include setting the exact convergence metric and thresholds, defining any capability tiers and their performance floors, and establishing lifecycle rules for refreshing datasets/models without complicating certification. This approach preserves interoperability and limits model proliferation while leaving a controlled path for expansion if truly needed.

RAN4 shall retain a single reference encoder per frozen test decoder, selecting the lowest-complexity encoder that still meets the agreed performance target; additional reference encoders would be justified only if RAN4 explicitly defines distinct performance tiers tied to UE capability classes.

If RAN4 defines distinct performance tiers, RAN4 needs to set the exact convergence metric (SGCS, NMSE) define the capability tiers and their minimum performance floors .

# 3	Construction of the synthetic dataset and data alignment using PSE

The central problem is ensuring that the dataset used to train the test decoder realistically represents the range of channel conditions the model will encounter in actual deployments. In previous simulations, even though companies used the same agreed-upon assumptions, differences emerged in datasets due to inconsistencies like amplitude or phase normalization. Such mismatches mean the latent space learned by the decoder might not reflect real-world conditions, causing degraded interoperability and inconsistent encoder performance across vendors.

For a Work Item (WI), accumulating extensive real-world measurement data within 3GPP RAN4 is impractical. Therefore, the dataset must come from carefully designed simulations covering sufficient diversity frequency bands, environments/channel models, mobility levels, antenna configurations, and SNR ranges to be representative of real deployment scenarios. Equally critical is ensuring strict alignment between datasets produced by different companies, through standardized preprocessing steps and verification procedures, so all encoders trained against the resulting latent space remain interoperable and robust in real-world networks.

To address dataset realism and alignment, it is proposed that RAN4 define a standardized simulation framework covering a diverse set of scenarios relevant to real-world deployments. This framework would explicitly specify frequency bands, environments (urban macro, urban micro, indoor, rural), mobility levels (static, pedestrian, vehicular, high-speed), antenna configurations, and SNR distributions. A common preprocessing procedure including precise amplitude/phase normalization, quantization methods, shall be documented and strictly followed by all contributing companies.

Datasets generated under this unified procedure must pass statistical alignment tests based on predefined metrics such as power spectral entropy (PSE). Companies wishing to introduce proprietary data must demonstrate statistical equivalence with the reference dataset; if alignment fails, companies must augment or re-weight their data accordingly or default to the official dataset provided by RAN4. This disciplined, reproducible approach ensures the resulting dataset captures a realistic, interoperable latent space for the test decoder, maintaining consistency and reliability in certification.

Standardized Dataset Collection for Test Decoder Training

RAN4 should ensure every test decoder is trained on realistic, interoperable data:

- Simulation framework: Define common frequency bands, environments, antenna setups, and SNR distributions.

- Dataset alignment: Verify consistency across vendors using metrics such as power spectral entropy (PSE).

- Outcome – Guarantee that the trained test decoder reflects true wireless channel scenarios and provides a common latent space for interoperability.

Power Spectral Entropy (PSE) is a way to summarize how the power of a wireless channel is distributed across the physical dimensions of space and delay. We start with the channel state information (CSI), which describes how each transmit port communicates with each receive port on each frequency subcarrier. This raw CSI contains the fine details of multipath propagation but is tied to a specific antenna layout and is hard to compare across different systems.

The first step is to represent the CSI on universal physical axes that everyone can share. The channel is transformed from the antenna-port view into a beamspace view so that received and transmitted angles of arrival and departure are represented on a common grid. At the same time, a delay transform converts the frequency information into multipath delays. The result is a three-dimensional cube showing how much signal power lies in each combination of transmit angle, receive angle, and delay.

Next, only the magnitude of the energy matters. The complex channel coefficients are converted into power values, so each small cell of the cube shows the amount of power coming from that direction and delay. All these power values are then normalized so that they sum to one. This creates a probability map, meaning each cell can be thought of as the probability that a unit of signal energy comes from that particular angle and delay.

Finally, PSE measures how evenly this probability is spread. If most of the power is concentrated in a few cells, as in a clear line-of-sight channel, the entropy is low. If power is spread across many cells, as in a rich scattering environment, the entropy is high. In this way a single number, or a compact set of numbers, captures the structural richness of the wireless channel. Because PSE is based on a shared physical grid and on power rather than phase, it is well suited for aligning and comparing datasets from different companies or measurement setups.

We are proposing the following steps:

## 1. Transform CSI to a 3-D power cube

Each channel snapshot provides a complex-valued frequency response H(f) between all transmit and receive antenna ports.
This raw matrix is not directly comparable across different antenna layouts, so we first convert it to a common physical grid.

Step 1.1 – Beamspace projection
Apply fixed, agreed-upon DFT beamforming matrices:

- B_rx of size N_r × N_rx for the receive array

- B_tx of size N_t × N_tx for the transmit array

These matrices express the channel in terms of directional beams instead of antenna elements, which removes dependence on each vendor’s exact antenna geometry.

Step 1.2 – Delay transform
Apply an inverse FFT across subcarriers (size K) to resolve multipath delays.
This converts the frequency response into the time (delay) domain.

Step 1.3 – Build the power cube
Combine the two spatial transforms and the delay transform to form a 3-D non-negative power cube:

S[u,v,τ] = | ∑_f ( B_rx[u,:] · H(f) · B_tx[:,v] ) e^{j2π f τ Δτ} |^2

where

- u = 0 … N_r − 1 is the RX beam index,

- v = 0 … N_t − 1 is the TX beam index,

- τ = 0 … K − 1 is the delay-bin index.

This cube S tells us how much signal energy arrives from each RX direction, leaves via each TX direction, and at each excess delay.

## 2. Convert to a probability map and compute PSE

Different channel snapshots can have different absolute powers, so we normalize each cube to make them comparable.

Step 2.1 – Probability normalization
p[u,v,τ] = S[u,v,τ] / Σ_{u',v',τ'} S[u',v',τ'}

This converts the power cube into a probability distribution over all N_r * N_t * K bins.

Step 2.2 – Power Spectral Entropy
Calculate Shannon entropy and normalize it by the maximum possible entropy:

PSE = - Σ_{u,v,τ} p[u,v,τ] * log2(p[u,v,τ]) / log2(N_r * N_t * K)

- PSE close to 0 means power is concentrated in a few bins (sparse, line-of-sight channels).

- PSE close to 1 means power is spread across many bins (rich, highly scattering channels).

This single number is a compact, geometry-independent fingerprint of each channel snapshot.

## 3. Compare PSE distributions across datasets

Now each dataset is simply a collection of PSE numbers.

Step 3.1 – Empirical CDFs
Compute empirical cumulative distribution functions (CDFs) F_A(x) and F_B(x) for datasets A and B.

Step 3.2 – Wasserstein-1 distance 
W1 = ∫ |F_A(x) - F_B(x)| dx

This is the minimum “work” needed to reshape one PSE distribution into the other and captures overall distributional shift.

Step 3.3 – Kolmogorov–Smirnov statistic
D_KS = sup_x |F_A(x) - F_B(x)|

This is the largest vertical gap between the two CDFs and reveals localized mismatches even when means are similar.

Together, W1 and D_KS provide a global and local picture of how different the two datasets are in terms of spatial–temporal channel richness.

## 4. Make an alignment decision

Define acceptable tolerances (for example, W1 ≤ 0.05 and D_KS ≤ 0.1).
Optionally compute a single alignment score:

Score = 100 * [1 - γ1 * (W1 / τ_W1) - γ2 * (D_KS / τ_KS)]

where γ1 and γ2 weight the two distances and τ_W1, τ_KS are the tolerances.

- Score ≥ 80 → Aligned: datasets are aligned and can be used to evaluate or deploy the trained CSI compression model with confidence.

- Score 60–80 → Partially aligned: Consider reweighting the test dataset, filtering outlier snapshots, or collecting additional samples before relying on performance metrics

- Score < 60 → Misaligned: datasets differ too much for direct model sharing.

PSE can also be used for performance monitoring as an input-distribution drift metric.

Embedding PSE into network performance monitoring lets operators track input-distribution drift and act before compression accuracy degrades. Each trained CSI compression model can be shipped with a reference PSE “envelope” (mean, variance, or histogram). During operation, the UE or gNB computes rolling PSE statistics from live CSI feedback and compares them with the reference envelope. Persistent deviations signal that the deployed model is encountering channel conditions different from those it was trained on, prompting retraining or an on-the-fly model switch. Standardizing PSE computation and reporting would give operators and vendors a common, low-overhead guardrail for maintaining CSI compression performance throughout a network’s lifetime.

PSE-Based Dataset Alignment Methodology

RAN4 should adopt a power spectral entropy (PSE) method to verify cross-vendor dataset consistency:

- Raw channel requirement – Provide raw CSI/channel data so that each CSI snapshot can be converted into a common beamspace-delay power map for PSE calculation.

- PSE computation – Compute PSE to quantify how widely signal power is spread across beams and delays.

- Statistical comparison – Treat PSE values from different datasets as distributions and compare them with distance metrics such as Wasserstein distance.

- Alignment criterion – Datasets are considered aligned if the measured distances stay below agreed thresholds, ensuring consistent training data for standardized test decoders.

– PSE for Online Performance Monitoring and OOD Detection

RAN4 should use power spectral entropy (PSE) as a live KPI to detect input-distribution drift and monitor model performance:

- Continuous PSE computation – Calculate PSE in real time from live CSI feedback during network operation.

- Reference PSE envelope – Each deployed CSI compression model carries a PSE envelope derived from its training dataset.

- OOD detection – Compare live PSE statistics to the reference envelope; persistent deviations signal out-of-distribution (OOD) conditions.

- LCM action triggers – Detected drift can trigger model retraining, adapter updates, or switching to a more suitable model.

- Unified methodology – The same PSE framework used for dataset alignment in certification ensures consistency between offline validation and online performance monitoring.

# 4 Inter-vendor training collaboration and interoperability for two-sided AI/ML

In RAN Meeting #109 a revised WID was captured in RP-252445 with the following aggrements:

Inter-vendor training collaboration for two-sided AI/ML models

- Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback

  - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback

- Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

Interoperability and RRM requirement [RAN4]:

- To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability

  - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration

- Necessary RRM requirement for specified LCM procedures.

- NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

Below we give an overview of Direction C, Direction A, sub-option 3a-1 and Option 3 for fully specified test decoder:

Direction C:

Direction C is the fully specified reference-model approach for two-sided AI/ML CSI feedback.
Its purpose is to guarantee inter-vendor interoperability and minimum field performance by standardizing at least one side of the model: the encoder, the decoder, or both. The standardized side would be fully defined in 3GPP: its architecture, layer configuration, input/output format, and, if required, trained weights are all fixed in the specification. With this fixed anchor, any vendor can develop or fine-tune the opposite side of the model so that the end-to-end CSI compression and reconstruction meet the agreed KPIs.

This flexible structure allows multiple deployment choices. If the decoder is standardized, every UE vendor can train its own encoder to generate compressed CSI that is guaranteed to work with the reference decoder. If the encoder is standardized, network vendors can train their own decoders to match the standardized encoder output. And if needed, both encoder and decoder may be specified together as a complete reference model for direct use.
In every case, vendors retain freedom to re-engineer or fine-tune their non-standardized side, while the common interface ensures compatibility.

Direction C also embeds the scalability and generalization principles studied in RAN1.
The reference model must handle a wide range of antenna-port counts, sub-band sizes, and payload budgets, and it must be robust when real field data differ from synthetic training data.
Vendors may fine-tune the non-standardized side to local conditions, but the standardized reference side guarantees that any compliant encoder–decoder pair will interoperate and deliver at least the minimum required performance.

Finally, Direction C serves as a foundation for other inter-vendor collaboration methods, such as Direction A sub-option 3a-1 (parameter exchange).
The RAN4 test decoder and reference encoder developed for Option 3 can be built on the Direction C backbone, ensuring that laboratory conformance tests and field deployments share the same standardized anchor. In this way Direction C provides the gold-standard reference model for commercial deployment, while remaining flexible enough to accommodate vendor innovation on the complementary side of the AI/ML CSI feedback chain.

Option3 test decoder:

Option 3 is RAN4’s fully specified test-decoder approach for AI/ML-based CSI feedback.
Its purpose is to give every vendor a common, reproducible benchmark for conformance testing.
RAN4 defines the complete structure of the decoder: including backbone type, layers, dimensions, activation functions, and any normalization or attention mechanisms, and fixes the trained weights.
This “test decoder” becomes the gold-standard receiver for compressed CSI: any UE encoder that passes conformance testing against it is proven to interoperate at or above the agreed performance floor.
By freezing both the architecture and the parameters, RAN4 ensures that test results are repeatable across labs and independent of any individual vendor’s implementation.

To make the test decoder meaningful, RAN4 also specifies a co-trained reference encoder.
This reference encoder is the exact partner that was used when training the test decoder.
It is stored in 3GPP documentation or linked repositories so that test equipment vendors and implementers can reproduce the decoder’s original training and verify its performance.
While commercial UEs are not required to deploy this reference encoder, it serves several critical roles:

- providing the dataset and latent outputs used to validate the test decoder,

- acting as a baseline against which any new UE encoder can be compared, and

- supporting inter-vendor training collaboration by giving all parties a known, trusted encoder side when developing or adapting their own models.

The goal of deriving the test decoder with its reference encoder is therefore twofold.
First, it provides a single, authoritative test environment to certify UE encoders for Release-20 CSI feedback.
Second, it supplies a stable reference model that RAN1 and other working groups can leverage for inter-vendor training collaboration: whether in Direction C (fully specified reference model) or Direction A sub-option 3a-1 (parameter exchange without CSI sharing).
Through this work, RAN4 guarantees that AI/ML-based CSI compression achieves minimum interoperability and performance across all vendors and network deployments.

Direction A 3a-1:

Direction A 3a-1 is a parameter-exchange method for inter-vendor training collaboration in two-sided AI/ML CSI feedback.
Its goal is to let a network vendor train an encoder–decoder model and then share only the trained encoder parameters and structural metadata with UE vendors, while keeping proprietary CSI data private.
No target-CSI samples are exchanged.

The procedure begins with network-side joint training of an encoder and decoder using network data.
Once training is complete, the network exports the encoder’s weights and any necessary model-structure information.
A UE vendor then imports these parameters and trains its own decoder locally, using only UE-collected channel data, to reconstruct the CSI from the shared encoder outputs.
After freezing this decoder, the UE fine-tunes its own encoder so that the final UE encoder–decoder chain performs optimally under real operating conditions.
Inference in the field then runs with the UE’s trained encoder and a compatible network decoder, all without exchanging raw CSI.

This approach provides a secure and efficient path to interoperability.
Because every vendor uses the same standardized encoder structure specified by RAN4, the shared parameters can be interpreted unambiguously.
The UE’s locally trained decoder guarantees compatibility with the network’s decoder family, while allowing each vendor to adapt to its own antenna layouts, RF chains, and processing preferences.
Direction A 3a-1 thus enables joint or semi-joint model development across vendors, meets performance and RAN4 requirements, and preserves data privacy, fulfilling a key part of the Release-20 AI/ML work on inter-vendor training collaboration.

Fully specified test decoder with standardized reference encoder for Option 3 testing and Direction C field use, supporting Direction A encoder-parameter exchange

Investigate feasibility of one RAN4-specified test decoder together with a fully specified reference encoder structure and parameters that serves multiple purposes:
(1) provides the Option 3 lab conformance benchmark,
(2) acts as the Direction C standardized reference side for field deployments, and
(3) supplies the standardized encoder structure and parameter format needed for Direction A 3a-1 inter-vendor collaboration (which does not require a standardized decoder).

- Complete model specification – Specify the test-decoder design and trained weights plus the full reference-encoder structure
Applicability: (1) ensures reproducible Option 3 testing; (2) provides the reference side for Direction C; (3) provides a standardized encoder architecture and a common parameter format so that encoder weights can be exchanged and interpreted unambiguously for Direction A parameter exchange.

- Unified test and field use – Allow the same decoder/encoder pair to serve as the Option 3 test object and as the Direction C field reference, while the standardized encoder structure alone enables Direction A training collaboration without mandating a decoder.

Study LCM requirements on KPIs and triggers for model lifecycle actions for CSI compression in inter-vendor collaboration

RAN4 should investigate LCM requirements for AI/ML CSI-compression models, covering when to activate, switch, or fall back

- Clarify key performance indicators (KPIs) to be observed, ensuring they reflect both end-to-end link quality

- Define the principles for decision criteria, such as how KPIs indicate stable improvement or persistent degradation before a model change.

- RAN4 to discuss requirements for minimum observation and hold times to ensure KPI issues persist and new models run long enough, preventing rapid toggling that could disrupt scheduling or RRM performance.

- Outline monitoring cadence expectations, balancing early detection of issues with acceptable signaling and processing load.

- Specify how these high-level requirements will be tested and validated in RAN4 conformance procedures.

# Conclusion

In the previous sections we made the following observations:

- Single Frozen Backbone Test Decoder for Interoperability

RAN4 should standardize one common backbone to simplify conformance and avoid unnecessary model proliferation:

- Single backbone and reference encoder – Define a frozen test decoder with one associated reference encoder, trained on a diverse dataset spanning agreed frequency bands, environments, antenna configurations, and SNR ranges.

- Evidence-based expansion – Allow additional full decoders only if objective metrics (e.g., agreed NMSE or SGCS spread thresholds across distinct requirement scenarios) show that one backbone cannot meet performance requirements.

- Goal – Ensure full interoperability while minimizing the number and size of standardized decoder models.

Backbone + Scenario-Specific Adapter Framework

RAN4 should enable targeted specialization while preserving a common interoperable core:

- Common frozen backbone – Defines a shared latent space for all scenarios, ensuring interoperability and stable conformance.

- Lightweight scenario-specific adapters – Trained on captured datasets from distinct deployment environments. These enable tailoring to scenario conditions and support more accurate scenario-specific test decoders

- Minimal overhead – Only adapters are trained and validated, reducing storage, update, and maintenance requirements while keeping the backbone unchanged.

On-Demand Adapter Generation Using Standardized Datasets

RAN4 should simplify scenario handling by creating adapters only when needed:

- Single main test decoder – Keep one frozen backbone as the base for all scenarios.

- Standardized per-scenario datasets – For each new scenario, only the dataset is captured; no new full decoder is defined.

- On-demand adapter training – At certification, fine-tune the backbone with the standardized dataset to create the scenario-specific adapter and final test decoder.

- Benefits – Eliminates per-scenario versioning, minimizes test-equipment storage, and scales smoothly to future channel models and deployments.

Test-Decoder Deployment Variability Assessment

RAN4 should evaluate when multiple test decoders are truly needed by comparing different training strategies:

- Model families to test – (i) universal model trained across all scenarios, (ii) standalone per-scenario models, and (iii) hybrid with shared backbone plus scenario-specific experts.

- Scenario diversity – Vary carrier frequency, antenna configuration, environment, and SNR.

- Evaluation method – Compare “diagonal” errors (model tested on its own scenario) with “off-diagonal” errors (tested on other scenarios).

- Goal – Identify when more than one decoder is required and whether compact expert heads can match standalone model accuracy while scaling more efficiently.

RAN4 shall retain a single reference encoder per frozen test decoder, selecting the lowest-complexity encoder that still meets the agreed performance target; additional reference encoders would be justified only if RAN4 explicitly defines distinct performance tiers tied to UE capability classes.

If RAN4 defines distinct performance tiers, RAN4 needs to set the exact convergence metric (SGCS, NMSE) define the capability tiers and their minimum performance floors .

Standardized Dataset Collection for Test Decoder Training

RAN4 should ensure every test decoder is trained on realistic, interoperable data:

- Simulation framework: Define common frequency bands, environments, antenna setups, and SNR distributions.

- Dataset alignment: Verify consistency across vendors using metrics such as power spectral entropy (PSE).

- Outcome – Guarantee that the trained test decoder reflects true wireless channel scenarios and provides a common latent space for interoperability.

PSE-Based Dataset Alignment Methodology

RAN4 should adopt a power spectral entropy (PSE) method to verify cross-vendor dataset consistency:

- Raw channel requirement – Provide raw CSI/channel data so that each CSI snapshot can be converted into a common beamspace-delay power map for PSE calculation.

- PSE computation – Compute PSE to quantify how widely signal power is spread across beams and delays.

- Statistical comparison – Treat PSE values from different datasets as distributions and compare them with distance metrics such as Wasserstein distance.

- Alignment criterion – Datasets are considered aligned if the measured distances stay below agreed thresholds, ensuring consistent training data for standardized test decoders.

– PSE for Online Performance Monitoring and OOD Detection

RAN4 should use power spectral entropy (PSE) as a live KPI to detect input-distribution drift and monitor model performance:

- Continuous PSE computation – Calculate PSE in real time from live CSI feedback during network operation.

- Reference PSE envelope – Each deployed CSI compression model carries a PSE envelope derived from its training dataset.

- OOD detection – Compare live PSE statistics to the reference envelope; persistent deviations signal out-of-distribution (OOD) conditions.

- LCM action triggers – Detected drift can trigger model retraining, adapter updates, or switching to a more suitable model.

- Unified methodology – The same PSE framework used for dataset alignment in certification ensures consistency between offline validation and online performance monitoring.

Fully specified test decoder with standardized reference encoder for Option 3 testing and Direction C field use, supporting Direction A encoder-parameter exchange

Investigate feasibility of one RAN4-specified test decoder together with a fully specified reference encoder structure and parameters that serves multiple purposes:
(1) Provides the Option 3 lab conformance benchmark,
(2) Acts as the Direction C standardized reference side for field deployments, and
(3) Defines the standardized encoder structure and parameter format needed for Direction A 3a-1 inter-vendor collaboration (which does not require a standardized decoder).

- Complete model specification – Specify the test-decoder design and trained weights plus the full reference-encoder structure
Applicability: (1) ensures reproducible Option 3 testing; (2) provides the reference side for Direction C; (3) provides a standardized encoder architecture and a common parameter format so that encoder weights can be exchanged and interpreted unambiguously for Direction A parameter exchange.

- Unified test and field use – Allow the same decoder/encoder pair to serve as the Option 3 test object and as the Direction C field reference, while the standardized encoder structure alone enables Direction A training collaboration

- Scalability study of encoder and decoder structures to ensure support for different antenna-port counts, sub-band configurations, and payload sizes

Study LCM requirements on KPIs and triggers for model lifecycle actions for CSI compression in inter-vendor collaboration

RAN4 should investigate LCM requirements for AI/ML CSI-compression models, covering when to activate, switch, or fall back

- Clarify key performance indicators (KPIs) to be observed, ensuring they reflect both end-to-end link quality

- Define the principles for decision criteria, such as how KPIs indicate stable improvement or persistent degradation before a model change.

- RAN4 to discuss requirements for minimum observation and hold times to ensure KPI issues persist and new models run long enough, preventing rapid toggling that could disrupt scheduling or RRM performance.

- Outline monitoring cadence expectations, balancing early detection of issues with acceptable signaling and processing load.

- Specify how these high-level requirements will be tested and validated in RAN4 conformance procedures.

# References

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Qualcomm.