3GPP TSG-RAN WG4 Meeting #116bis                                  	                        R4-2513597

# Prague, Czech Republic, October 13 – October 17, 2025

Agenda item:	7.10.3

Source:	Apple Inc.

Title:	On SCell Activation Delay Requirements with Enhanced CSI Acquisition

Release:	Rel-20

Document for:	Discussion

# Scope

A new WID on NR MIMO Phase 6 has been finished. First, to improve the performance of TDD systems, especially for cell edge user’s experience, SRS capacity and coverage need to be further enhanced by introducing the utilization of more contiguous symbols for SRS configuration via introducing SRS transmission across two adjacent slots (S and U slots). Second, as a UE is transitioned from IDLE/INACTIVE to CONNECTED mode, from SCell deactivation to activation, or from SCell dormancy state to active state, due to the lack of priori measurement/reporting the user perceived throughput (UPT) is low, so early CSI is introduced. Finally, larger number of antenna ports (up to 128) has been introduced in Rel-19 NR, partly to cater for the emergence of higher frequency bands in FR1. However, as the number of antenna ports increases, the linear increase in RS overhead from the current CSI-RS design becomes a bottleneck, so it is beneficial to reduce the CSI-RS frequency-domain density for larger number of CSI-RS ports aggregated across multiple CSI resources to enhance throughput in larger-scale MIMO deployments

On the first topic, improvement of SRS capacity and coverage, we do not see obvious RRM impact. On the second topic on early CSI triggering there is clear RRM impact since the activation delay requirements should be modified. Finally, for CSI-RS overhead reduction we don’t see impact.

Observation #1: Current MIMO Phase 6 WI contains three main parts: 1) Enhancing SRS UL capacity and coverage, 2) Early Triggering of SRS-AS/CSI/CSI-RS, and 3) Reducing CSI-RS overhead.

Proposal #1: RAN4 to prioritize Early Triggering of SRS-AS/CSI/CSI-RS for discussion in RAN4#116bis since is the scenario with obvious RRM impact.

# Early CSI Acquisition for SCell Transition from Deactivated to Activated State

Existing 3GPP TS 38.321 specifications explicitly prohibit UEs from transmitting SRS, reporting CSI feedback, and monitoring PDCCH on a still deactivated SCell. This effectively creates a situation where the CSI stage cannot start unless the SCell has completely transitioned to activated state. Hence, one way to improve the total delay between the UE receiving the SCell activation command and the point in which the UE sends the first valid CSI report is by intertwining the activation phase and the CSI phase.

Observation #2: There are a number of restrictions on what the UE can do when a SCell is deactivated, being one of them PDCCH monitoting.

One way would be to allow UE to perform concurring PDCCH monitoring after MAC CE command. Under this approach, TS 38.321 should be modified to allow a UE to begin monitoring for control information immediately after receiving the SCell activation command via MAC CE. In this way, the network could already coordinate for SRS/CSI/CSI-RS triggering using a standard Downlink Control Information (DCI) message. In this way, existing design can be reused without needed to change the existing MAC CE format. However, it is worth noting that the network can only send the DCI once the UE has acknowledged the MAC CE command.

Observation #3: One alternative to reduce SCell activation delay discussed in RAN1 is concurrent MAC CE command processing and PDCCH monitoring during deactivated state.

Another perhaps more efficient way would be to expand the scope of the SCell activation MAC CE Command. Under this approach, MAC CE command should be enhanced to include new fields for triggering aperiodic SRS/CSI/CSI-RS. In this way, the SCell activation command and the CSI trigger gets collapsed into a single signaling message, hence the CSI part can potentially start even sooner.

Observation #4: A more efficient alternative is to consider an enhanced MAC CE command with early triggering support.

As mentioned before, another aspect worthy of analysis is the scope of this feature. As discussed, the SCell activation timeline can be long, so the benefits of reducing processing time due to CSI acquisition may be limited. While it’s advantageous to include the feature for known cells, we should consider unknown cell scenarios and restrict the feature to a subset of them if necessary. For example, one avenue could be to restrict the feature to Rel-17 Fast SCell activation scenarios, where the SCell is already known to the UE and tracking can be established quickly.

Proposal #2: RAN4 start in RAN4#117 an analysis of the shortened timeline for Early Triggering of SRS-AS/CSI/CSI-RS in SCell Activation and Fast SCell activation for known cells based on legacy MAC CE SCell activation command based on progress during the RAN1 October meeting.

At the moment, the related agreements in RAN1 are as follows:

At present, companies have agreed to support at least periodic SRS-AS transmission and periodic CSI reporting, associated with at least aperiodic CSI-RS for CSI on an SCell, triggered based on the legacy SCell activation command. While some companies have identified the necessity of considering not only aperiodic CSI reporting but also periodic and semi-persistent CSI, we believe it remains to be determined whether interested companies will contribute with timeline analysis and potential requirements.

Observation #5: Some companies have proposed in RAN1 considering not only using aperiodic CSI-RS but also periodic and semi-persistent CSI-RS resources.

Proposal #3: For early triggering of CSI/CSI-RS, RAN4 to prioritize aperiodic CSI reporting/resources while waiting for RAN1 further progress.

In what follows, we briefly describe some of the different variations of the SCell activation delay requirements. The activation delay, Tactivation_time, is the maximum time allowed from when a User Equipment (UE) receives an SCell activation command until it must be capable of transmitting a valid Channel State Information (CSI) report for that cell. This activation delay widely varies as a function of several possible scenarios, and we briefly describe some of them below.

The state of the UE's knowledge regarding the SCell is the primary determinant of activation delay. An SCell is "known" if the UE has recently sent valid measurement reports and the cell's Synchronization Signal Block (SSB) has remained detectable. Otherwise, the SCell is "unknown". This distinction dictates whether extra time for cell identification is required.

| Condition | Tactivation_time formula |
| --- | --- |
| Known Cell - Measurement period ≤ 2400 ms | TFirstSSB + 5 ms |
| Known Cell - Measurement period > 2400 ms | TFirstSSB_MAX + Trs + 5 ms |

where

TFirstSSB: Time to the end of the first complete SSB burst after the activation command.

TFirstSSB_MAX: Time to an occasion when the SCell and all active serving cells transmit their SSB bursts in the same slot.

Trs: The SMTC periodicity of the SCell being activated

An "unknown" SCell requires a more complex search procedure, and the delay requirement varies based on network aspects and on certain UE capabilities, e.g. shortMeasInterval-r18. Below we present a subset of unknown cell use cases to contrast against the known cell use cases.

| Condition | Tactivation_time formula |
| --- | --- |
| Unknown Cell – Periodic CSI | 3 ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, Tuncertainty_RRC + TRRC_delay) |
| Unknown Cell – Semi-Persistent CSI | 6 ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2 ms, Tuncertainty_SP) |
| Unknown Cell – Periodic CSI, shortMeasInterval-r18 supported | 3 ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, Tuncertainty_RRC + TRRC_delay) |
| Unknown Cell – Semi-Persistent CSI, shortMeasInterval-r18 supported | 6 ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2 ms, Tuncertainty_SP) |

In addition, we have use cases in which we can say there is a “good reference cell” conditions that also expedite the activation delay. For example, we can call a "good reference cell", considering an SSB-based FR1 SCell, by the following crireria:

1. The reference cell must be contiguous and in the same band as the SCell.

2. It must have the same ssb-PositionInBurst and SMTC offset.

3.  RTD must be ≤ 260 ns.

4. Received power difference must be ≤ 6 dB.

If one of these conditions is not met, then a longer activation delay (+ 2*Trs) applies.

| Condition | Tactivation_time formula |
| --- | --- |
| Standard UE with Reference Cell | TFirstSSB_MAX + TSMTC_MAX + Trs + 5 ms |
| Standard UE without Reference Cell | TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5 ms |
| Advanced UE (shortMeasInterval-r18) with Reference Cell | TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5 ms |
| Advanced UE (shortMeasInterval-r18) without Reference Cell | TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5 ms |

Observation #6: Current framework for SCell Activation containes a variety of cases depending on knowlege of the SCell, UE measurement capability, and other aspects. In addition, the difference between Rel-15 vs Rel-17 frameworks is another aspect to consider.

Proposal #4: RAN4 to study what specific scenarios in the SCell activation framework to consider for early triggering of SRS-AS/CSI/CSI-RS.

# Early CSI Acquisition for Transition from IDLE/INACTIVE to CONNECTED Mode

This particular use case has progressed the most in RAN1, with a confirmed procesure for IDLE mode. The general 4-step procedure is as follows:

Step 1: The UE receives resource/reporting configuration in System Information (SI) before Msg3.

Step 2: The UE reports its CSI capability for the feature via Msg3.

Step 3: The UE receives a Msg4 that triggers the early SRS/CSI/CSI-RS.

Step 4: The UE performs the triggered aperiodic transmission/reception/reporting.

During last RAN1 meeting, the followint agreements were made:

This is, it was agreed to support at least aperiodic SRS-AS transmission and aperiodic CSI reporting (on PUSCH) triggered via Msg4 of the 4-Step RACH. In addition, both PMI-based reporting (Rel-15 Type-I SP codebook) and PMI-free reporting are supported.

In addition, the following two agreements were also made during the past meeting.

In a first agreement, it is detailed the four-step procedure for early CSI reporting for a UE in transition from IDLE to CONNECTED mode. In the second agreement, RAN1 narrows down into three options on how to report the corresponding UE capability of CSI resouce/report configurations.

Observation #7: There is no obvious connection between a change in RAN4 delay requirements due to early CSI triggering when UE transitions from IDLE to CONNECTED mode.

Proposal #5: RAN4 do not consider new timeline related requirements for early CSI triggering when UE transitions from IDLE to CONNECTED mode.

# Early CSI Acquisition for SCell Transition from Dormant to non-Dormant State

On a similar path as in SCell activation, the idea here is to make CSI available immediately after the UE switches out of dormancy. Similar to the SCell activation case, a UE in the dormant state on an SCell BWP cannot perform PDCCH monitoring, reporting aperiodic CSI or transmitting SRS to the network. According to the WID, the focus, since is perhaps most efficient method, is to enable the triggering of aperiodic SRS/CSI/CSI-RS using the same DCI that signals the switch out of dormancy state. How this can avoid the extra delay and overhead of a separate signalling, wether to reuse or improve existing DCI formats, or create a new DCI format is still being discussed. In addition, the use and / or enhancement for periodic or semi-persistent SRS remains to be discussed.

Observation #8: No agreements have been made in RAN1 room regarding the SCell Transition from Dormant to non-Dormant State use case.

Observation #9: Current SCell Dormancy framework already contains CSI reporting, so it’s not entirely clear how this feature would impact RAN4 requirements in this use case.

Proposal #6: Postpone the discussion of SCell transition from dormant to non-dormant state until further progress has been achieved in RAN1.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: Current MIMO Phase 6 WI contains three main parts: 1) Enhancing SRS UL capacity and coverage, 2) Early Triggering of SRS-AS/CSI/CSI-RS, and 3) Reducing CSI-RS overhead.

Proposal #1: RAN4 to prioritize Early Triggering of SRS-AS/CSI/CSI-RS for discussion in RAN4#116bis since is the scenario with obvious RRM impact.

Observation #2: There are a number of restrictions on what the UE can do when a SCell is deactivated, being one of them PDCCH monitoting.

Observation #3: One alternative to reduce SCell activation delay discussed in RAN1 is concurrent MAC CE command processing and PDCCH monitoring during deactivated state.

Observation #4: A more efficient alternative is to consider an enhanced MAC CE command with early triggering support.

Proposal #2: RAN4 start in RAN4#117 an analysis of the shortened timeline for Early Triggering of SRS-AS/CSI/CSI-RS in SCell Activation and Fast SCell activation for known cells based on legacy MAC CE SCell activation command based on progress during the RAN1 October meeting.

Observation #5: Some companies have proposed in RAN1 considering not only using aperiodic CSI-RS but also periodic and semi-persistent CSI-RS resources.

Proposal #3: For early triggering of CSI/CSI-RS, RAN4 to prioritize aperiodic CSI reporting/resources while waiting for RAN1 further progress.

Observation #6: Current framework for SCell Activation containes a variety of cases depending on knowlege of the SCell, UE measurement capability, and other aspects. In addition, the difference between Rel-15 vs Rel-17 frameworks is another aspect to consider.

Proposal #4: RAN4 to study what specific scenarios in the SCell activation framework to consider for early triggering of SRS-AS/CSI/CSI-RS.

Observation #7: There is no obvious connection between a change in RAN4 delay requirements due to early CSI triggering when UE transitions from IDLE to CONNECTED mode.

Proposal #5: RAN4 do not consider new timeline related requirements for early CSI triggering when UE transitions from IDLE to CONNECTED mode.

Observation #8: No agreements have been made in RAN1 room regarding the SCell Transition from Dormant to non-Dormant State use case.

Observation #9: Current SCell Dormancy framework already contains CSI reporting, so it’s not entirely clear how this feature would impact RAN4 requirements in this use case.

Proposal #6: Postpone the discussion of SCell transition from dormant to non-dormant state until further progress has been achieved in RAN1.

# References

- RP-252957 “New WID: NR Demodulation Performance, Phase 6”, by RAN4 chair (Apple), 3GPP TSG-RAN Meeting #109, Beijing, China, September 15-18, 2025