# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513608

# Prague, Czech Republic, October 13 – 17, 2025

Agenda item:	4.22.3

Source:	Apple

Title:	On RRM core requirements for LP-WUR

Document for:	Discussion

# Introduction

In last RAN4#116 meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [1], and open issues have been summarized in [2].

In this contribution, we continue the discussion on the open issues in [1].

# General aspects

Issue 1-1-2: Threshold for switch between different cases

So far, RAN2 still didn’t have clear conclusion about whether neighbor cell measurement can be triggered by LR measurement or not. Our understanding is LR measurement can be used for UE to decide switch from case#1 to case#3, that is, serving cell measurement on LR will be used to decide (1)if the neighbor cell measurement is triggered and (2)if the neighbor measurement can be relaxed. Previously in R16/17/18, the neighbor cell triggering and relaxation criteria checking is based on the MR measurement.

In previous RAN2 meeting it was agreed that,

- RAN2 #130 meeting - The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR. - It is up to NW to configure the condition for LP-WUS monitoring and/or [R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading], as in the current RRC running CR. - The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR. - RAN2 #129bis meeting - RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). - It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell. - The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ. - How to define LP-RSRP and LP-RSRQ is up to RAN1.

This issue has been discussed in RAN4 for couple of meetings, but we cannot foresee RAN2’s plan on discussing this dedicated issue. Thus, one possible way is to directly send LS to ask for RAN2’s guidance.

Proposal 1: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

If RAN4 cannot make conclusion on issue 1-1-2, we recommend RAN4 sending LS to RAN2 for guidance.

Issue 1-1-16: LP-WUR operation with EMR

Agreement: When both Rel-16 EMR and Rel-19 LP-WUR are configured: When T331 is running - When Case#1 (RRM offloading) conditions are met: - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s). - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s). - When Case#3 (RRM relaxation) conditions are met: - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s). - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s). - if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ - the UE shall search for NR inter-frequency layers configured and not configured for idle mode CA/DC measurements by following the legacy measurement requirements (no relaxation) when T331 is not running, - Serving cell, intra-frequency and NR inter-frequency layers not configured for idle mode CA/DC measurements and NR inter-frequency layers configured for idle mode CA/DC measurements and mobility measurement follow corresponding agreed requirements defined in case 1 and case 3 when they are satisfied.

In last meeting, the agreement for LP-WUR based RRM and EMR indicate that if T331 is running and neighbor equal or lower priority inter-frequency measurement is not triggered, UE will perform EMR carrier and higher priority carrier with the same measurement period, i.e., measurement interval is 60s. Actually, that’s different logic from the R16 power saving, and it could cause quite different UE implementation on MR to treat higher priority carrier and EMR carrier. In current spec, it was defined following for power saving:

4.2.2.10.3 Measurements for UE fulfilling not-at-cell edge criterion The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running. When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, then regardless of whether the UE is configured with highPriorityMeasRelax [2] or not, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

- Considerations on higher priority frequency layer - For case 1: - MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60 - Note: RAN4 assumes Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met for case 1. - For case 3: - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60

It’s very clear that in R16 RRM relaxation with EMR, the EMR carrier measurement is not relaxed and the higher priority carrier measurement is using 60sec time interval. However, in case #1 and case #3, the EMR carrier has also been relaxed to be equivalent as higher priority carriers. By doing so, it will cause impact to the mobility because of relaxing EMR measurement to 60 sec and impact to the MR power saving because of tightening higher priority carrier measurement to 60sec. We prefer to not change the previous UE behavior too much on MR, and therefore we propose followings:

Proposal 2: When both Rel-16 EMR and Rel-19 LP-WUR are configured:

- When T331 is running

  - When Case#1 (RRM offloading) conditions are met:

    - The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC.

    - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements in LR based RRM (i.e., based on K2*Thigher_priority_search and K2 = 60).

  - When Case#3 (RRM relaxation) conditions are met:

    - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ

      - The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC.

      - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements in LR based RRM (i.e., based on K2*Thigher_priority_search and K2 = 60).

# Others RRM relaxation

Issue 1-5-4: LR based RRM when stationary or low-mobility criteria is configured for power saving feature

In RAN2, there were some agreements related with power saving criteria as followings:

In RAN2#127bis meeting, followings were agreed: - RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE - Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. In RAN2#129bis meeting, followings were agreed: - RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). - RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both. - Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion). In RAN2#130 meeting, followings were agreed: - The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR. - It is up to NW to configure the condition for LP-WUS monitoring and/or [R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading], as in the current RRC running CR. - The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR.

The low mobility related relaxation was not concluded yet in RAN2, and UE behavior is unclear for relaxation if low-mobility criteria is configured and not met. Like in figure 2, UE is in case#1 based on the fulfilled condition of case#1, but UE does not meet the low mobility criteria from the UE power saving feature, we think it might be very risky from mobility perspective for UE to turn off the MR completely. Since there might be different UEs in cell coverage, some of them may support LP-WUR and power saving feature and some of them may only support power saving feature, network configures both criteria for MR offloading and UE power saving for different UEs via the system information, thus it’s worthwhile to investigate this case when low mobility criteria is configured but not met in case#1 and case#3.

The case #1 and case #3 will also relax the serving cell measurement, and it was not allowed in R16/17 power saving feature. If mobility condition/criteria are configured for UE power saving (either configured with not-at-cell-edge criteria or configured without NCE not-at-cell-edge criteria) and UE does not meet the low mobility condition, the UE behavior for relaxation from case #1 and case #3 shall not be applied since it will cause big potential negative impact on the mobility performance.

Thus, our view is: in case #1 and case #3, if R16/17 low mobility or stationary condition/criteria is configured, including with not-at-cell-edge(NCE) criteria and without NCE criteria, UE shall follow the principle of RRM relaxation in R16/17 power saving requirement.

Figure 2. example for UE with high mobility in case#1

Proposal 3: in case #1 and case #3 for LP-WUR based RRM, if R16/17 low mobility or stationary condition/criteria is configured (including with/without not-at-cell-edge(NCE) criteria), UE shall follow the principle of RRM relaxation in R16/17 power saving requirements for IDLE/Inactive mode.

# Simulation Assumptions

Simulations conducted by RAN4 are continuously aligning with the evaluations performed in RAN1. In addition, RAN4 RF session continues working on determining the minimum SNR for LP-WUS detection, which in turns will determine the RRM side condition.

During the past RAN4#114bis, some companies have raise the issue of LP-SS into LP-SS interference. This is, when the desired OOK signal is interfered by another OOK signal from another cell, which is configured with the same time and frequency resources. This, in contrast to the scenario that LP-SS receives interference from an OFDM source like PDSCH or another NR channel.

One can observe that the pattern of the interfering LP-SS can play a relevant role, since the definition of RSRP is the total energy seen during the on-duration minus the energy seen during the off-duration, which is clearly different when an OOK waveform is the interference source. As shown above, RAN1 has desiged the LP-SS to have up to four different patterns for each value of M and L.

Observation 1: Companies have raised concerns about the deployment scenario where LP-SS signals are scheduled on the same frequency resources across co-channel cells, suggesting precluding such operation as a way forward.

Observation 2: RSRP/RSRQ accuracy results might be dependent on the binary patterns of desired LP-SS vs the binary patterns of the interfering LP-SS, i.e. the “opposite sequence” problem.

However, it results quite impractical to determine requirements based on which specific LP-SS combination of serving vs interference cell is considered. A better approach might be simply to take the worst case scenario – although determining such scenario might be time consuming.

Proposal 4: RAN4 to discuss if RSRP/RSRQ accuracy requirements should be based on the worst case scenario of colliding LP-SS binary patterns between serving cell and interfering cell, or instead the measurement requitements should not be applicable to the “opposite sequence” case.

# Conclusion

In this contribution, we continue the discussion on the open issues in [2].

Proposal 1: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

Proposal 2: When both Rel-16 EMR and Rel-19 LP-WUR are configured:

- When T331 is running

  - When Case#1 (RRM offloading) conditions are met:

    - The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC.

    - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements in LR based RRM (i.e., based on K2*Thigher_priority_search and K2 = 60).

  - When Case#3 (RRM relaxation) conditions are met:

    - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ

      - The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC.

      - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements in LR based RRM (i.e., based on K2*Thigher_priority_search and K2 = 60).

Proposal 3: in case #1 and case #3 for LP-WUR based RRM, if R16/17 low mobility or stationary condition/criteria is configured (including with/without not-at-cell-edge(NCE) criteria), UE shall follow the principle of RRM relaxation in R16/17 power saving requirements for IDLE/Inactive mode.

Observation 1: Companies have raised concerns about the deployment scenario where LP-SS signals are scheduled on the same frequency resources across co-channel cells, suggesting precluding such operation as a way forward.

Observation 2: RSRP/RSRQ accuracy results might be dependent on the binary patterns of desired LP-SS vs the binary patterns of the interfering LP-SS, i.e. the “opposite sequence” problem.

Proposal 4: RAN4 to discuss if RSRP/RSRQ accuracy requirements should be based on the worst case scenario of colliding LP-SS binary patterns between serving cell and interfering cell, or instead the measurement requitements should not be applicable to the “opposite sequence” case.

# References

[1] R4-2512139, WF on RRM requirements for NR_LPWUS, Moderator (vivo), RAN4 #116

[2] R4-2509065, Topic summary for [116][222] NR_LPWUS, vivo, RAN4 #116