# 3GPP TSG-RAN WG4 Meeting #116bis                             R4-2513662

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.11.3

Source: 	CMCC

Title: 			 Discussion on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion core requirements for CSI prediction.

- Discussion

Issue 1-1: Performance monitoring Agreement: RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction FFS which requirements are to be introduced. Issue 1-2: Requirement baseline for monitoring Agreement: Introduce the following requirements for CSI performance monitoring: - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this Issue 1-4: Reporting delay requirement Agreement: wait for RAN1 decision

For performance monitoring, RAN1 agreed to support SGCS as a performance metric for performance monitoring type 3 (i.e. UE reports performance metric to NW). In detail, reporting contents of UE assisted performance monitoring includes two SGCS: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

RAN1 agreements in RAN1#120bis meeting Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. RAN1 agreements in RAN1#121 meeting Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1 Agreement For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs. SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. - v is the value of the reported RI in the associated inference report. RAN1 agreements in RAN1#122 meeting Agreement Adopt following TPs for TS38.214 Section 5.2.1.4.6. Reason for change; In current specification, there is no SGCS quantization mapping table.. Summary of change; SGCS quantization mapping table is added.. Consequences if not approved; Incomplete capture of the agreement.. TS38.214 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> - each calculated SGCS value is quantized to a 4-bit field, where the first codepoint ‘0000’ is used to indicate (0 0.3] value range, and the value range [0.3, 1] is quantized in a linear scale with 0.05 step size. Detailed mapping is specified in [xx, TS 38.abc]Table 5.2.1.4.6-1. Table 5.2.1.4.6-1: SGCS Quantization Mapping Code Point; Range. 0; [0.0000, 0.3000]. 1; (0.3000, 0.3500]. 2; (0.3500, 0.4000]. 3; (0.4000, 0.4500]. 4; (0.4500, 0.5000]. 5; (0.5000, 0.5500]. 6; (0.5500, 0.6000]. 7; (0.6000, 0.6500]. 8; (0.6500, 0.7000]. 9; (0.7000, 0.7500]. 10; (0.7500, 0.8000]. 11; (0.8000, 0.8500]. 12; (0.8500, 0.9000]. 13; (0.9000, 0.9500]. 14; (0.9500, 1.0000]. 15; Reserved. << Unchanged parts are omitted >>.

Normally, RAN4 need to define reporting mapping for the reported quantity. SGCS is newly introduced, and reporting mapping is required. According to RAN1 agreements, the reported SGCS value is quantized with 4-bit. 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale, and one codepoint is used to indicate the value range of (0 0.3]. According to RAN1 latest agreements, SGCS quantization mapping table is added to TS 38.214. RAN4 no need to define the reporting mapping for SGCS.

Proposal 1: for CSI prediction, RAN4 no need to define report mapping for SGCS, since RAN1 has defined the SGCS quantization mapping table in TS38.214.

In RAN1 #121 meeting, it was agreed that for CSI prediction using UE-side model, for inference, in addition to legacy Z/Z’ for doppler codebook, UE may report the value of t per SCS. Based on RAN1 agreements, the inference reporting delay shall be at least legacy Z/Z’ plus t, t is reported by UE.

RAN1 agreements in RAN1#121 Agreement For CSI prediction using UE-side model, for inference, in addition to legacy Z/Z’ for doppler codebook, UE may report the value of t per SCS - Detailed value of t can be discussed in UE feature

Proposal 2: based on RAN1 agreements, for CSI prediction, the inference reporting delay is legacy Z/Z’ plus t, where t is reported by UE.

- Conclusion

This contribution provides discussion on CSI prediction. The proposals are:

Proposal 1: for CSI prediction, RAN4 no need to define report mapping for SGCS, since RAN1 has defined the SGCS quantization mapping table in TS38.214.

Proposal 2: based on RAN1 agreements, for CSI prediction, the inference reporting delay is legacy Z/Z’ plus t, where t is reported by UE.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2502970, Baseline Simulation assumptions for AIML based CSI prediction