# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513668

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.14.3

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements for A-IoT

Document for:	Discussion

- Introduction

In RAN #107 meeting, the revised WID on Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. According to the WID, the objectives of RAN4 scope are duplicated as following.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

In last meeting, there is discussion on RRM performance requirements for A-IoT device 1, and a WF was approved [2]. This contribution provides discussion on this topic.

- Discussion

In last meeting, it was agreed that random access and D2R transmit timing requirements will be tested in RAN4 test cases.  According to the core part discussion in last meeting, TD2R_min is defined, and the value of TD2R_min is max {3* TD2R_chip, 10 us}, where TD2R_chip is the chip length of D2R transmission. TD2R_min is the minimum time between a D2R transmission and the corresponding R2D transmission following it. A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. It can be seen that TD2R_min is about device processing time between D2R transmission and R2D reception. It is proposed to discuss whether to define test for TD2R_min.

Proposal 1: it is proposed to discuss whether to define test for TD2R_min, which is about device processing time between D2R transmission and R2D reception.

Issue 2-1-1: Test scenario Agreement: - The random access and D2R transmit timing requirements will be tested in RAN4 test cases, FFS whether dedicated RRM test or combined with RF/demod test.

For FFS whether dedicated RRM test or combined with RF/demod test, in our view, it is preferred to follow legacy way to define dedicated RRM tests. We understand the motivation to be combined with RF/demod test is to reduce test cost. But we are not sure whether it is feasible to verify RRM requirements in RF/demod test. Following is the summary of RRM impact in the core part discussion. We are not sure whether RRM requirements, e.g. A-IoT random access procedure, can be verified in RF/demod test.

Agreements in RAN4 #115 meeting (R4-2508275) Issue 1-1: A-IoT random access procedure Agreement: - RAN4 to include following aspects for RRM requirements for A-IoT random access. The details of UE behaviour will be updated based on RAN2 spec. - CBRA procedure - the correct behaviour when transmitting A-IoT MSG1(Random ID transmission) - the correct behaviour when receiving A-IoT MSG2 - the correct behaviour when not receiving A-IoT MSG2 - CFRA procedure - the correct behaviour when transmitting First D2R message Issue 1-2-1: Device transmit timing Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any. Agreements in RAN4 #116 meeting (R4-2508275) Issue 1-2-1: Device transmit timing Agreement: - For D2R transmission, the device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last transition edge of PRDCH of the corresponding R2D transmission to the starting time of D2R transmission. The reference point for the device transmission timing is TR→D after the end of R2D transmission intended for the device as define in 7.1.2 TS 38.291 - TR→D is - For CBRA MSG1 X=1: TR→D is Toffset1 after the end of corresponding R2D transmission - For CBRA MSG1 X=2: TR→D is Toffset1+ Toffset2 after the end of corresponding R2D transmission - For CBRA MSG3: TR→D is Toffset3 after the end of corresponding R2D transmission - For CFRA first D2R: TR→D is Toffset3 after the end of corresponding R2D transmission - For D2R transmission except for Msg1 and Msg3: TR→D is Toffset4 after the end of corresponding R2D transmission - Note: considering that TR→D is in use in RAN1 spec (TS 38.291), above details about TR→Dwill not be captured in TS38.191 Issue 1-2-2: The value for TD2R_min Agreement: - The value of TD2R_min is max {3* TD2R_chip, 10 us}, where TD2R_chip is the chip length of D2R transmission Issue 1-2-3: Whether to define accuracy requiements for TD2R_min Agreement: - Not define accuracy requirements on TD2R_min.

Proposal 2: it is preferred to define dedicated RRM test to verify RRM requirements.

For legacy UE transmit timing test, the transmit timing is verified by the UE transmitting SRS. After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB. Similarly, for A-IoT, the transmit timing is verified by device D2R transmission. Taking RACH as an example, the test equipment will verify that the timing of the device is within  a period, e.g. (Toffset1 ± Te_SFO),  of the end the corresponding R2D transmission triggering random access.

Proposal 3: For RACH, the transmit timing is verified by the device D2R transmission, and the test equipment will verify that the timing of the device is within a period of the end the corresponding R2D transmission triggering random access, where the period is related with TR→D and timing error due to SFO.

- Conclusion

This contribution provides discussion on RRM performance requirements for A-IoT. The proposals are:

Proposal 1: it is proposed to discuss whether to define test for TD2R_min, which is about device processing time between D2R transmission and R2D reception.

Proposal 2: it is preferred to define dedicated RRM test to verify RRM requirements.

Proposal 3: For RACH, the transmit timing is verified by the device D2R transmission, and the test equipment will verify that the timing of the device is within a period of the end the corresponding R2D transmission triggering random access, where the period is related with TR→D and timing error due to SFO.

- Reference

- RP-250796, Revised WID: New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc

- R4-2512134, WF on RRM for Ambient_IoT_Solutions, CMCC

- R4-2508275, WF on RRM requirements for Ambient_IoT_Solutions, CMCC

- R4-2504905, WF on RRM requirements for Ambient_IoT_Solutions, CMCC