3GPP TSG-RAN WG4 Meeting #116bis			R4-2513680

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	4.25.3

Source: 	CMCC

Title: 			   Discussion on the RRM requirement maintenance for NTN phase3

Document for:	Discussion

- Introduction

In last meeting, the core part of NTN phase 3 has been finished, but some maintenance issue still need to be further discussed[1][2].

Besides, a LS was received from RAN2#131 meeting[3], which inform RAN4 about additional agreements on SMTC enhancements. RAN4 need to take them into account when define the RRM requirements.

For downlink coverage enhancement, RAN2 further made the following agreements related to SMTC enhancement: - We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress). - The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs). - RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements. - In connected mode the NW will only configure up to 4 STMC. - RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only. RAN2 kindly request RAN4 to take above agreement into account when designing RAN4 RRM requirement for SMTC enhancement and provide feedback if needed.

In this contribution, we further discuss the maintenance issues.

- Maintenance for Downlink coverage enhancements

## [Online # 2] Issue 0-A: More than 4 SMTCs per frequency (i.e. 6)

RAN4 to further discuss the following in RAN4#116-bis:

For RRC Idle/Inactive mode RRM requirements, no update is needed except for the following addition to the specification where relevant:

- The SMTCs referenced in the subsequent requirements refer to the SMTCs selected by the UE.

- Note: The SMTC selection rule may be further discussed and specified or left to UE implementation.

In RRC_IDLE/Inactive mode, we support the rule in above first bullet, that the SMTCs referenced in the subsequent requirements refer to the SMTCs selected by the UE.

Regarding the SMTC selection rule, RAN2 agreed that:

- RAN2 introduce a capability to support STMC enhancements to FR1

- If the NW configures more than 4 SMTCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements.

- RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band).

The details of two capabilities are as follows:

Location based SMTC selection for RRC_IDLE/RRC_INACTIVE It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support SMTC selection based on UE position and associated location for each SMTC in NTN, as specified in TS 38.331 [9].

twoSMTC-Periodicities-r19 Indicates whether the UE supports NTN SSB based RRM measurements on target cells using two SMTC periodicities on a single frequency carrier. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17.

Based on RAN2’s further agreement, the selection rule shall be also captured in RAN4 spec. We propose following wording on top of the existing specification, take intra-frequency cell measurement as an example:

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, and the number of SMTC periodicity doesn’t exceed 1 if UE is not capable of twoSMTC-Periodicities-r19, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. If UE doesn’t support twoSMTC-Periodicities-r19,  the selection of SMTCs to be used shall have same periodicity with smtc1. If UE support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, the selection of SMTCs to be used is based on UE position and associated location for each SMTC in NTN, as specified in TS 38.331 [2], otherwise, the selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected.

Proposal 1: In RRC-IDLE/Inactive, update the SMTC selection rule, following wording of intra-frequency cell measurement is proposed as an example:

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, and the number of SMTC periodicity doesn’t exceed 1 if UE is not capable of twoSMTC-Periodicities-r19, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. If UE doesn’t support twoSMTC-Periodicities-r19,  the selection of SMTCs to be used shall have same periodicity with smtc1. If UE support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, the selection of SMTCs to be used is based on UE position and associated location for each SMTC in NTN, as specified in TS 38.331 [2], otherwise, the selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Proposal 1: In RRC-IDLE/Inactive, update the SMTC selection rule, following wording of intra-frequency cell measurement is proposed as an example:

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, and the number of SMTC periodicity doesn’t exceed 1 if UE is not capable of twoSMTC-Periodicities-r19, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. If UE doesn’t support twoSMTC-Periodicities-r19,  the selection of SMTCs to be used shall have same periodicity with smtc1. If UE support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, the selection of SMTCs to be used is based on UE position and associated location for each SMTC in NTN, as specified in TS 38.331 [2], otherwise, the selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected.

- Reference

- R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)

- R4-2512337, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- R2-2506298, LS on additional agreements on SMTC enhancements, RAN2