# 3GPP TSG-RAN WG4 Meeting #116bis		  R4-2513693

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.14.4

Source:		Qualcomm Incorporated

Title:			Views on demodulation requiremetns for AIoT device 1

Document for:	Approval

# 1  Introduction

In RAN4#116, the WF was approved in [1] with the following agreements on whether to define PRDCH demodulation requirements for AIoT device 1.

Issue 2-1: Whether to define PRDCH demodulation requirements for Ambient-IoT - Way forward - Option 1: Define PRDCH demodulation requirement. - Option 2: Not define PRDCH demodulation requirements for Ambient-IoT considering that it can be verified by the RF REFSENS requirement

In this paper, we provide our view on demodulation requirements for AIoT device 1.

# 2  Discussion

For the R2D link, there is only one physical channel—PRDCH—which carries both control and data information. In the case of PRDCH, only OOK modulation is used for R2D, which differs from NR PDSCH that supports multiple modulation schemes such as QPSK, 16QAM, 64QAM, etc. Additionally, the reference sensitivity level for OOK in R2D has been specified under the AWGN channel in TS 38.191 [2]. Therefore, the main distinction between R2D reference sensitivity and PRDCH demodulation requirements testing lies in whether a fading channel, such as TDLA, is emulated.

Observation 1: The main difference between R2D reference sensitivity and demodulation requirements testing for PRDCH is whether fading channel, e.g., TDLA, is emulated.

Since highly integrated architecture for device 1 is assumed, conducted antenna connector is not available. Therefore, OTA testing is applied for device 1 RF requirements verification such as backscattering, reference sensitivity. Hence, if RAN4 agrees to define the demodulation requirements for PRDCH, the OTA test method to allow modelling of TDLA fading channel in the chamber needs to be studied.

From our understanding, the following testability issues need to be solved for PRDCH demodulation requirements testing:

- Fading channel model

When the OTA test method was discussed for verifying AIoT device RF requirements, it was agreed that reusing the legacy TRP and TRS test setup would help keep testing costs reasonable. It is commonly understood that the industry does not expect to build a new test setup for AIoT devices, as they are considered a low-cost category. However, with the current TRP and TRS setup, integrating a channel emulator to generate a TDL-A fading channel remains a challenge.

Observation 2:  With the current TRP and TRS test setup, it is challenging to add channel emulator to generate TDL-A fading channel.

In addition, since the time to complete charging, and the duration the device can remain ON after charging is not specified and is varying by implementation, the fading channel condition could differ across devices. This inconsistency poses a challenge that must be addressed if the fading channel is to be used for PRDCH demodulation requirements testing.

Observation 3: The fading channel condition could differ across devices. This inconsistency poses a challenge that must be addressed if the fading channel is to be used for PRDCH demodulation requirements testing.

- Measurement setup and test parameters

As specified in [3], FR2 UE demodulation is based on OTA test method. To control DL SNR at reference point (i.e., QZ), ‘wireless cable’ approach is adopted in which the test system will transmit the useful signal and noise signals together to emulate target SNR condition. For example, wanted noise is set as 6dB higher than UE thermal noise to ensure at most 1dB difference between reference point SNR and baseband SNR at peak EIS direction. Therefore, if the similar approach is applied for AIoT device demodulation testing, the test procedure would be very complicated including finding the proper test direction, calculating the feasible SNR range, etc.

Observation 4:  Feasibility of applying ‘wireless cable’ approach to PRDCH demodulation testing is unclear and the test procedure would be very complicated including finding the proper test direction, calculating the feasible SNR range, etc., which is not proper for low-cost AIoT device testing.

With above observations, we propose NOT to specify PRDCH demodulation requirement for device 1 in Rel-19.

Proposal 1: RAN4 do NOT specify PRDCH demodulation requirement for device 1 in Rel-19.

# 3. Conclusion

In summary, we have following observations and proposals.

Observation 1: The main difference between R2D reference sensitivity and demodulation requirements testing for PRDCH is whether fading channel, e.g., TDLA, is emulated.

Observation 2:  With the current TRP and TRS test setup, it is challenging to add channel emulator to generate TDL-A fading channel.

Observation 3: The fading channel condition could differ across devices. This inconsistency poses a challenge that must be addressed if the fading channel is to be used for PRDCH demodulation requirements testing.

Observation 4:  Feasibility of applying ‘wireless cable’ approach to PRDCH demodulation testing is unclear and the test procedure would be very complicated including finding the proper test direction, calculating the feasible SNR range, etc., which is not proper for low-cost AIoT device testing.

Proposal 1: RAN4 do NOT specify PRDCH demodulation requirement for device 1 in Rel-19.

# 4  References

- R4-2512603, Way Forward for [116][335] A-IoT_demod, RAN4#116, Aug 2025.

- 3GPP TS 38.191v1.0.0

- 3GPP TR 38.810