3GPP TSG-RAN WG4 Meeting #116bis	R4-2513707

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussion on RRM performance for Rel-19 NES adaptation

Agenda Item:	6.15.2.2

Document for:	Discussion

- Introduction

The core part of Rel-19 NES was completed in last meeting with agreements captured in [1]. In this contribution, we provide our analysis on performance part for SSB adaption.

- Discussion

The performance part has been initially discussed in last meeting with following agreements.

Issue 3-3-1: Test case list Agreement RAN4 to at least define the following SSB adaptation test case. - L3 intra-frequency measurement RAN4 to discuss whether the following SSB adaptation test case list can be defined. - L1-RSRP - L1-SINR - CBD Issue 3-3-2: General principle RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed. - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only. - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - without SSB time index detection.

As agreed in last meeting, RAN4 will define TC for L3 intra-frequency measurement. In details, UE will measure the serving Cell with additional SMTC upon receiving DCI 2-9. To verify the behaviour, Event A1 or A2 shall be configured, and the condition of serving Cell is changed to satisfy the Event threshold at the same time as DCI 2-9. And it is verified whether UE can trigger event within the delay based on the mapped additional SMTC.

Proposal 1: RAN4 to define one test for L3 Intra-frequency serving Cell measurement.

Proposal 2: For L3 Intra-frequency serving Cell measurement, the details are proposed as follows:

- UE is configured with Event A1 or A2.

- Condition of serving Cell is changed to satisfy the Event threshold at the same time as DCI 2-9

- It is verified whether UE can trigger event within the delay based on the mapped additional SMTC.

- Conclusion

Proposal 1: RAN4 to define one test for L3 Intra-frequency serving Cell measurement.

Proposal 2: For L3 Intra-frequency serving Cell measurement, the details are proposed as follows:

- UE is configured with Event A1 or A2.

- Condition of serving Cell is changed to satisfy the Event threshold at the same time as DCI 2-9

- It is verified whether UE can trigger event within the delay based on the mapped additional SMTC.

References

[1] R4-2512327	WF on RRM requirements for Netw_Energy_NR_enh_Part1