3GPP TSG-RAN WG4 Meeting # 116-bis														 R4-2513735

Prague, Czech, 13th – 17th October, 2025

Agenda item:			7.8.3

Source:	Samsung

Title:	Views on UE RF requirements for LB CA switching

Document for:	Discussion

1	Introduction

At the latest RAN plenary meeting, RAN#109, a package of Rel-20 RAN4-led Work Items (WIs) was approved including a WI on enhancement of low NR band (LB) carrier aggregation (CA) via switching. The WID includes following objectives based on precedent discussions in Rel-19 [1].

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

In this contribution, given the switching scenario between Case 1 and Case 3 under LB CA and its constraints as noted in the WID, we would like to provide our initial views focusing on UE RF aspects based on the outcome of the Rel-19 discussion [2, 3].

2	Discussion

In Rel-19, RAN4 specified the framework of LB CA via switching, particularly focusing on scenarios involving FDD and SDL carriers. The key idea was to allow UEs that do not have the capability to support conventional low-band CA with dual receive chains to still benefit from a form of aggregation by enabling switching between carriers. The Rel-19 WI established the uplink time mask as the central mechanism to guarantee coexistence, ensuring that a UE could safely switch between UL on an FDD carrier and DL reception on an SDL carrier. This framework served well in providing limited CA-like benefits while keeping implementation complexity under control.

For Rel-20, the scope expands beyond simple FDD/SDL switching into the dual downlink scenario, where both an FDD DL carrier and an SDL carrier may be received simultaneously under the switching framework. While this appears at first glance to resemble conventional DL CA, the critical difference is that the objective remains to support single-Rx implementations that cannot economically or technically realize full CA in low-band frequencies. The Rel-20 case therefore raises new questions for RAN4 on how receiver requirements should be applied and what additional constraints are necessary to keep the specification both realistic and implementable.

Observation 1:	The scope of the Rel-20 WI expands beyond simple FDD/SDL switching into the dual downlink scenario, where both an FDD DL carrier and an SDL carrier may be received simultaneously under the switching framework.

The uplink behavior under Rel-20 dual DL switching remains fundamentally identical to Rel-19. In the Rel-19 framework, uplink transmission was controlled via strict time mask definitions, ensuring that UL on the FDD carrier and DL on the SDL carrier could coexist without excessive interference. In the Rel-20 dual DL scenario, the situation is in fact simplified: while two downlink carriers are active simultaneously, uplink transmission on the FDD DL carrier is not permitted. This means that no new uplink-related requirements are necessary, and the existing Rel-19 UL time mask can be directly reused without any modification. By doing so, continuity between releases is preserved, and redundant specification work is avoided.

Observation 2:	The uplink behavior under Rel-20 dual DL switching remains fundamentally identical to Rel-19.

Proposal 1:		No new uplink-related requirements are necessary, and the existing Rel-19 UL time mask can be directly reused without any modification.

The WID initially describes its scope as applying to adjacent FDD DL and SDL band combinations under the condition of “no-gap impact.” To capture this condition more clearly, it is proposed that “no-gap impact” be redefined as a contiguity-equivalent condition. Specifically, a maximum allowable separation, e.g., Δf_LB, should be considered, below which the two carriers are considered to be effectively contiguous. Furthermore, this maximum separation should not be considered in isolation, but rather in conjunction with the maximum aggregated channel bandwidth (CBW) that a single Rx chain can process. This is because the effective impact on the Rx front-end depends not only on the absolute gap but also on the total span of frequencies that must be passed through the analog chain, including filters, LNAs, and ADC front-ends.

By defining “no-gap impact” in this way, the feature can accommodate more practical band combinations in the market, which include very small separations but are still effectively contiguous from an RF perspective. At the same time, this approach ensures that configurations with larger separations, which would stress the Rx chain beyond feasible limits, are not inadvertently included within the WI scope.

Observation 3:	The WID initially describes its scope as applying to adjacent FDD DL and SDL band combinations under the condition of “no-gap impact.”

Proposal 2:		A maximum allowable separation below which the two carriers are considered in conjunction with the maximum aggregated channel bandwidth (CBW) that a single Rx chain can process.

Regarding RF architecture considerations, in our view, the motivation for this WI lies in supporting UEs with only a single low-band Rx chain. UEs that include dual low-band Rx chains can already declare conventional DL CA support, and for those UEs the existing CA requirements apply directly without any modification. However, the main target of this WI would be the large class of UEs that rely on a single low-band Rx chain for reasons of cost, size, or power efficiency to support LB. For such devices, the dual DL switching feature can be understood as a process where the single Rx chain alternates between narrower and wider spans. For example:

- Case 1: The Rx chain is configured to cover only the FDD DL carrier (narrowband).

- Case 3: The Rx chain is reconfigured to extend its passband such that both the FDD DL and SDL carriers are included simultaneously (wider band).

Figure 1 shows the difference between the single-Rx and dual-Rx architectures. In the single-Rx case, the same antenna, filter, LNA, and LO chain must handle the entire span, whereas in the dual-Rx case, separate chains process each carrier independently. This architectural distinction is fundamental to the scope of the WI.

Case 1: Single-Rx switching (dual DL via single Rx chain)

Case 2: Dual-Rx conventional CA (dual Rx chains)

Figure 1: Examples of UE Rx architectures for CA_n12A–n29A

Observation 4:	The main purpose of this WI is to enable UEs with only a single low-band Rx chain to realize some of the benefits of DL CA.

Based on the clarification, one of the most important technical questions raised by the Case-3 scenario would be how reference sensitivity (REFSENS) requirements should be applied. By following the legacy requirement of intra/inter-band CA, it would be simplest to state that each downlink component carrier must continue to meet the single-carrier REFSENS values. However, in practice, when the Rx chain expands to cover an extended span that includes both carriers, several factors could degrade performance relative to the single-carrier case. These include reduced filter selectivity, increased noise across the wider band, greater AGC load, and reduced quantization dynamic range in the ADC.

Observation 5:	When the Rx chain expands to cover an extended span that includes both carriers, several factors could degrade performance relative to the single-carrier case.

Therefore, simply requiring the UE to meet the original single-carrier REFSENS for each carrier in Case-3 may be unrealistic in some configurations. To resolve this without creating entirely new numerical sensitivity figures, RAN4 needs to consider a maximum aggregated channel bandwidth between two adjacent LBs, which can guarantee the single-Rx performance with legacy REFSENS criteria.

Proposal 3:		RAN4 needs to consider a maximum aggregated channel bandwidth between two adjacent LBs, which can guarantee the single-Rx performance with legacy Rx requirements.

By doing so, the existing per-carrier single-carrier REFSENS values apply. No new REFSENS values would be introduced into the specification, however, each downlink carrier should satisfy existing single-carrier requirements for the feature, which is following the legacy intra-band contiguous CA mechanism. The same reasoning applies to other receiver requirements such as blocking and adjacent channel selectivity (ACS). Each downlink carrier must individually meet the single-carrier requirements, but additional verification under simultaneous dual-carrier presence may be needed to ensure robustness. Since no uplink transmission occurs during dual downlink operation, self-interference scenarios involving UL leakage are absent, and no new intermodulation or spurious response requirements are proposed at this stage. Only if specific aggregated CBWs or separated gaps are later identified would new conditions need to be defined.

Proposal 4:		Each downlink carrier should satisfy existing single-carrier requirements for the feature, which is following the legacy intra-band contiguous CA mechanism.

Proposal 5:		No new intermodulation or spurious response requirements are proposed at this stage if specific aggregated CBWs or separated gaps are later identified.

Lastly, it is also essential to clarify that the scope of this WI is strictly limited to two-band combinations consisting of one FDD DL carrier and one SDL carrier. Extensions to three- or four-band combinations are explicitly excluded as discussed in Rel-19. In our view, introducing such multi-band cases would inevitably require considering transmissions on additional bands, uplink coexistence, and more complex interference conditions. Restricting the scope in this way ensures that the WI remains focused, manageable, and aligned with its original objective of supporting single-Rx UEs that cannot declare conventional DL CA capability.

Proposal 6:		Restricting the scope to two-band combinations consisting of one FDD DL carrier and one SDL carrier ensures that the WI remains focused, manageable, and aligned with its original objective of UEs that cannot support conventional DL CA.

3	Conclusion

In this contribution, we provide our initial views focusing on the UE RF aspects considering the agreed assumption and restriction in the Rel-20 WID [1].  Following observations and proposals were derived.

Observation 1:	The scope of the Rel-20 WI expands beyond simple FDD/SDL switching into the dual downlink scenario, where both an FDD DL carrier and an SDL carrier may be received simultaneously under the switching framework.

Observation 2:	The uplink behavior under Rel-20 dual DL switching remains fundamentally identical to Rel-19.

Proposal 1:		No new uplink-related requirements are necessary, and the existing Rel-19 UL time mask can be directly reused without any modification.

Observation 3:	The WID initially describes its scope as applying to adjacent FDD DL and SDL band combinations under the condition of “no-gap impact.”

Proposal 2:		A maximum allowable separation below which the two carriers are considered in conjunction with the maximum aggregated channel bandwidth (CBW) that a single Rx chain can process.

Observation 4:	The main purpose of this WI is to enable UEs with only a single low-band Rx chain to realize some of the benefits of DL CA.

Observation 5:	When the Rx chain expands to cover an extended span that includes both carriers, several factors could degrade performance relative to the single-carrier case.

Proposal 3:		RAN4 needs to consider a maximum aggregated channel bandwidth between two adjacent LBs, which can guarantee the single-Rx performance with legacy Rx requirements.

Proposal 4:		Each downlink carrier should satisfy existing single-carrier requirements for the feature, which is following the legacy intra-band contiguous CA mechanism.

Proposal 5:		No new intermodulation or spurious response requirements are proposed at this stage if specific aggregated CBWs or separated gaps are later identified.

Proposal 6:		Restricting the scope to two-band combinations consisting of one FDD DL carrier and one SDL carrier ensures that the WI remains focused, manageable, and aligned with its original objective of UEs that cannot support conventional DL CA.

Reference

[1] 	RP-252953	New WID on enhancement of low NR band carrier aggregation via switching		RAN#109

[2]	R4-2509202	Draft TR 38.768 v1.1.0 Low NR Band Carrier Aggregation via Switching			RAN4#116

[3]	R4-2511894	Introducing low NR band aggregation via switching to TS38.101-1					RAN4#116