3GPP TSG-RAN WG4 Meeting #116bis										R4-2513739

Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	5.11.3

Source: 	Fujitsu

Title: 	Discussion on BS-RF requirements for US 4.9GHz new band

Document for:	Approval

# 1. Introduction

At the RAN#108 meeting, a spectrum-related work item [1] was approved to specify a new NR TDD 4.9GHz band for US operation. In this paper, we would like to provide our view on BS-RF requirements for US 4.9GHz new band.

# 2. Discussion

(m) Emission Mask M. For high power transmitters (greater that 20 dBm) operating in the 4940-4990 MHz frequency band, the power spectral density of the emissions must be attenuated below the output power of the transmitter as follows:

(1) On any frequency removed from the assigned frequency between 0-45% of the authorized bandwidth (BW): 0 dB.

(2) On any frequency removed from the assigned frequency between 45-50% of the authorized bandwidth: 568 log (% of (BW)/45) dB.

(3) On any frequency removed from the assigned frequency between 50-55% of the authorized bandwidth: 26 + 145 log (% of BW/50) dB.

(4) On any frequency removed from the assigned frequency between 55-100% of the authorized bandwidth: 32 + 31 log (% of (BW)/55) dB.

(5) On any frequency removed from the assigned frequency between 100-150% of the authorized bandwidth: 40 + 57 log (% of (BW)/100) dB.

(6) On any frequency removed from the assigned frequency between above 150% of the authorized bandwidth: 50 dB or 55 + 10 log (P) dB, whichever is the lesser attenuation.

(7) The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz. The power spectral density is the power measured within the resolution bandwidth of the measurement device divided by the resolution bandwidth of the measurement device. Emission levels are also based on the use of measurement instrumentation employing a resolution bandwidth of at least one percent of the occupied bandwidth.

Regarding the BS-RF requirements for US 4.9GHz, the emission mask M has been stated as the applicable emission masks for 4940-4990 MHz in the FCC Title 47 Part 90.210 (m) for BS. The excerpt is as follows.

Observation 1: Emission mask M has been stated as the applicable emission masks for 4940-4990 MHz in the FCC Title 47 Part 90.210 (m) for BS.

6.6.4.2.5	Basic limits for additional requirements

6.6.4.2.5.1	Limits in FCC Title 47

In addition to the requirements in clauses 6.6.4.2.1, 6.6.4.2.2, 6.6.4.2.3 and 6.6.4.2.4, the BS may have to comply with the applicable emission limits established by FCC Title 47 [8], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

The above regulatory requirements need to be incorporated to the 3GPP spec. In TS 38.104 Section 6.6.4, the Basic limits for Wide Area BS, Medium Range BS and Local Area BS are defined as OBUE requirements. Additionally, in TS 38.104, the following has stated the Basic limits for additional requirements related to the FCC Title 47.

Observation 2: The Basic limits for additional requirements related to the FCC Title 47 has been stated in TS 38.104 Section 6.6.4.2.5.1.

In our understanding, the above description seems to cover all emission requirements established by FCC Title 47. It means that FCC Title 47 Part 90.210 (m) is also included, and there is no need to update the current description in TS 38.104. Considering it, our proposal is as follows.

Proposal: RAN4 to confirm whether TS 38.104 Clause 6.6.4.2.5.1 includes FCC Title 47 Part 90.210 (m) or not.

# 3. Conclusions

In this contribution, we provided our view on BS-RF requirements for US 4.9GHz new band as follows.

Observation 1: Emission mask M has been captured as the applicable emission masks for 4940-4990 MHz in the FCC Title 47 Part 90.210 (m) for BS.

Observation 2: The Basic limits for additional requirements related to the FCC Title 47 has been stated in TS 38.104 Section 6.6.4.2.5.1.

Proposal: RAN4 to confirm whether TS 38.104 Clause 6.6.4.2.5.1 includes FCC Title 47 Part 90.210 (m) or not.

# 4. Reference

- RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu