3GPP TSG-RAN WG4 Meeting #116BIS							   R4-2513760

Prague, Czech, 13th – 17th Oct, 2025

Agenda Item: 	7.1.4

Source: 	MediaTek Inc.

Title: 	Initial view for TN HP related topics for both HHUE and FWA

Document for: 	Discussion & Decision

# Introduction

In the last RAN plenary # 109 Meeting, RAN approved the UE RF enhancements for NR FR1, Phase 5 WI [1] as following work scope. In this paper we propose our initial view how to define HP related UE RF requirements for both FWA and HHUE in Rel-20.

- 4x26dBm (up to 32dBm) high Power UE for FWA only

- 2x26dBm PC1.5 UE in FDD band for both HHUE and FWA

- Feasibility study from 4Q 2026 as following two sub-topics (start 4Q 2026)

  - PC1.5 1Tx (1x29dBm) for TDD bands for FWA UE

  - PC1 2Tx (2x29dBm) for TDD bands for FWA UE

# 4x26dBm FWA UE RF requirements

## UE Tx requirements

### Maximum output power for 4x26dBm FWA in TN

RAN#109 Plenary approved the following Objectives for 4x26dBm FWA device in TN bands [1].

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

  - Co-existence study for ACLR, specify ACLR requirements if needed

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 4x4 UL MIMO and TxD

  - Specify the band specific requirements for exemplary bands

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

The Power class 1 is already defined in TS36.102 [2] up to 31dBm as follow in table 6.2B.1-1 and PC1 is also defined in TS38.101-1 [3] for public safety device and non-smart phone device in several NR bands as shown in Table 6.2.1-1.

Observation 1: In TS36.102 and TS38.101-1, RAN4 already defined Power Class 1 maximum output power is 31dBm. If RAN4 define different power class up to 32dBm TN FWA device, then the other power class shall be considered.

Table 6.2B.1-1: UE Power Class [2]

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 255 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 254 |  |  |  |  | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  |  |  | 23 | +/-2 | 20 | +/-2 |
| 252 |  |  |  |  | 23 | +/-2 | 20 | +/-2 |
| 249 |  |  |  |  | 23 | +/-2 | 20 | +/-2 |

Table 6.2.1-1: UE Power Class [3]

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n2 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n3 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n5 |  |  |  |  | 26 | +2/-33 | 23 | ±2 |
| n7 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n8 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n12 |  |  |  |  |  |  | 23 | ±23 |
| n13 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n14 | 316,7 | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n18 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±23 |
| n24 |  |  |  |  |  |  | 23 | +2/-33 |
| n25 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n26 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n28 |  |  |  |  | 26 | +2/-33 | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n31 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n34 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n40 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n41 | 316 | +2/-3 | 295 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| n47 |  |  |  |  |  |  | 23 | ±2 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  | 23 | ±2 |
| n51 |  |  |  |  |  |  | 23 | ±2 |
| n53 |  |  |  |  |  |  | 23 | ±2 |
| n54 |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  | 23 | ±2 |
| n66 | 316 | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n68 |  |  |  |  |  |  | 23 | ±2 |
| n70 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n71 | 316 | +2/-3 |  |  | 26 | +2/-3 | 23 | +2/-2.5 |
| n72 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n74 |  |  |  |  |  |  | 23 | ±2 |
| n77 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n81 |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n84 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n85 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n86 |  |  |  |  |  |  | 23 | ±2 |
| n87 |  |  |  |  |  |  | 23 | ±2 |
| n88 |  |  |  |  |  |  | 23 | ±2 |
| n89 |  |  |  |  |  |  | 23 | ±2 |
| n91 |  |  |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n97 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n98 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n99 |  |  |  |  |  |  | 23 | +2/-33 |
| n100 | 316, 8 | +2/-3 |  |  |  |  | 23 | ±2 |
| n101 | 316, 8 | +2/-3 |  |  |  |  | 23 | ±2 |
| n104 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n105 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n106 |  |  |  |  |  |  | 23 | ±2 |
| n109 |  |  |  |  |  |  | 23 | ±23, 4 |
| n110 |  |  |  |  |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: Achieved via Tx diversity. NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. NOTE 8: PC1 in Band n100 and n101 is allowed only for FRMCS cab-radio’s, i.e. user equipments installed on trains with external antenna on top of the train roof at approximately 4m over ground. |  |  |  |  |  |  |  |  |

So, if RAN4 want to support 4x26dBm (up to 32dBm) FWA power class, the n RAN4 need to define new power class and it is not matched with legacy PC1 maximum output power.

Proposal 1: If RAN4 define different power class with 32dBm FWA device, then PC1bis or other designated power class would be considered to distinguish different maximum output power level between 31dBm and 32dBm.

### Maximum output power reduction for 4x26dBm FWA in TN

For the PC1 (up to 32 dBm) with 4Tx antenna, RAN4 need to define MPR requirements in suffix D for UL-MIMO UE and suffix G for Tx Diversity UE.

Table 6.2D.2-5: Maximum power reduction (MPR) for power class 1.5 with 4 Tx [3]

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 7.5 | ≤ 2.0 | ≤ 0.5 |
|  | QPSK | ≤ 8.0 | ≤ 2.5 | ≤ 0.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 3.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 4.0 | ≤ 3.5 |
|  | 256 QAM | ≤ 8.0 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 8.0 | ≤ 4.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 8.0 | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 5.0 | ≤ 5.0 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ 8.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Based on the Table 6.2D.2-4 format, RAN4 can derive the delta value on top of the existing MPR requirements for 4x26dBm FWA device.

Proposal 2: To derive MPR requirements for 4x26dBm (up to 32dBm) FWA device, RAN4 can define delta value on top of the existing MPR requirements in Table 6.2D.2-5.

### Additional Maximum output power reduction for 4x26dBm FWA in TN

For n41 and n77/n78 as example operating bands, RAN4 can define A-MPR requirements based on the n41 existing A-MPR requirements according to additional SE limits as follow

- n41:

  - When FWA UE received these NS_04 and NS_47 for up to 32dBm FWA device: Apply new A-MPR requirements based on the modified MPR behaviour approach for FWA (32dBm) device from rel-20.

  - Define delta A-MPR based on the existing Power class 1 A-MPR requirements in clause 6.2.3.2 in TS38.101-1.

- n77: No need to A-MPR requirements when NS_55 and NS_57 are received.

- n78: No need to A-MPR requirements

- n79: No need to A-MPR requirements

- n104: Need to wait additional emission limits in each country. This is dependent on regulatory requirements by RAN4 contribution if applicable.

Proposal 3: For A-MPR requirements for 4x26dBm (up to 32dBm) FWA device in n41, RAN4 can define delta value on top of the existing A-MPR requirements for PC1 in clause 6.2.3.2. For the other example NR bands, RAN4 do not need to define A-MPR requirements.

### Configured Tx power and other Tx RF requirements for 4x26dBm FWA

For the 4Tx antenna configuration, the FWA 4Tx/4Rx device could be follow the existing the ∆TRxSRS with usage in SRS-ResourceSet set considering with SRS-TxSwitch capability in clause 6.2.4.

Hence, RAN4 do not need to define additional configured Tx power for FWA 32dBm device due to SRS tx switching perspectives. However, RAN4 need further discuss how to treat the SAR regulatory limits for FWA device.

Proposal 4: For configured Tx power, RAN4 can further discuss how to treat the SAR regulatory limits for FWA device. Other Tx requirements can be reused the existing Tx RF requirements except ACLR requirements for FWA 32dBm device.

Proposal 5: For ACLR requirements for FWA 4x26dBm (up to 32dBm) output power device, RAN4 will decide the ACLR requirements based on SLS coexistence evaluation results to protect legacy systems.

## UE Rx requirements

In this section, we can propose the Rx requirements for 4x26dBm (up to 32dBm) FWA device.

### REFSENS for 4x26dBm FWA in TN

To derive REFSENS requirements, RAN4 can reuse the existing REFSENS requirements in Table 7.3.2-1b for 2Rx receivers and the additional ΔRIB,4R in Table 7.3.2-2 and ΔRIB,8R in Table 7.3.2-2a can be supported as optional feature.

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n297 | 15 | 5,10 | -97 + 10log10(NRB/25) | SDL |
|  | 30 | 10 | -94.1 + 10log10(NRB/24) |  |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 35, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40 | -97.5 + 10log10(NRB/11) |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -97.5 + 10log10(NRB/11) |  |
| n411, n901 | 15 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | -94.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11) |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 50 | -99 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11) |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 |  |
|  | 60 | 10 | -97.5 |  |
| n54 | 15 | 5 | -100 | TDD |
| n677 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) |  |
| n757 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10,15,20,25,30,40,50 | -97.5 + 10log10(NRB/11) |  |
| n767 | 15 | 5 | -95.3 + 10log10(NRB/52) | SDL |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n791 | 15 | 10, 20, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n91 | 15 | 5 | -100 | FDD |
| n92 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
| n93 | 15 | 5 | -100 | FDD |
| n94 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24) |  |
| n1041,10 | 15 | 20, 30, 40, 50 | -90.7 + 10log10(NRB/106) | TDD |
|  | 30 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -90.8 + 10log10(NRB/51) |  |
|  | 60 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -91.1 + 10log10(NRB/24) |  |
| n10911 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) |  |

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n13, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n1, n3, n7, n25 | -4.5 |
| n34, n39, n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor. |  |

Proposal 6: For FWA REFSENS requirements for FWA 4x26dBm (up to 32dBm) device, RAN4 can reuse the existing 2Rx REFSENS requirements. RAN4 also will support 4Rx and 8Rx FWA REFSENS requirements with the additional delta values in Table 7.3.2-2 and Table 7.3.2-2a.

### ACS and other Rx requirements for 4x26dBm FWA in TN

For ACS requirements for FWA 4x26dBm (up to 32dBm) output power device, RAN4 will decide the FWA ACS requirements based on SLS coexistence evaluation results to protect legacy systems.

However, other RX requirements can be reused for FWA 32 dBm device.

Proposal 7: For ACS requirements for FWA 4x26dBm (up to 32dBm) device, RAN4 will decide the ACS requirements based on SLS coexistence evaluation results to protect legacy systems. Other Rx requirements can be reused for FWA 32dBm device.

# Summary

In this paper, we provide our views on the FWA 4x26dBm (up to 32dBm) device Tx/Rx RF requirements . Based on the above analysis, we propose as following proposals in paper.

Proposal 1: If RAN4 define different power class with 32dBm FWA device, then PC1bis or other designated power class would be considered to distinguish different maximum output power level between 31dBm and 32dBm.

Proposal 2: To derive MPR requirements for 4x26dBm (up to 32dBm) FWA device, RAN4 can define delta value on top of the existing MPR requirements in Table 6.2D.2-5.

Proposal 3: For A-MPR requirements for 4x26dBm (up to 32dBm) FWA device in n41, RAN4 can define delta value on top of the existing A-MPR requirements for PC1 in clause 6.2.3.2. For the other example NR bands, RAN4 do not need to define A-MPR requirements.

Proposal 4: For configured Tx power, RAN4 can further discuss how to treat the SAR regulatory limits for FWA device. Other Tx requirements can be reused the existing Tx RF requirements except ACLR requirements for FWA 32dBm device.

Proposal 5: For ACLR requirements for FWA 4x26dBm (up to 32dBm) output power device, RAN4 will decide the ACLR requirements based on SLS coexistence evaluation results to protect legacy systems.

Proposal 6: For FWA REFSENS requirements for FWA 4x26dBm (up to 32dBm) device, RAN4 can reuse the existing 2Rx REFSENS requirements. RAN4 also will support 4Rx and 8Rx FWA REFSENS requirements with the additional delta values in Table 7.3.2-2 and Table 7.3.2-2a.

Proposal 7: For ACS requirements for FWA 4x26dBm (up to 32dBm) device, RAN4 will decide the ACS requirements based on SLS coexistence evaluation results to protect legacy systems. Other Rx requirements can be reused for FWA 32dBm device.

# References

- [1] RP-252952, “New WID: UE RF enhancements for NR FR1, Phase 5,” Moderator (vivo)

- [2] TS36.102

- [3] TS38.101-1