3GPP TSG RAN WG4 Meeting #116-bis		R4-2513783

Prague, Czech Republic, October 13th-17th, 2025

Agenda item:	6.9.2.1

Source:	Telecom Italia S.p.A., Vodafone, Deutsch Telekom, Orange

Title:	Proposals for Rel-19 TRP TRS requirements working procedure

WI/SI:	TRP_TRS_MIMO_OTA_Ph3

Release:	Rel-19

Document for:	Approval

# 1	Introduction

During RAN4#116 meeting, discussion on Rel-19 TRP TRS requirements related to the WI [1] was held which outcomes and agreements have been captured in the WF [2]. This contribution proposes some changes to the working procedure for TRP TRS Performance Test Campaign updated according to the WF Issue 4-2-4 [2] to address some of the remaining open points.

# 2	1Tx FR1 TRP TRS requirements

Proposal 1: Adopt the following changes to the working procedure to define the TRP TRS requirements:

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: n3, n5, n7, and n8 [2Tx at bands n41]

  - Use scenarios:

    - For 1Tx, consider both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR;

    - For 2Tx, only consider Browsing mode (Hand phantom only) requirements in Rel-19;

  - Hand Phantom: Wide Grip Hand and PDA hand

  - Operation mode: SA with 1Tx as phase 1. SA with 2Tx (non-coherent UL-MIMO) as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: UE (width >56mm and ≤92mm)

  - DUT capability: support for Bands those listed in the latest WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: 2022 2nd-half-2025 first-half

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. PC3 for all 1Tx bands. PC2 for 2Tx band n41.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

    - Same UE model supporting different sets of bands can be measured. For this case, the UE model should be marked as different model e.g., model A-1, model A-2 (guidance on how to manage this case are provided in the UE information sheet in [TBD]

  - Logistical aspects for devices provisioning to the labs are managed by the interested parties

- Test results submitting:

  - UE information disclosure: the following rules should be considered

    - UE model information based on template R4-2512613 can be shared to MCC. RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure.

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - laboratories use the same template to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

      - Information of the devices that are going to be measured shall be shared with the RAN4 Secretary by the end of the RAN4#116-bis meeting using the template defined in R4-2512613

      - The RAN4 Secretary includes the received device information in the summary table as per the template R4-2512613 and share to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds in a timely manner and take the proper actions if these are not met

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

      - The complete device information sheet shall be available during the post-meeting week of RAN4#116-bis meeting (i.e., not later than the TDoc submission deadline) to evaluate the accomplishment of the defined thresholds. and act accordingly in case these are not.  Decision on how to proceed will be taken through email discussion.

    - Statistics of UE information can be shared to RAN4 reflector after TBD meeting by RAN4 secretary after post-processing

    - Fulfilment of the following thresholds (for each band and power class) related to the devices pool for Rel-19:

      - Minimum number of devices for each band, each device size, each power class: 60

      - Minimum number of device models: 10 less than above

      - Minimum number of devices' vendors: 6

      - Percentage of devices from second-half 2022 to first-half 2025: 100%

      - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

      - Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: between 80% and 85%

        - If for any band with the number of measured data points < 60 this threshold is not met, RAN4 can request the test labs to measure additional devices to shift the ratio closer to the target value. The number of additional devices is determined based on the delta between the current percentage and the threshold.

  - Laboratories use the same worksheet template in R4-2512686 to submit the measurement results for Rel-19 3GPP TRP TRS performance data pool. Separate worksheets will be used for submission of data from the 1Tx and 2Tx measurement campaigns.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The minimum number of submitted devices from each lab is 3

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

  - All the measurement results shall be submitted by RAN4#117 meeting (i.e., not later than the TDoc submission deadline) to provide enough time to the companies to analyse the provided results and let RAN4 have an initial discussion on the requirements’ values during the meeting.

- If the time plan defined above will not be met, the WI shall be extended of at least one meeting period.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30]

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19.

  - Method of limits derivation: per-band Data driven approach, working procedure in Rel-18 on how to specify the requirement is baseline.

  - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - Only PC3 requirements of selected bands for 1Tx.

  - No differentiation of Size 1 and Size 2 TRP/TRS requirements

  - The data pool will be refined based on the following criteria:

    - Total number of devices = 60: remove 1 best and 1 worst-performing data

    - Total number of devices between 61 to 65: remove 2 best and 2 worst-performing data

    - Total number of devices = 66 to 70: remove 3 best and 3 worst-performing data

    - Total number of devices = 71 to 75: remove 4 best and 4 worst-performing data

    - Total number of devices = 76 to 80: remove 5 best and 5 worst-performing data

    - Total number of devices > 81: remove 6 best and 6 worst-performing data

- Test lab procedures for 1Tx

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - Only PC3 testing is needed.

  - For new labs of 1Tx requirements work, those not declared aligned in Rel-17 and Rel-18 also need to complete 1Tx lab alignment on at least n78 (high band) and n28 (low band).

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

- Test lab procedures for 2Tx

  - Only non-coherent device.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

  - The baseline TRP measurement should be performed with all the Tx antenna ON and consistently operating at maximum power level condition. Fixed TPMI=2 is the configuration to be used for non-coherent UE. For any additional test configuration refer to TR 38.870 clause 7.5.3.1.

  - FFS other details

# 3	Conclusions

In the following, the proposal of the contribution is summarized:

Proposal 1: Adopt the following changes to the working procedure to define the TRP TRS requirements:

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: n3, n5, n7, and n8 [2Tx at bands n41]

  - Use scenarios:

    - For 1Tx, consider both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR;

    - For 2Tx, only consider Browsing mode (Hand phantom only) requirements in Rel-19;

  - Hand Phantom: Wide Grip Hand and PDA hand

  - Operation mode: SA with 1Tx as phase 1. SA with 2Tx (non-coherent UL-MIMO) as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: UE (width >56mm and ≤92mm)

  - DUT capability: support for Bands those listed in the latest WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: 2022 2nd-half-2025 first-half

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. PC3 for all 1Tx bands. PC2 for 2Tx band n41.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

    - Same UE model supporting different sets of bands can be measured. For this case, the UE model should be marked as different model e.g., model A-1, model A-2 (guidance on how to manage this case are provided in the UE information sheet in [TBD]

  - Logistical aspects for devices provisioning to the labs are managed by the interested parties

- Test results submitting:

  - UE information disclosure: the following rules should be considered

    - UE model information based on template R4-2512613 can be shared to MCC. RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure.

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - laboratories use the same template to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

      - Information of the devices that are going to be measured shall be shared with the RAN4 Secretary by the end of the RAN4#116-bis meeting using the template defined in R4-2512613

      - The RAN4 Secretary includes the received device information in the summary table as per the template R4-2512613 and share to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds in a timely manner and take the proper actions if these are not met

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

      - The complete device information sheet shall be available during the post-meeting week of RAN4#116-bis meeting (i.e., not later than the TDoc submission deadline) to evaluate the accomplishment of the defined thresholds. and act accordingly in case these are not.  Decision on how to proceed will be taken through email discussion.

    - Statistics of UE information can be shared to RAN4 reflector after TBD meeting by RAN4 secretary after post-processing

    - Fulfilment of the following thresholds (for each band and power class) related to the devices pool for Rel-19:

      - Minimum number of devices for each band, each device size, each power class: 60

      - Minimum number of device models: 10 less than above

      - Minimum number of devices' vendors: 6

      - Percentage of devices from second-half 2022 to first-half 2025: 100%

      - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

      - Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: between 80% and 85%

        - If for any band with the number of measured data points < 60 this threshold is not met, RAN4 can request the test labs to measure additional devices to shift the ratio closer to the target value. The number of additional devices is determined based on the delta between the current percentage and the threshold.

  - Laboratories use the same worksheet template in R4-2512686 to submit the measurement results for Rel-19 3GPP TRP TRS performance data pool. Separate worksheets will be used for submission of data from the 1Tx and 2Tx measurement campaigns.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The minimum number of submitted devices from each lab is 3

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

  - All the measurement results shall be submitted by RAN4#117 meeting (i.e., not later than the TDoc submission deadline) to provide enough time to the companies to analyse the provided results and let RAN4 have an initial discussion on the requirements’ values during the meeting.

- If the time plan defined above will not be met, the WI shall be extended of at least one meeting period.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30]

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19.

  - Method of limits derivation: per-band Data driven approach, working procedure in Rel-18 on how to specify the requirement is baseline.

  - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - Only PC3 requirements of selected bands for 1Tx.

  - No differentiation of Size 1 and Size 2 TRP/TRS requirements

  - The data pool will be refined based on the following criteria:

    - Total number of devices = 60: remove 1 best and 1 worst-performing data

    - Total number of devices between 61 to 65: remove 2 best and 2 worst-performing data

    - Total number of devices = 66 to 70: remove 3 best and 3 worst-performing data

    - Total number of devices = 71 to 75: remove 4 best and 4 worst-performing data

    - Total number of devices = 76 to 80: remove 5 best and 5 worst-performing data

    - Total number of devices > 81: remove 6 best and 6 worst-performing data

- Test lab procedures for 1Tx

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - Only PC3 testing is needed.

  - For new labs of 1Tx requirements work, those not declared aligned in Rel-17 and Rel-18 also need to complete 1Tx lab alignment on at least n78 (high band) and n28 (low band).

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

- Test lab procedures for 2Tx

  - Only non-coherent device.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

# 4	References

- RP-252187, “Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3”, vivo, CAICT, September 2025

- R4-2512693, “Way Forward for [116][337] TRP_TRS_MIMO_OTA_Ph3”, vivo, August 2025