3GPP TSG-RAN WG4 Meeting #116bis                                                               R4-2513784

Prague, CZ, 13th-17th, Oct, 2025

Source:	Samsung

Title:	On general aspects for Rel-20 AI/ML for NR air interface

Agenda item:			7.9.2

Document for:	Discussion

# 1. Introduction

Based on Rel-18 study on AI/ML Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface and the follow-up study on the CSI compression with two-sided model has been conducted in Rel-19. Accordingly, Rel-20 new WID was originally approved in RAN#108, which has been further updated in RAN#109 [1]. Among objectives of this work item, the following are RAN4 relevant work scope on inter-vendor training collaboration for two-sided AI/ML models, and RAN4 requirements of interoperability and RRM:

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

As provided in the WID, two parts of work need to be done by RAN4. Firstly, RAN4 will work on test/reference models which RAN1 will use to confirm inter-vendor training collaboration for two-sides AI/ML models, related to Direction C and Direction A. It should be noted that there is a check-point in RAN#110, before which there will be only one RAN4 meeting except this meeting RAN4#116bis. Secondly, RAN4 will work on standardizing the interoperability and RRM requirement. The throughput gains over non-AI method will be verified and the performance requirements of encoders will be specified.

Besides, following topics for two-sides models were summarized in last RAN4 meeting WF [2], which could provide the early guidance for WI discussion in Rel-20.

- Discuss and align on the dataset assumptions and dataset - Discuss and align on assumptions for the test decoder and reference encoder structures - Also may need to consider how many test decoders and encoders - Discuss and align on the scalability alternatives - Discuss and agree on expectations on whether the decoder/encoder for RAN4 are the same as for option C - Discuss whether to decide this before or after agreeing the model - Develop a means to capture model structures and model parameters in 3GPP - Discuss and agree the test metric for the performance requirements. - Discuss and agree channel model for the requirements. - For the dataset, discuss the following: - Eigenvectors or raw channel (or both) ? - What it is important to capture within the dataset - metrics for checking alignment - For example,. PSE - Consider examples from SI datasets that did not align

In this contribution to the first RAN4 meeting in Rel-20 AI/ML work item, we aim to discuss the general aspects and workplan for CSI feedback enhancement.

# 2. Discussion

## 2.1 RAN4 work plan for RAN#110 check-point

As aforementioned, there are two RAN4 meetings left before the check-point for inter-vendor training collaboration Direction C and Direction A in RAN#110. What should be fed back needs to be discussed. For the expected RAN4 checking, the complete RAN4 evaluation may involve aligning model structure, scalability method, etc.

Based on the experiences in Rel-19 and the new expected work on aligning scalable models, the following complete procedure could be required to check scalability outcome from RAN1.

- Generate mixed dataset

  - Determine the link-level simulation parameters for generating training/testing data, including different values on scalable parameters.

  - Companies generate and share their own datasets in a common format.

  - Select and mix the datasets according to specific metric to generate the mixed dataset

- Align the model structure, model hyperparameter, training hyperparameters and scalability methods.

  - Determine how many decoders should be aligned. For instance, different decoders with different backbones are used to achieve different throughput gains over non-AI legacy method.

  - Companies propose model structures along with the scalability methods and report the SGCS performances.

  - Check on performance alignment. Share models if seeing bad performance alignment.

  - Select one or more models to further verify the throughput gains.

- Confirm the RAN1 scalability outcome and confirm the scalability-relevant design is compatible with Option 3 test decoder

Observation 1: For RAN#110 check-point, the complete RAN4 procedure required to check scalability outcome from RAN1 may include

- Generate mixed dataset

  - Determine the link-level simulation parameters for generating training/testing data, including different values on scalable parameters.

  - Companies generate and share their own datasets in a common format.

  - Select and mix the datasets according to specific metric to generate the mixed dataset

- Align the model structure, model hyperparameter, training hyperparameters and scalability methods.

  - Determine how many decoders should be aligned. For instance, different decoders with different backbones are used to achieve different throughput gains over non-AI legacy method.

  - Companies propose model structures along with the scalability methods and report the SGCS performances.

  - Check on performance alignment. Share models if seeing bad performance alignment.

  - Select one or more models to further verify the throughput gains.

- Confirm the RAN1 scalability outcome and confirm the scalability-relevant design is compatible with Option 3 test decoder

Obviously, the remaining two meetings are not enough to cover the complete procedure, and RAN4 needs to set up the reasonable scope during two meetings to confirm RAN1 scalability outcome. If the above scope is determined to be too large in RAN4, RAN4 can consider reasonable scope and plan with reasonable cases to be verified only.

Proposal 1: Reasonable RAN4 plan and scope shall be planned for RAN#110 check-point.

## 2.2 Model structure scalability

In Rel-19, RAN4 studied the feasibility of model interoperability without considering scalable input and output. However, model scalability is essential for model specification due to the potential various input sizes and payload sizes in practice. RAN1 has study the model scalability on three scalable dimensions based on the agreed transformer-based model.

RAN1 #120 agreements Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters - Notes - Other Alternatives are not precluded. - Different Alternatives may be used in combination. - Same/similar approach is applied at the decoder side. - Evaluations to consider: - Case 1 (scalable structure): Scalable model structure described above - Using model structure as indicated in above diagram with fixed hyperparameters, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth} assuming a common model structure. - To report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports) - Case 2 (dedicated structure): Using model structure as indicated in above diagram with different hyperparameters, where the input and the output related hyperparameters are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations. - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths. - For each scalable model structure choice, to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}. Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

Observation 2: Three scalability aspects, the number of Tx ports, CSI feedback payload size and subband number are considered variables to verify the model scalability in RAN1.

There are too many potential combinations of the variable three aspects. To reduce the workload of simulation, RAN4 need to select a subset of all the alternatives, including typical Tx port number and antenna layout, typical payload sizes and typical subband number.

Proposal 2: RAN4 should determine a set of combinations with typical values of Tx port number and antenna layout, CSI feedback payload size and subband number to align the scalability solutions.

## 2.3 Mixed dataset generation

In Rel-19, following system-level simulation parameters are used to generate training/testing dataset [3].

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

In Rel-20, RAN4 not only works on aligning scalable models, but also performs evaluation on throughput gains with AI/ML models in link-level simulation. It is suggested to use single set of link-level simulation parameters to both generate training/testing dataset and evaluate the throughput gains.

Proposal 3: A single set of link-level simulation parameters are suggested to both generate training/testing dataset and evaluate throughput gains from the AI/ML method.

Considering the new requirement of aligning model scalability, different alternatives of Tx port number and antenna layout, CSI feedback payload size and subband number should be included into the simulation parameters. To accelerate the alignment, RAN4 should select the typical alternatives of the three scalable dimensions.

Proposal 4: Typical values of Tx port number and antenna layout, CSI feedback payload size and subband number need to be discussed and included in the simulation parameters.

## 2.4 Model structure backbone

In Rel-19, RAN4 takes CNN as the backbone of models, and verifies the feasibility of a common CNN decoder for future-proof compatible to encoders with different backbones and with different complexities, wherein Transformer-based encoder is included and achieves the best SGCS performance. Meanwhile in Rel-19, RAN1 selects Transformer as the backbone of models and evaluates the solutions of model scalabilities.

Aiming at reusing the study outcome from RAN1, and for the purpose of obtaining acceptable throughput gains in later link-level simulation, Transformer is suggested as one backbone. Legacy CNN and more other backbone can be considered.

Proposal 5: Transformer is taken as the reference backbone of encoder and decoder to reuse the study outcome on model scalability and achieve acceptable intermediate performance (SGCS) and final performance (throughput). Other backbones are not precluded.

Following figure and table illustrate the agreed RAN1 Transformer backbone [4]. RAN4 can take it as the starting point and discuss the model hyperparameters.

Figure 1. RAN1 Transformer backbone Table 2. RAN1 Transformer-based encoder description Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer); Number of tokens Feature dimension of each token Output dimension dmodel per token. Positional encoding; Number of tokens positions Dimension of positional encoding for each token position: dmodel. Transformer blocks; Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice. Output linear layer; Reshape the matrix to a vector Output dimension zdim. Quantization; Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment. Decoder description: The decoder has a mirroring design as the encoder.

Proposal 6: RAN4 takes RAN1 Transformer encoder and decoder as the starting point and determine the model hyperparameters.

It is observed that both scalar quantization and vector quantization are described in the above table. RAN4 will specify only quantized encoder and decoder in Rel-20. In Rel-19, scalar quantization is used. Whether supporting vector quantization is suggested to be discussed.

Proposal 7: Only quantized encoder and decoder should be specified. RAN4 could discuss whether or not to support vector quantization in specifying reference/test models.

2.5 Training method

In Rel-19, RAN4 verifies the feasibility of compatibility between a common CNN decoder and several encoders with different backbones and with different complexities. In common, the smaller the complexity of the encoder is, the worse the SGCS performance is. To guarantee the SGCS performance of lower complexity encoder, the common decoder needs to be trained by N(encoders)-to-1(decoder) method.

For the convenience of evaluating throughput gains, several encoders with different complexities are suggested to be aligned. Finally, based on the tradeoff between throughput gains and complexities, at least one encoder is specified as the reference encoder. N-to-1 joint training method is proposed to be reused in Rel-20 RAN4 work.

Proposal 8: For one decoder of specific backbone, several encoders with different complexities can be considered with N-to-1 joint training method being used.

Proposal 9: Base on the tradeoff between throughput gains and model complexities, at least one model structure shall be specified as the reference model structure to derive the performance requirement.

## 2.6 Test metric for the performance requirements

The overall work in Rel-20 could be divided into two parts. The first one is to align and specify the test decoder and reference encoder. The second one is to evaluate the throughput gains using the specified models. For the first one, intermediate metric is set to evaluate the absolute accuracy of reconstructed precoding matrixes. Same as in Rel-19, SGCS is proposed. For the second one, final metric is set to evaluate the absolute throughput and/or throughput gains over the legacy non-AI CSI feedback method if the gain is testable.

Proposal 10: For results aligning for two-sided model training and inference evaluation, absolute SGCS is proposed as the performance metric.

Proposal 11: For verify the throughput gains and specify the performance requirements of the encoder, RAN4 can take the throughput related metrics as performance metric, e.g., absolute throughput or throughput gain over the non-AI CSI feedback or others depending on testability.

When comparing with non-AI CSI feedback, eType II codebook should be treated as the benchmark. And which parameter combination should be used needs further discussion. The overhead of non-AI method should be comparable with the one of AI/ML models. For the best, the two overheads of the two methods are equivalent.

Proposal 12: When RAN4 discusses the achievable performance gain of AI/ML-based CSI feedback enhancement, the eType II-based codebook can be taken as the baseline. The overheads of eType II and AI/ML models should be comparable or equivalent.

# 3. Conclusion

In this contribution, we provided our viewpoints on the general aspects and workplan of two-sided models, accordingly the following observations and proposals are obtained:

Observation 1: For RAN#110 check-point, the complete RAN4 procedure required to check scalability outcome from RAN1 may include

- Generate mixed dataset

  - Determine the link-level simulation parameters for generating training/testing data, including different values on scalable parameters.

  - Companies generate and share their own datasets in a common format.

  - Select and mix the datasets according to specific metric to generate the mixed dataset

- Align the model structure, model hyperparameter, training hyperparameters and scalability methods.

  - Determine how many decoders should be aligned. For instance, different decoders with different backbones are used to achieve different throughput gains over non-AI legacy method.

  - Companies propose model structures along with the scalability methods and report the SGCS performances.

  - Check on performance alignment. Share models if seeing bad performance alignment.

  - Select one or more models to further verify the throughput gains.

- Confirm the RAN1 scalability outcome and confirm the scalability-relevant design is compatible with Option 3 test decoder

Proposal 1: Reasonable RAN4 plan and scope shall be planned for RAN#110 check-point.

Observation 2: Three scalability aspects, the number of  Tx ports, CSI feedback payload size and subband number are considered variables to verify the model scalability in RAN1.

Proposal 2: RAN4 should determine a set of combinations with typical values of Tx port number and antenna layout, CSI feedback payload size and subband number to align the scalability solutions.

Proposal 3: A single set of link-level simulation parameters are suggested to both generate training/testing dataset and evaluate throughput gains from the AI/ML method.

Proposal 4: Typical values of Tx port number and antenna layout, CSI feedback payload size and subband number need to be discussed and included in the simulation parameters.

Proposal 5: Transformer is taken as the reference backbone of encoder and decoder to reuse the study outcome on model scalability and achieve acceptable intermediate performance (SGCS) and final performance (throughput). Other backbones are not precluded.

Proposal 6: RAN4 takes RAN1 Transformer encoder and decoder as the starting point and determine the model hyperparameters.

Proposal 7: Only quantized encoder and decoder should be specified. RAN4 could discuss whether or not to support vector quantization in specifying reference/test models.

Proposal 8: For one decoder of specific backbone, several encoders with different complexities can be considered with N-to-1 joint training method being used.

Proposal 9: Base on the tradeoff between throughput gains and model complexities, at least one model structure shall be specified as the reference model structure to derive the performance requirement.

Proposal 10: For results aligning for two-sided model training and inference evaluation, absolute SGCS is proposed as the performance metric.

Proposal 11: For verify the throughput gains and specify the performance requirements of the encoder, RAN4 can take the throughput related metrics as performance metric, e.g., absolute throughput or throughput gain over the non-AI CSI feedback or others depending on testability.

Proposal 12: When RAN4 discusses the achievable performance gain of AI/ML-based CSI feedback enhancement, the eType II-based codebook can be taken as the baseline. The overheads of eType II and AI/ML models should be comparable or equivalent.

# 4. Reference

[1] RP-252445 (revised from RP-251870), “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, RAN#109.

[2] R4-2511849, “WF on [116][132] NR_AIML_air_part2”, Ericsson, RAN4#116.

[3] R4-2401570, “WF on AI/ML”.

[4] R1-2501474, “Final summary of Additional study on AI/ML for NR air interface: CSI compression”.