3GPP TSG-RAN WG4 Meeting #116bis	R4-2513789

# Prague, Crech, 13th - 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0737 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction CR to TS38.104 for SBFD-capable BS RF requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CR to TS 38.104 for maintenance purpose to correct typos and other correction changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | (1) The absolute basic limit for band is corrected for n104 for in-channel adjacent subband leakage for SBFD. (2) Other correct typos and other correction changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BS RF requirements for SBFD-capable BS contain error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3, 5, 12 (newly added), B.8 (newly added), C.8 (newly added), D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.141-1/2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change 1 >>

### 12.1.2	General SBFD-capable BS Operating Requirements

An SBFD-capable BS shall only operate in SBFD subband configurations (i.e., DL/UL subbands) that have been declared in accordance with the provisions in this specification.

The RF requirements for SBFD-capable BS have been derived based on the simulation assumption that only SBFD-capable BSs are deployed with the identical SBFD configuration under each BS class in the same geographical area.

## << End of change 1 >>

## << Start of change 2 >>

### 12.1.3	SBFD-capable BS Types and Configuration Constraints

#### 12.1.3.1 General

SBFD-capable BSs are categorized as either configuration-limited or configuration-flexible according to the classification defined in Table 12.1.3-1. The classification is based on BS class and rated output power specified in clause 12.5.2.

Table 12.1.3.1-1: SBFD-capable BS classification

| BS Class | Rated Output Power (Prated,c,sys) | SBFD BS type |
| --- | --- | --- |
| FR1 WA BS | -- | Configuration-limited |
| FR1 MR BS | ≥ 36 dBm | Configuration-limited |
| FR1 MR BS | < 36 dBm | Configuration-flexible |
| FR1 LA BS | -- | Configuration-flexible |
| FR2 WA BS | -- | Configuration-flexible |
| FR2 LA BS | -- | Configuration-flexible |
| NOTE 1: A power level threshold of Prated,c,sys = 36 dBm is used to distinguish between configuration-limited and configuration-flexible SBFD-capable BSs in the FR1 MR BS class. |  |  |

#### 12.1.3.2 Configuration-limited SBFD-capable BS

For configuration-limited SBFD-capable BSs, the following requirements apply:

For each channel bandwidth supported for SBFD operation, a maximum of three SBFD subband configurations may be declared. The three configurations shall cover all supported SBFD patterns, including both DUD and DU/UD cases. A single UL subband size used in both DU and UD configurations shall be counted as two configurations. For example, a UD 20-80 MHz and a DU 80-20 MHz configuration shall be counted as two SBFD subband configurations.

For each supported channel bandwidth and for each supported UL subband size: Only one DL subband size may be declared and tested for the DU/UD pattern. Only one DL subband size may be declared and tested for the DUD pattern.

A configuration-limited SBFD-capable BS shall not operate in any declared SBFD configuration (i.e., DL/UL subband) unless it has been tested.

#### 12.1.3.3 Configuration-flexible SBFD-capable BS

For configuration-flexible SBFD-capable BSs, the following requirements apply:

There is no restriction on the number of UL subband sizes that may be declared.

There is no restriction on the number of DL subband sizes that may be declared.

For each declared channel bandwidth and for each declared SBFD pattern (i.e., DUD or DU/UD), the following configurations shall be tested:

- The configuration with the narrowest UL subband size, and the corresponding worst-case SBFD DL transmission bandwidth configuration:

  - For DU/UD: the worst case configuration is the one with the largest NRB,SBFD,DL.

  - For DUD: the worst case configuration is the one with the largest NRB,SBFD,DL,1 or with the largest NRB,SBFD,DL,2.

- The configuration with the widest UL subband size, and the corresponding worst-case SBFD DL transmission bandwidth configuration:

  - For DU/UD: the worst case configuration is the one with the largest NRB,SBFD,DL.

  - For DUD: the worst case configuration is the one with the largest NRB,SBFD,DL,1 or with the largest NRB,SBFD,DL,2.

## << End of change 2 >>

## << Start of change 3 >>

### 12.2.8	Transient period between SBFD and non-SBFD operation

#### 12.2.8.1	General

Transmitter transient period between SBFD and non-SBFD operation requirements apply only to SBFD-capable BS.

The transmitter transient period between SBFD and non-SBFD operation is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period between SBFD and non-SBFD operation is illustrated in figure 12.2.8.1-1.

Transmitter output power frequency regions for transition

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 12.2.8.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period between SBFD and non-SBFD operation

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different cases as illustrated in table 12.2.8.1-1:

Table 12.2.8.1-1: Transmitter transient period between SBFD and non-SBFD operation

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| SBFD symbol to DL symbol | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| SBFD symbol to UL symbol | SBFD DL subband | BS transmitter ON-to-OFF |
| DL symbol to SBFD symbol | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| UL symbol to SBFD symbol | SBFD DL subband | BS transmitter OFF-to-ON |

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

#### 12.2.8.2	Minimum requirement for BS type 1-H

For BS type 1-H, the transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.2.8.2-1.

Table 12.2.8.2-1: Minimum requirement for the transmitter transient period between SBFD and non-SBFD operation for BS type 1-H

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

## << End of change 3 >>

## << Start of change 4 >>

### 12.2.9	In-channel adjacent subband leakage for SBFD

#### 12.2.9.1	General

In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of SBFD UL subband within NR carrier.

The requirement shall apply during the transmitter ON period.

#### 12.2.9.2	Limits and Basic limits

For SBFD operation in the unpaired spectrum, the ICASL shall be less than or equal to the value specified in table 12.2.9.2-1.

Table 12.2.9.2-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit for bands other than band n104 | ICASL absolute basic limit for band n104 |
| --- | --- | --- |
| Category A Wide Area BS | -13 dBm/MHz | -10 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz | -12 dBm/MHz |
| Medium Range BS | -25 dBm/MHz | -22 dBm/MHz |
| Local Area BS | -32 dBm/MHz | -29 dBm/MHz |

Table 12.2.9.2-2: Filter parameters for the UL subband

| RAT | Filter on the assigned UL subband and corresponding filter bandwidth |
| --- | --- |
| NR | Square (the declared bandwidth of UL subband) |

#### 12.2.9.3	Minimum requirement for SBFD BS type 1-H

The ICASL requirements are that for each TAB connector TX min cell group and each applicable basic limit in clause 12.2.9.2, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell).

NOTE:	Conformance to the in-channel adjacent subband leakage requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the in-channel adjacent subband leakage power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The  in-channel adjacent subband leakage power at each TAB connector shall be less than or equal to the limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

## << End of change 4 >>

## << Start of change 5 >>

### 12.3.4	In-band selectivity and blocking for SBFD

#### 12.3.4.1	Adjacent Channel Selectivity (ACS) for SBFD

##### 12.3.4.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver’s ability to receive a wanted signal at its assigned channel frequency at the TAB connector for BS type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

NOTE: SBFD UL subband is not configured adjacent to the Radio Bandwidth edge where cross link interference from adjacent channel may appear.

##### 12.3.4.1.2	Minimum requirement for SBFD capable BS type 1-H

For SBFD operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-H TAB connector using the parameters in table 12.3.4.1.2-1 for ACS. The reference measurement channel for the wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

Table 12.3.4.1.2-1: Base station ACS requirement for SBFD

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -35 | ±10 | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |
| NOTE 1: PREFSENS for SBFD UL subband bandwidth depends on the BS channel bandwidth as specified in clause 12.3.2. NOTE 2: For lowest/highest carrier which is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. NOTE 3: For the carrier which is adjacent to sub-block gap and configured in DU/UD SBFD operation, the interfering signal only applies when DL subband is adjacent to sub-block edge inside a sub-block gap. |  |  |  |  |

#### 12.3.4.2	In-band blocking for SBFD

##### 12.3.4.2.1	General

The in-band blocking characteristics is a measure of the receiver's ability to receive a wanted signal at its assigned channel at the TAB connector for BS type 1-H in the presence of an unwanted interferer, which is an NR signal for general blocking.

NOTE 1: For FR1 WA BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz with 288 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements.

NOTE 2: For FR1 MR BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz outdoor deployment with 167 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements or in the context of indoor.

NOTE 3: SBFD UL subband is not configured adjacent to the Radio Bandwidth edge where cross link interference from adjacent channel may appear.

##### 12.3.4.2.2	Minimum requirement for SBFD capable BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-H TAB connector using the parameters in table 12.3.4.2.2-1 for general blocking for SBFD. The reference measurement channel for the wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

Minimum conducted requirement is defined at the TAB connector for BS type 1-H.

Table 12.3.4.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area BS: -25 Medium Range BS: -32 Local Area BS: -35 | ±30 | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |
| NOTE 1: PREFSENS for SBFD UL subband bandwidth depends on the BS channel bandwidth as specified in clause 12.3.2. NOTE 2: For lowest/highest carrier which is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. NOTE 3: For the carrier which is adjacent to sub-block gap and configured in DU/UD SBFD operation, the interfering signal only applies when DL subband is adjacent to sub-block edge inside a sub-block gap. |  |  |  |  |

## << End of change 5 >>

## << Start of change 6 >>

### 12.3.5	Out-of-band blocking for SBFD capable BS

#### 12.3.5.1 General

The general requirement of out-of-band blocking for SBFD capable BS is the same as general requirement specified in clause 7.5.1.

#### 12.3.5.2 Minimum requirement for SBFD capable BS type 1-H

The minimum requirements are the same as those for the BS type 1-H specified in clause 7.5.2, and the PREFSENS depends on the SBFD UL subband as specified in clause 12.3.2.

#### 12.3.5.3 Co-location minimum requirements for SBFD capable BS type 1-H

The co-location minimum requirements are the same as those for the BS type 1-H specified in clause 7.5.3, and the PREFSENS depends on the SBFD UL subband as specified in clause 12.3.2.

## << End of change 6 >>

## << Start of change 7 >>

### 12.5.10	OTA transient period between SBFD and non-SBFD

#### 12.5.10.1	General

The transmitter transient period between SBFD and non-SBFD operation is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period between SBFD and non-SBFD operation is illustrated in figure 12.5.10.1-1.

Transmitter output power frequency regions for transition

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 12.5.10.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period between SBFD and non-SBFD operation

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different cases as illustrated in table 12.5.10.1-1:

Table 12.5.10.1-1: Transmitter transient period between SBFD and non-SBFD operation

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| SBFD symbol to DL symbol | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| SBFD symbol to UL symbol | SBFD DL subband | BS transmitter ON-to-OFF |
| DL symbol to SBFD symbol | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| UL symbol to SBFD symbol | SBFD DL subband | BS transmitter OFF-to-ON |

This requirement shall be applied at each RIB supporting transmission in the operating band.

#### 12.5.10.2	Minimum requirement for BS type 1-O

For BS type 1-O, the OTA transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.5.10.2-1.

Table 12.5.10.2-1: Minimum requirement for the OTA transmitter transient period between SBFD and non-SBFD operation for BS type 1-O

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

#### 12.5.10.3	Minimum requirement for BS type 2-O

For BS type 2-O, the OTA transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.5.10.3-1.

Table 12.5.10.3-1: Minimum requirement for the OTA transmitter transient period between SBFD and non-SBFD operation for BS type 2-O

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 3 |
| ON to OFF | 3 |

## << End of change 7 >>

## << Start of change 8 >>

### 12.6.5	OTA in-band selectivity and blocking for SBFD

#### 12.6.5.1	OTA Adjacent Channel Selectivity for SBFD

##### 12.6.5.1.1	General

OTA Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

NOTE: SBFD UL subband is not configured adjacent to the Radio Bandwidth edge where cross link interference from adjacent channel may appear.

##### 12.6.5.1.2	Minimum requirement for SBFD capable BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For SBFD operation, the OTA wanted and the interfering signal are specified in table 12.6.5.1.2-1 for OTA ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

Table 12.6.5.1.2-1: OTA ACS requirement for BS type 1-O for SBFD

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB | Wide Area BS: -28 - ΔOTAREFSENS Medium Range BS: -32 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS | ±10 | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |
| NOTE 1: EISREFSENS is given in clause 12.6.3. NOTE 2: For lowest/highest carrier is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. |  |  |  |  |

##### 12.6.5.1.3	Minimum requirement for SBFD capable BS type 2-O

The general minimum requirements are the same as those specified in clause 10.5.1.3, except for EISREFSENS. The EISREFSENS depends on the SBFD capable BS class and the SBFD UL subband, and it is specified in clause 12.6.3.

#### 12.6.5.2	OTA in-band blocking for SBFD

##### 12.6.5.2.1	General

The OTA in-band blocking characteristics is a measure of the receiver's ability to receive an OTA wanted signal at its assigned channel in the presence of an unwanted OTA interferer, which is an NR signal for general blocking or an NR signal with one RB for narrowband blocking.

NOTE 1: For FR1 WA BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz with 288 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements.

NOTE 2: For FR1 MR BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz outdoor deployment with 167 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements or in the context of indoor.

NOTE 3: SBFD UL subband is not configured adjacent to the Radio Bandwidth edge where cross link interference from adjacent channel may appear.

##### 12.6.5.2.2	Minimum requirement for SBFD capable BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with OTA wanted and OTA interfering signal specified in tables 12.6.5.2.2-1 for general OTA blocking requirements. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The OTA in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

Table 12.6.5.2.2-1: General OTA blocking requirement for BS type 1-O for SBFD

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB | Wide Area BS: -25 - ΔOTAREFSENS Medium Range BS: -32 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS | ±30 | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |
| NOTE 1: EISREFSENS is given in clause 12.6.3. NOTE 2: For lowest/highest carrier which is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. NOTE 3: For the carrier which is adjacent to sub-block gap and configured in DU/UD SBFD operation, the interfering signal only applies when DL subband is adjacent to sub-block edge inside a sub-block gap. |  |  |  |  |

##### 12.6.5.2.2	Minimum requirement for SBFD capable BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For BS type 2-O, the OTA wanted and OTA interfering signals are provided at RIB using the parameters in table 12.6.5.2.2-1 for general OTA blocking requirements. The reference measurement channel for the wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The OTA blocking requirements are applicable outside the Base Station RF Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

Table 12.6.5.2.2-1: General OTA blocking requirement for BS type 2-O

| Frequency Range | BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA wanted signal mean power (dBm) | OTA interfering signal mean power (dBm) | OTA interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of OTA interfering signal |
| --- | --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200, 400 | EISREFSENS + 6 dB | -44 | ±75 | 50 MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| NOTE 1: EISREFSENS and EISREFSENS_50M are given in clause 12.6.3. NOTE 2: For lowest/highest carrier which is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. NOTE 3: For the carrier which is adjacent to sub-block gap and configured in DU/UD SBFD operation, the interfering signal only applies when DL subband is adjacent to sub-block edge inside a sub-block gap. |  |  |  |  |  |

## << End of change 8 >>