3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2513795

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	Discussion on RRM requirements for Rel-20 AIML

Agenda item:			7.9.4

Document for:	Discussion

- Introduction

In RAN#108, RAN plenary approved a new AI/ML work item of Rel-20 AI/ML for NR air interface Phase 2 in [1], which has been further revised in [2] in RAN#109.

The latest objectives of Rel-20 AI/ML for NR air interface Phase 2 are described in [2] including:

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In this contribution, we analyze the overall impacts to RRM requirements related to the above objectives.

- Discussion

RAN1 started discussion in last RAN1#122 meeting for Rel-10 AI/ML for NR air interface Phase 2. According to the conclusion of Rel-19 study for AI/ML for NR air interface, other than Rel-19 AI/ML for NR air interface Phase 1, further CSI compression via two-sided AI/ML model will be enhanced in RAN1 and RAN4.

In last RAN1#122 meeting, RAN1 has already discussed the target CSI type, pairing ID, quantization codebook.

The initial agreements are achieved as follow:

Agreement For CSI feedback via two-sided model, support at least precoding matrix as target CSI type. - FFS whether consider channel matrix as further improvement, starting with evaluations, e.g. potential fusion with SRS measurement (SRS period/hopping, DL/UL reciprocity), frequency granularity of channel matrix, number of Rx antennas, etc. Agreement Specify the precoding matrix feedback via two-sided model as follows: For a certain layer l and rank ν, - A precoding matrix is mapped with a latent message zl=z0l,z1l,⋯,zdzl,ν-1l, with l = 1, ..., v where l is the layer index. - The mapping is subject to [at least] a pairing ID configured by high-layer signalling. - FFS dzl,ν is common or specific for different layers or ranks - Note: the terminology “latent message” is used for discussion purposes, the detailed name can be discussed further - zl is quantized and mapped to bit sequence before reporting. Agreement For precoding matrix feedback, for a certain layer l of rank ν, the latent message zl contains dzl,ν real values - If scalar quantization (SQ) supported, it refers to the quantization of each real value independently, - The payload size for layer l and rank ν is determined by dzl,ν×Ql,ν, where Ql,ν is the number of bits per scalar. - If vector quantization (VQ) is supported, it refers to the quantization of a segment of L (L>1) consecutive real values jointly, and the segment(s) are separately quantized. - The payload size for layer l and rank ν is determined by dzl,νL×Ql,ν, where L is the length of the vector segment, Ql,ν is the number of bits per segment - Note: Combining two real-values as a complex value and performing amplitude-phase quantization is precluded. Agreement: For precoding matrix feedback, consider the following aspects for payload determination - Support only SQ, or only VQ or both - dzl,ν and / or Ql,ν and / or L are fixed values or determined from multiple values based on NW configuration or UE reporting. - dzl,νand / or Ql,ν is layer-common or layer-specific, rank-common or rank-specific. - If configured, dzl,νand / or Ql,ν and / or L above are configured separately from pairing ID, or configured by Pairing ID. - The configuration may be via other parameters which are used to derive dzl,ν, L and Ql,ν. Note: Consider limiting the total number of possible payload sizes considering the inter-vendor training challenges. Note: dzl,ν, Ql,ν and L are independent on subband and port configurations, - FFS each combination of dzl,ν, Ql,ν and L is applicable to a certain set of subband and port configurations.

In non-AI PMI mechanism in 5GNR previous releases, RAN4 specified the PMI for demodulation requirements. There are no RRM requirements for precoding matrix indicator related.

However, in Rel-19 AI/ML for NR air interface, RAN4 discussed and agreed to specify AI/ML for CSI compression and CSI prediction; beam management and positioning accuracy enhancement. Take an example, RAN4 achieved the consensus that the RRM shall consider to define RRM requirements for LCM procedures for AI/ML use case in RAN4#111 meeting:

Issue 1-5: LCM Requirements Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - identification - selection - activation - deactivation - switching - fallback to non-AI operation - performance monitoring - Others are not precluded

If there is no standardization, each vendor can use their siloed AI solutions which cannot communicate or be managed. The 3GPP defined LCM procedures for testing procedures to ensure the performance. In the new use case in RAN1 for PMI, similar as Rel-19 AI/ML, the LCM procedure is also the key RRM impacts.

According to Rel-19 AI/ML discussion, the LCM procedures are based on activation; deactivation; switching; fallback to non-AI operation, and performance monitoring.

Proposal 1: RAN4 to discuss RRM requirements for Rel-20 AI/ML for NR air interface Phase 2, related to: RRM requirements for LCM procedure. RAN4 to use Rel-19 LCM framework as a baseline for Rel-20 AI/ML.

RAN4 to discuss whether/how to specify RRM requirements for LCM procedures including:

- Activation

- Deactivation

- Switching

- Fallback to non-AI operation

- Performance monitoring

In Rel-19 AI/ML study discussion, RAN4 deprioritize the discussion on the switching and fallback delay requirements.

Switching delay and Fallback delay: - Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI, discussion.

Although it is deprioritized in Rel-19 AI/ML study for the switching delay and fallback delay, whether RAN4 shall specify the delay requirements for switching and fallback to non-AI/ML operation, it depends on the use case and what is the AI/ML operation in RAN1/RAN2. There is no clear procedure in RAN1/RAN2 procedure for new use case until now.

Proposal 2: FFS on switching delay and fallback delay in Rel-20 AI/ML for LCM RRM requirements.

In Rel-19 AI/ML discussion, RAN4 discussed for beam prediction of activation delay and achieved the agreements as:

Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI Agreement: - No need to create deactivation requirements for AI functionality

For Rel-19 use case of beam prediction, it is agreed for the start point of activation delay and no need for deactivation requirements. In Rel-20 AI/ML for new use case, we think the start point of activation delay is quite similar as that in beam predication in Rel-19. For the deactivation requirements, it may be not the same as that in Rel-19. Further RAN1 progress is needed.

Proposal 3: For Rel-20 AI/ML for new use case, RAN4 can use the same start point of activation delay. Although no deactivation requirements for Rel-19 AI/ML, Rel-20 AI/ML use case may be different. Further RAN1/RAN2 progress is needed.

The performance monitoring is another key RRM requirements for LCM procedures. In RAN4#11b meeting, RAN4 achieved the agreements for CSI reporting requirement and testing framework for CSI prediction

Performance monitoring Agreement: RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction FSS which requirements are to be introduced. Agreement: Introduce the following requirements for CSI performance monitoring: Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this

For beam management, RAN4 achieved the agreements:

Agreement: - Do not define performance monitoring delay requirements for LCM purpose - If the monitoring delay is identificated as critical in the future, this issue can be revisited.

Typically, RAN4 shall discuss whether to define RRM requirements for performance monitoring including delay and accuracy requirements to ensure the correct functionality and the accuracy during the performance monitoring is met. In beam management, RAN4 do not define performance monitoring delay requirements. However, in Rel-20 AI/ML CSI compression, we prefer to wait further RAN1 progress to discuss the delay and accuracy requirements for performance monitoring requirements.

Proposal 4: RAN4 to discuss the delay and accuracy requirements for performance monitoring in LCM procedures.

- Conclusion

In this contribution, we provide our proposals for overall RRM impacts of Rel-20 AI/ML for NR air interface Phase 2:

Proposal 1: RAN4 to discuss RRM requirements for Rel-20 AI/ML for NR air interface Phase 2, related to: RRM requirements for LCM procedure. RAN4 to use Rel-19 LCM framework as a baseline for Rel-20 AI/ML.

RAN4 to discuss whether/how to specify RRM requirements for LCM procedures including:

- Activation

- Deactivation

- Switching

- Fallback to non-AI operation

- Performance monitoring

Proposal 2: FFS on switching delay and fallback delay in Rel-20 AI/ML for LCM RRM requirements.

Proposal 3: For Rel-20 AI/ML for new use case, RAN4 can use the same start point of activation delay. Although no deactivation requirements for Rel-19 AI/ML, Rel-20 AI/ML use case may be different. Further RAN1/RAN2 progress is needed.

Proposal 4: RAN4 to discuss the delay and accuracy requirements for performance monitoring in LCM procedures.

- Reference

[1] RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Qualcomm

[2] RP-252445, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm