3GPP TSG-RAN4 Meeting #116bis	        R4-2513809  Prague, Czech Republic, Oct. 13-17, 2025

Title:	LS to RAN5 on LP-WUS/WUR Testability

Response to:

Release:	Release 19

Work Item:	NR_LPWUS-Core

Source:	RAN WG4

To:	RAN5

Cc:

Contact Person:

#### Name:	Ruixin Wang

#### E-mail Address:	ruixin.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments: None	None

- Overall Description:

RAN4 has concluded the RF core requirements for Rel-19 LP-WUS/WUR WI in RAN4#116 meeting, and the WI is formally closed in RAN#109 meeting.

According to the WID objective, RAN4 also discussed the testability issue of LP-WUS/WUR and reached the following agreements:

- The following settings can be used for Rx requirements related testing:

  - LP-WUS, LPSS and SSB all share the same EPRE

  - LPSS shares the same M and same length as LP-WUS

  - LPSS shares the same length in OFDM symbols as LP-WUS

  - LPSS is transmitted at lowest periodicity (160 ms), or equivalently at highest rate of repetition

  - SSB is transmitted at 20 ms periodicity

- For the case when LR supporting both OOK and OFDM waveforms, only test with OFDM waveform

- Testing time reduction of LP-WUS RF requirements can be considered when define the test cases

- For whether introduce a test mode for UE LP-WUS/WUR testing, leave the decision to RAN5

RAN4 would like to ask RAN5 to take above testability agreements into account when developing conformance test specifications.

2. Actions:

To RAN5.

ACTION: 3GPP RAN4 respectfully asks RAN5 to take the above information into account when developing LP-WUS/WUR RF conformance test specifications.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting#117		17th – 21st November 2025		Dallas, USA