3GPP TSG-RAN4 Meeting #116bis	R4-2513837

Prague, Czech Republic, 13 October – 17 October 2025

Source:			Ericsson

Title:			US 4.9 GHz band – BS RF aspects

Agenda item:		5.11.2

Document for:		Approval

# Introduction

In RAN#108, a WI ([1]) has been approved to introduce a new 4.9GHz band for US.

In last RAN4#116 meeting, as captured in Chair notes ([3]), RAN4 agreed to conclude the study phase, meaning also that the specification work could start from this meeting. This contribution is discussing the UE RF requirements, analyzing the relevant regulation.

# Discussion

## Background

The Commission allocated the 4.9 GHz band for public safety use in 2002. Since then, the rules have evolved in response to changing conditions, including band use and technological innovation. In 2020, to stabilize the band, the Public Safety and Homeland Security Bureau (PSHSB) and the Wireless Telecommunications Bureau (WTB) announced a freeze on applications in this band. In January 2023, the Seventh Report and Order and Ninth Further Notice established a comprehensive and coordinated nationwide approach to managing the 4.9 GHz band, adding the role of a nationwide Band Manager. In the Eight Report and Order ([2]), FCC enforces the role of the Band Manager, further describing his role and responsibilities.

It should also be noted that the existing NR band n79 (4.4-5.0 GHz) encompasses this new 4.9 GHz band.

## FCC part 90 and 3GPP impacts

### Overview

The new 4.9 GHz US band is a public safety band and is specified in FCC §90. The following sub-parts are notably key inputs for the specification of this new band in 3GPP:

- §90.1213: band plan.

- §90.1215: Power limits.

- §90.210: Emission masks.

### Power limits (§90.1215)

FCC specified the following maximum conducted output power (below table), mentioning that higher power devices, using other channel bandwidth or not, are limited to a peak PSD of 21dBm/MHz.

According to §90.1215 (2), for “high power point-to-point and point-to-multipoint operations (both fixed and temporary-fixed rapid deployment) may employ transmitting antennas with directional gain up to 26 dBi without any corresponding reduction in the maximum conducted output power or spectral density”, this would correspond to BS.

And, as stated in §90.1215 (f), for BS, the maximum EIRP must not exceed 85 dBm. There is an additional restriction when the path length is less than 17 km (New EIRP limit = 55 dBW-40*log(17/B) dBW, where B = the actual path length in kilometers) but as RAN4 doesn’t capture requirement which might be deployment related, this restriction should not be captured in 3GPP specifications (the restriction still applies of course).

It should be noted that, even if RAN4 usually doesn’t specify any maximum output power for BS, similar type of requirements has already been specified for Railway bands n100 and n101 (as shown below, captured as regional requirement). RAN4 should then take the same approach for this US 4.9 GHz band.

Proposal 1: Capture the FCC maximum EIRP limit (85dBm) as an additional regional requirement for the BS.

### Emission Masks (§90.210)

FCC specified 2 emissions masks in §90.210: mask L for low power transmitter (20dBm or less) and mask M for high power transmitter (greater than 20 dBm). When specifying the US 4.9 GHz band, UE and BS shall then follow the emission mask M.

Similar to the UE, it should be noted that the FCC emission mask M starts inside the channel bandwidth: (m)(2) specifies a limit in the 45-50% of the authorized bandwidth, meaning the edge RB(s) might not be usable or, at least, they would have to be de-boosted.

Comparing to the WA OBUE mask for NB bands above 1 GHz and for category A (Table 6.6.4.2.1-2 in TS 38.104), after converting the 3GPP measurement bandwidth to 1 MHz when relevant, the FCC emission mask M is more stringent for 10MHz and partly more stringent for 50MHz channel bandwidth, as shown in Figure 3.

Observation 1: The FCC mask M is in general more stringent than the 3GPP OBUE cat A mask.

This means that an additional OBUE requirement (clause 6.6.4.2.5) for this new US 4.9GHz band should be specified in TS 38.104.

Proposal 2: Specify an additional OBUE requirement for the US 4.9 GHz band based on FCC mask M.

Also, as the FCC emission mask M goes beyond the OBUE domain, in the spurious domain, an additional spurious emission limit (clause 6.6.5.2.3) for the new US 4.9 GHz band would need to be introduced as well in TS 38.104.

Observation 2: The FCC mask M covers the 3GPP OBUE domain (including inside the channel bandwidth, at 5% of the edge) and the 3GPP spurious domain.

Proposal 4: Specify an additional spurious requirement for the US 4.9 GHz band.

Figure 3:  Comparison of 3GPP OBUE Cat A with FCC Mask M for 10MHz (left) and 50MHz (right) channel BW

## BS impact overview

The following Table 1 gives an overview of the TS 38.104 impacts, based on the different aspects discussed above.

Table 1: TS 38.104 impacts overview

| Requirements | Any impact? | Comments |
| --- | --- | --- |
| 4.5 Regional requirements | Yes | Add subclauses with US 4.9 GHz band requirements in Table 4.5-1. |
| 6. Conducted transmitter requirements |  |  |
| 6.1 General | No |  |
| 6.2 Base station output power | Yes | Add an additional requirement in clause 6.2.4 as described in 2.2.4.2. |
| 6.3 Output power dynamics | No |  |
| 6.4 Transmit ON/OFF | No |  |
| 6.5 Transmitted signal quality | No |  |
| 6.6 Unwanted emissions | No |  |
| 6.6.1 General | No |  |
| 6.6.2 Occupied bandwidth | No |  |
| 6.6.3 ACLR | No |  |
| 6.6.4 Operating Band Unwanted Emissions | Yes | Add the new US 4.9GHz band to the list of bands for which table 6.6.4.2.1-2 applies. Add an additional requirement in clause 6.6.4.2.6 as described in 2.2.4.2 |
| 6.6.5 Transmitter spurious emission | Yes | Add the new US 4.9GHz band in the coexistence Table 6.6.5.2.3-1. Add the new US 4.9GHz band in the colocation Table 6.6.5.2.4-1. Add an additional requirement in clause 6.6.5.2.3 as described in 2.2.4.2. |
| 6.7 Transmitter intermodulation | No |  |
| 7. Conducted receiver characteristics |  |  |
| 7.1 General | No |  |
| 7.2 Receiver sensitivity level | No |  |
| 7.3 Dynamic range | No |  |
| 7.4 In-band selectivity and blocking | No |  |
| 7.5 Out of band blocking | No |  |
| 7.6 Receiver spurious emissions | No |  |
| 7.7 Receiver intermodulation | No |  |
| 7.8 In channel selectivity | No |  |
| 9. Radiated transmitter characteristics |  |  |
| 9.1 General | No |  |
| 9.2 Radiated transmit power | No |  |
| 9.3 OTA base station output power | Yes | Add an additional requirement in clause 9.3.4 as described in 2.2.4.2. |
| 9.4 OTA output power dynamics | No |  |
| 9.5 OTA transmit ON/OFF | No |  |
| 9.6 OTA transmitted signal quality | No |  |
| 9.7 OTA Unwanted emissions | No |  |
| 9.7.1 General | No |  |
| 9.7.2 Occupied bandwidth | No |  |
| 9.7.3 ACLR | No |  |
| 9.7.4 Operating Band Unwanted Emissions | No | Note that this subclause refers to 6.6.4.2 |
| 9.7.5 Transmitter spurious emission | No | Note that this subclause refers to 6.6.5.2.3 |
| 9.8 OTA transmitter intermodulation | No |  |
| 10 Radiated receiver characteristics |  |  |
| 10.1 General | No |  |
| 10.2 OTA sensitivity | No |  |
| 10.3 OTA receiver sensitivity level | No |  |
| 10.4 OTA dynamic range | No |  |
| 10.5 OTA in-band selectivity and blocking | No |  |
| 10.6 OTA out of band blocking | No |  |
| 10.7 OTA receiver spurious emissions | No |  |
| 10.8 receiver intermodulation | No |  |
| 10.9 OTA in channel selectivity | No |  |

Proposal 5: Agree with Table 1 which lists the impacts to TS 38.104 when specifying the new US 4.9GHz band.

# Conclusion

In this contribution, we studied the FCC regulation for the new US 4.9 GHz band and made the following observations and proposals related to the BS RF impacts:

Proposal 1: Capture the FCC maximum EIRP limit (85dBm) as an additional regional requirement for the BS.

Observation 1: The FCC mask M is in general more stringent than the 3GPP OBUE cat A mask.

Proposal 2: Specify an additional OBUE requirement for the US 4.9 GHz band based on FCC mask M.

Observation 2: The FCC mask M covers the 3GPP OBUE domain (including inside the channel bandwidth, at 5% of the edge) and the 3GPP spurious domain.

Proposal 4: Specify an additional spurious requirement for the US 4.9 GHz band.

Proposal 5: Agree with Table 1 which lists the impacts to TS 38.104 when specifying the new US 4.9GHz band.

# References

- RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu

- Eight Report and Order, WP Docket No. 07-100, 22 October 2024.

- RAN4 Chair notes: https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_116/Inbox/Chair_Notes/Main_Session_Notes/1.Mon/7.Draft_R4-2509228_%5B116%5D%5B115%5D_US4.9GHz_Topic_Summary_moderator.docx