3GPP TSG-RAN WG4 Meeting #116bis	R4-2513871

Prague, CZ, Oct. 13 – 17, 2025

Agenda item:	6.12.3

Source:	Samsung

Title:	Discussion on Rel-19 NR MIMO Phase 5 UE demodulation performance and CSI requirements

Document for:	Discussion

- Introduction

In the RAN plenary RAN#102 meeting the WID [1] for Rel-19 NR MIMO Phase 5 has been approved and further revised in RAN#105 meeting [2]. This is our third meeting on demodulation and CSI reporting performance requirements for Rel-19 NR MIMO Phase 5 WI. General impacts of each objective have been discussed in RAN4#115 and RAN4#116 meeting, agreements are captured in [3][4]. Until now, only PMI reporting requirements for typeI-SinglePanel-r19 has been agreed.

In addition, for objective 2 (CSI enhancement up to 128 CSI-RS ports), the impact of PMI reporting requirements for typeII-Doppler-r19 and eTypeII-r19 have been discussed, but no consensus. At the same time, one sub-object of objective 2 about SRS port grouping need further discussion. For objective 3 (UE reporting enhancement for CJT), UE demodulation/CSI reporting impact for UE reporting enhancement for CJT also need further analysis.

In this contribution, we provide our analysis and views on the scope and requirement assumptions for objectives which have potential impacts.

- General scope

  - CSI support up to 128 CSI-RS ports

In general, for FR1 CSI support up to 128 CSI-RS ports, which are expected to bring improvements in areas such as spectral efficiency, interference management, and beamforming capabilities, total five codebook types are extended, i.e., 'typeI-SinglePanel-r19', 'typeI-MultiPanel-r19', 'eTypeII-r19', 'typeII-FePortSelection-r19' and 'typeII-Doppler-r19'. These five codebook types fall into two categories as ‘Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources’ and ‘Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources’.

Clearly, the enhancements of CSI support up to 128 CSI-RS ports are not merely about increasing the number of antenna ports but also significantly improve on UE channel state estimation/reporting capabilities. Validating both aspects is essential for ensuring robust performance in real-world deployments. Higher port counts require more advanced precoding and CSI-RS density, which increasing the computational load. While, different codebook types supporting higher CSI-RS ports corresponding to different UE implementation complexity, it is hard to conclude UE could support all five Rel-19 codebook types once it supports one Rel-19 codebook type.

In RAN4#115 meeting, PMI reporting requirements for typeI-SinglePanel-r19 is agreed to be introduced by companies. Meanwhile, considering we didn’t define PMI reporting requirement for multi panel scenario and port selection scenario before, we think it is not suitable to introduce PMI reporting requirements for 'typeI-MultiPanel-r19' and 'typeII-FePortSelection-r19' in Rel-19. However, the typeII codebook improvements should be validated holistically- both in terms of scalable antenna configurations and CSI estimation/reporting performance. Therefore, we think new PMI reporting requirements for enhanced codebook ‘eTypeII-r19’ should be introduced. Furthermore, even up to 128 CSI-RS ports could be verified by 'typeI-SinglePanel-r19' test, and ‘eTypeII’ codebook could be verified by ‘eTypeII-r16’ test, it is still unable to deduce the conclusion that UE could support ‘eTypeII-r19’ considering higher computational complexity of eTypeII codebook.

In RAN4#116 meeting, some companies argued that eType II codebook is not used in practical network. However, firstly, since specification are usually ahead of practical network and eType II codebook is a typical codebook type widely used in Trial network, it’s not a reasonable reason for dropping the introduction of related performance requirements. Secondly, we can confirm that eType II has been supported by some companies’ product. Thirdly, a number of companies want eType II only for 6G. Finally, in this stage TU allocated for defining requirements for MIMO Phase 5 WI, and we have enough time to define related requirements. If we miss this time slot, it would be hard to find another time slot to define performance requirements for eTypeII-r19 in the future. Then we have to face the problem that no performance test requirements to promise the PMI reporting performance for UE supporting eTypeII-r19.

Observation 1: The enhancements of CSI support up to 128 CSI-RS ports are not merely about increasing the number of antenna ports but also significantly improve on UE channel state estimation/reporting capabilities.

Observation 2: It is hard to conclude UE could support all five Rel-19 codebook types once it supports one Rel-19 codebook type 'typeI-SinglePanel-r19'.

Observation 3: Even up to 128 CSI-RS ports could be verified by 'typeI-SinglePanel-r19' test, and ‘eTypeII’ codebook could be verified by ‘eTypeII-r16’ test, it is still unable to deduce the conclusion that UE could support ‘eTypeII-r19’ considering higher computational complexity of eTypeII codebook.

Observation 4: eType II codebook is a typical codebook type widely used in Trial network, and eType II has been supported by some companies’ product.

Observation 5: TU allocated for defining requirements for MIMO Phase 5 WI, and we have enough time to define related requirements. If we miss this time slot, it would be hard to find another time slot to define performance requirements for eTypeII-r19 in the future. Then we have to face the problem that no performance test requirements to promise the PMI reporting performance for UE supporting eTypeII-r19.

Proposal 1: Prefer to introduce new PMI reporting requirements for enhanced codebook ‘eTypeII-r19’ with first priority.

  - SRS ports grouping

The last one of objective 2 (CSI enhancement up to 128 CSI-RS ports) is ‘SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers’ which is targeting to facilitate more-than-4-layer DL reception, the description in WID [2] as below.

- SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

RAN1 introduced SRS port grouping in 38.214, the details are as below.

SRS port grouping in 38.214

For a UE configured with one or more SRS resource sets with higher layer parameter usage set to 'antennaSwitching', with a total of PSRS=6 or 8 ports across the resources intended for xT6R or xT8R, respectively, if the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI' and the UE is configured with the higher layer parameter SRSPortGrouping-r19, the UE can assume that SRS port group 0 corresponds to codeword 0 and comprises the even PSRS/2 ports, and that SRS port group 1 corresponds to codeword 1 and comprises the odd PSRS/2 ports out of the total PSRS ports. The SRS ports are indexed in an ascending order according to SRS resource ID and port number within each SRS resource, for one SRS resource set, or according to SRS resource set ID, SRS resource ID in a set and port number within each SRS resource, for multiple aperiodic SRS resource sets.

Although the SRS porting grouping is somehow to facilitate low complexity receiver of more-than-4-layer DL reception, in fact it only defines the mapping between SRS port groups and codewords, and the basic rule is no enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback and DCI format. For UEs which supporting SRS port grouping, it seems the implementation on UE receiver is still flexible, for 8Rx receiver, it could be two separate 4Rx MIMO detectors, could be two joint 4Rx MIMO detectors and even could be joint 8Rx MIMO detector. Hence the SRS port grouping should not be equivalent and restricted with receiver type as two separate 4Rx MIMO detectors.

About the verification of the mapping between SRS port grouping and codewords, both behaviors of network and UE could impact the final results. Considering no SRS related demodulation/CSI performance requirements are defined in current specification, the test methodology of SRS port grouping need further discussion if companies would like to introduce related performance test case.

Observation 6: For UEs which supporting SRS port grouping, it seems the implementation on UE receiver is still flexible, for 8Rx receiver, it could be two separate 4Rx MIMO detectors, could be two joint 4Rx MIMO detectors and even could be joint 8Rx MIMO detector.

Proposal 2: The SRS port grouping should not be equivalent and restricted with receiver type as two separate 4Rx MIMO detectors.

Proposal 3: About the verification of the mapping between SRS port grouping and codewords, we are open to discuss the test methodology if any.

  - UE reporting enhancement for CJT

This objective of UE reporting enhancement for CJT is targeting on the enhancement for CJT deployments under non-ideal synchronization and backhaul. In Rel-18, Type-II CSI has been enhanced to accommodate CJT assuming ideal synchronization and backhaul. Scenarios such as inter-site CJT and a base station equipped with distributed remote radio heads (RRHs) require additional delay and phase/frequency calibration. For TDD, additional inter-TRP DL/UL calibration across TRPs is also beneficial to ensure proper DL/UL reciprocity holds. As the UE possesses more knowledge on DL channel condition in both FDD and TDD, the need for inter-TRP calibration reporting measured from CSI-RS is evident not only to expand the deployment scenarios, but also to offer additional robustness to CJT operation. For inter-TRP delay and frequency/phase calibration for coherent joint transmission (CJT), CSI-RS per TRP is transmitted to the UE to measure the CSI-RS and estimate the reporting quantity including inter-TRP delay offset, frequency offset, or phase offset. A CSI report associated with the reporting quantity is carried on a PUSCH. RAN1 introduced new report quantity configurations as 'cjtc-Dd', 'cjtc-F', 'cjtc-Dd-F' and 'cjtc-P' for inter-TRP delay offset, frequency offset and phase offset separately. The definitions in 38.214 are copied as below.

38.214 Chapter 5.2.1

- 5.2.1.4.9 CJTC-Dd reporting of delay offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-Dd', the interval [δin,δin+1) in which the delay offset, Dn,offset, falls into, for CSI-RS resource set n=0,…,NTRP-1, n≠nref, is indicated by iDO=i0⋯iNTRP-1 in∈{0,1,…,MD-1} where the mapping from in to δin is given by δin=in⋅ADMD-1 and δMD=∞, and the values of AD∈{CP/2, CP}, where CP is the length of the cyclic prefix for symbols other than 0 and 7⋅2μ in the subframes associated with the NTRP configured CSI-RS resource sets for tracking, and MD∈{32, 64, 128, 256} are configured by the higher layer parameters valueOfAD and valueOfMD, respectively. The index value in=MD-1, corresponding to the interval [AD,∞), represents an out-of-range delay offset Dn,offset. - 5.2.1.4.10 CJTC-F reporting of frequency offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-F', the frequency offset value, FOn , for CSI-RS resource set n=0,…,NTRP-1, n≠nref, is indicated by kFO=k0⋯kNTRP-1 kn∈{0,1,…,MFO-1} where the mapping from kn to FOn is given by FOn=kn⋅AFOMFO-1,kn=0,1,…,MFO-2'invalid',kn=MFO-1 and the values of AFO∈{0.1ppm, 0.2ppm} and MFO∈{16, 32, 256} are configured by the higher layer parameters valueOfAFO and valueOfMFO, respectively. - 5.2.1.4.11 CJTC-P reporting of phase offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-P', if subbandSize is set to 'wideband', the phase offset value, Φn , for CSI-RS resource n=0,…,NTRP-1, n≠nref, is indicated by pΦ=p0⋯pNTRP-1 pn∈{0,1,…,MΦ-1} otherwise, the phase offset values, {Φn,s}, for the NSB-P configured subbands s=0,1,…,NSB-P-1 and CSI-RS resource n=0,…,NTRP-1, n≠nref, is indicated by pΦ=p0,0⋯p0,NSB-P-1⋯pNTRP-1,0⋯pNTRP-1,NSB-P-1 pn,s∈{0,1,…,MΦ-1} where the mapping from pn to Φn is given by Φn=pn⋅2πMΦ-1,pn=0,1,…,MΦ-2'invalid',pn=MΦ-1 the mapping from pn,s to Φn,s is given by Φn,s=pn,s⋅2πMΦ-1,pn,s=0,1,…,MΦ-2'invalid',pn,s=MΦ-1 and the value of MΦ∈{16, 32} is configured by the higher layer parameter valueOfMPhi.

Accordingly, the correct estimations, reporting and compensation of the delay offset, frequency offset and phase offset should be promised in order to get well performance for CJT deployments under non-ideal synchronization and backhaul. In which, estimations and reporting depend on UE, while the compensation depends on network.

About the estimations of delay offset, frequency offset and phase offset, accuracy requirements about delay and frequency offsets were agreed to be introduced on RAN4#114bis and RAN4#115 meetings RRM session [5][6] as below, and phase offset were agreed to be deprioritized in Rel-19 on RAN4#115 meeting RRM session [6] as below.

Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration? Agreement: - Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified)

Agreement on RAN4#114bis meeting RRM session

Agreement on RAN4#115 meeting RRM session

Issue 1-1: How to define measurement period requirements for delay and frequency offset reporting for CJT calibration? Agreement: - RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting. - RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19.

According to the definition in RAN1 spec, the functions to estimate and report delay/frequency/phase offsets should be straightforward to be implemented and delay/frequency offsets could be ensured by RRM accuracy requirements. Therefore, new requirements for CJT calibration offset reporting for the time/frequency offsets are not needed. Furthermore, the performance of delay/frequency/phase offsets compensation highly depends on network implementation, especially the phase offset for phase coherence of multi-TRPs should be guaranteed by network when it transmits signal, but not guaranteed by UE measurement and reporting. Hence, new requirements for CJT calibration offset reporting for the phase offset also seems not necessity.

Observation 7: The functions to estimate and report delay/frequency/phase offsets should be straightforward to be implemented and delay/frequency offsets could be ensured by RRM accuracy requirements.

Observation 8: The performance of delay/frequency/phase offsets compensation highly depends on network implementation, especially the phase offset for phase coherence of multi-TRPs should be guaranteed by network when it transmits signal, but not guaranteed by UE measurement and reporting.

Proposal 4: For UE reporting enhancement for CJT, prefer not to introduce new requirements for CJT calibration offset reporting for time/frequency/phase offsets.

In addition, for the Rel-19 aperiodic CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports are jointly triggered or separately triggered with delayOffsetCompensation indication, UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI. If UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report, new UE function to compensate it before CSI reporting is also needed. The detail descriptions are as below. Consequently, whether to introduce new PMI reporting test for CJTC Dd and Rel-18 eType-II CJT CSI reports with delay offset compensation need further discussion.

38.214 Chapter 5.2.1.4.2

- Subject to UE capability, the CSI-ReportConfig with reportQuantity set to 'cjtc-Dd' can be linked, by the higher layer parameter linkedCJTCReport, to an aperiodic CSI-ReportConfig with reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18' and with the corresponding CSI-RS resources for channel measurement that are aperiodic and not QCLed with the same reference signal with respect to {average delay}. The CSI-RS resource sets associated with the CJTC-Dd report are linked to the CSI-RS resources associated with the 'typeII-CJT-r18' report by a fixed correspondence between the resource set IDs and resource IDs, respectively, in sequential order of configuration in their respective Resource Setting. The UE expects that the number of CSI-RS resource sets configured in the Resource Setting for the CJTC-Dd report is the same as the number of CSI-RS resources in the Resource Setting associated with the linked 'typeII-CJT-r18' report. - When the two CSI reports are jointly triggered, the UE is expected to compensate for the delay offset values corresponding to the CSI-RS resources measured for 'typeII-CJT-r18' calculation and reported in the CJTC-Dd report in the same reporting instance. - When the two CSI reports are separately triggered, the higher layer parameter delayOffsetCompensation indicates whether the UE is expected to compensate for the delay offset values corresponding to the CSI-RS resources measured for 'typeII-CJT-r18' calculation and reported in the latest CJTC-Dd report whose reporting instance’s last symbol is before the first symbol of the DCI triggering the 'typeII-CJT-r18' report. If multiple 'typeII-CJT-r18' reports linked to CJTC-Dd report(s) are triggered in the same aperiodic trigger state and delayOffsetCompensation is configured, the UE is expected to apply delay offset compensation for all the linked 'typeII-CJT-r18' reports.

Observation 9: When linking CJTC Dd and Rel-18 eType-II CJT CSI reports are jointly triggered or separately triggered with delayOffsetCompensation indication, UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI.

Whether new UE PMI reporting test cases should be specified for Rel-18 Type-II CJT with delay offset compensation need further analysis. If we use the legacy test metric of PMI reporting as below, it implicitly means that the DO compensation should also be applied at BS/TE side in order to get the maximum throughput with following PMI, otherwise the precoders UE reported with DO compensation could not align with the transmission channel environment for PDSCH. However, since we are facing the demodulation test of UE, it's better if we can preclude the impact of TO/CFO Compensation which is implemented by BS/TE since our target is to test UE. If our target is to test BS, it's no doubt the impact of TO/CFO Compensation should be considered. About the test on UE TO/CFO reporting, RRM agreed to introduce accuracy tests for both FDD and TDD, with maximum TO=CP, CFO=350Hz, which we assume are enough for TO/CFO reporting. For PMI reporting requirements of Rel-18 Type-II CJT with delay offset compensation, we don't have strong preference to specify related test cases, since except the TO compensation function at UE side, other functions could be verified by agreed cases, e.g., PMI reporting performance of Rel-18 Type-II CJT could be verified by Rel-18 Type-II CJT PMI reporting requirements, TO estimation and reporting could be assured by RRM accuracy test. Hence, if TO compensation at BS/TE side is mandatory for PMI reporting test of Rel-18 Type-II CJT with delay offset compensation, we don’t prefer to introduce new PMI reporting test for it.

The requirements for transmission scheme 1 with higher layer parameter codebookType set to 'typeII' or 'typeII-r16' or 'typeII-doppler-r18' or 'typeII-CJT-r18' are specified in terms of the ratio: In the definition of γ, for 16TX PMI requirements, is 90 % of the maximum throughput obtained at using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

Proposal 5: Do not prefer to introduce new PMI reporting test for CJTC Dd and Rel-18 eType-II CJT CSI reports with delay offset compensation, since delay offset compensation is also needed at BS/TE side.

- Test assumptions

In this section, we analysis and share our views on potential tests for UE demodulation and CSI reporting requirements.

  - Assumptions for typeI-SinglePanel-r19

In RAN4#115 and RAN4#116 meeting [3][4], for assumptions for typeI-SinglePanel-r19, agreements achieved on duplex mode, number of Rx antennas, propagation channel, extension on correlation matrix models, rank value, number of aggregated NZP CSI-RS resources, typeI-codebookMode-r19, MCS level, CQI/RI/PMI delay value, N1 and N2 configuration for 64 CSI-RS ports, CSI-RS port mapping method, etc. In this meeting, further discussions are needed for antenna correlation configuration, test metric and final MCS values.

NZP-CSI-RS configurations for 64 CSI-RS ports

For the NZP-CSI-RS configurations, based on the test parameters for Single PMI with 32TX TypeI-SinglePanel Codebook, more information about the additional CSI-RS resource should be specified. The configurations we used in our simulation could be summarized as below table.

| NZP CSI-RS | CSI-RS resource 1 and 2 | CSI-RS resource Type |  | Aperiodic |
| --- | --- | --- | --- | --- |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  | CSI-RS resource 1 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (3, 10) |
|  | CSI-RS resource 2 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (5, 12) |
|  | CSI-RS resource 1 and 2 | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |

Proposal 6: For NZP-CSI-RS configurations for 64 CSI-RS ports, prefer to use below table.

| NZP CSI-RS | CSI-RS resource 1 and 2 | CSI-RS resource Type |  | Aperiodic |
| --- | --- | --- | --- | --- |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  | CSI-RS resource 1 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (3, 10) |
|  | CSI-RS resource 2 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (5, 12) |
|  | CSI-RS resource 1 and 2 | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |

Scaling factors for MIMO correlation matrices

The scaling factor a is used to obtain a positive semi-definite correlation matrix. A channel correlation matrix must be positive semi-definite because it is fundamentally a covariance matrix. Covariance matrices, by their mathematical defination and physical interpretation, are also positive semi-definite. This property guarantees that the calculated power or variance of any signal projection is non-negative, which is a physical necessity. Hence we need to promise the correlation matrix as a positive semi-definite correlation matrix. We get the scaling factor by following steps:

1) Calculate Rspat (with full precision)

2) Loop the value of the scaling factor a from zero with step size 10^(-5)

3) Calculate Rmedium = [Rspat+a*In]/(1+a)  (with full precision)

4) Round the elements of Rmedium to 4 digits

5) Check if the the eig values of Rmedium are all no less than 0. If no, go back to step 2); if yes, select a as the final scaling factor.

Accordingly, for 64Tx (8,4,2) 4Rx, a=0.00091 and for 64Tx (8,4,2) 2Rx, a=0.0005 are proposed.

Proposal 7: Use scaling factor a to guarantee channel correlation matrix as a positive semi-definite matrix, for 64Tx (8,4,2) 4Rx, a=0.00091 and for 64Tx (8,4,2) 2Rx, a=0.0005.

MCS level for 64 CSI-RS ports

About the MCS level for 64 CSI-RS ports, agreement achieved to do down selection between MCS20 and MCS22 based on simulation results on last RAN4#116 meeting [4]. Our companion contribution [7] shows the SNR working points at 90% of maximum throughput are really low with MCS20. Therefore, we prefer to use MCS22 for 64 CSI-RS ports.

Proposal 8: Use MCS22 (Table 1) for typeI-SinglePanel-r19 with 64 CSI-RS ports PMI reporting requirements.

Test metric for 64 CSI-RS ports

About the test metric for PMI reporting requirements, there was a proposal to modify the legacy test metric as below since zero throughput observed with random Type I codebook.

Issue 1-2-1 Test metric for 64 CSI-RS ports Proposals: - Option 1: Modify the legacy test metric to following: γ=tuetrnd tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trndis the throughput measured at SNRue+[XdB] with random precoding. X could be specified to make the γ value in the reasonable range.. Way forward: - FFS based on simulation results provided in following meetings

According our companion contribution [7], legacy test metric still works. Moreover, the difference between eTypeII-r19 and typeI-SinglePanel-r19 could be clearly captured by legacy test metric. Consequently, it seems no need to modify legacy test metric for typeI-SinglePanel-r19 with 64 CSI-RS ports PMI reporting requirements.

Proposal 9: Use legacy test metric for typeI-SinglePanel-r19 with 64 CSI-RS ports PMI reporting requirements.

typeI-codebookMode-r19

In RAN4#115 meeting, Scheme A agreed for TypeI-codebookMode-r19 [3]. Considering both Scheme A and Scheme B are introduced in TS38.214, in which Scheme A is a straightforward extension of legacy Type I single panel codebook with low implementation complexity, while Scheme B introduces independent SD basis selection for each layer which may have better performance than Scheme A. The Scheme will be supported by UE should be based on UE capability. If we only define PMI reporting performance requirements for Scheme A, it is hard to promise UE performance which only support Scheme B. Hence, we’d like to propose additional test case with Scheme B for typeI-SinglePanel-r19.

Proposal 10: Specify additional test case for typeI-SinglePanel-r19, with typeI-codebookMode-r19 as Scheme B.

Configurations for 128 CSI-RS ports (if introduced)

Firstly, for the portMappingMethod, in order to avoid ambiguity for UE associating measurements from the aggregated K CSI-RS resources to the codebook port indices for PMI calculation assuming a single virtual CSI-RS resource, and the port mapping can also facilitate sub-array-based co-existence between pre-Rel-19 and Rel-19 UEs where only a portion of a larger array is used for pre-Rel-19 UEs. RAN1 introduced two types, i.e., ‘method1’ and ‘method2’, the definitions are as below. In which, ‘method1’ is a N1-dimension aggregation, while ‘method2’ is a N2-dimension aggregation.

38.211 chapter 7.4.1.5.3

- if the number of CSI-RS ports Ntot∈48, 64, 128 - if the higher-layer parameter portMappingMethod equals ‘method1’ and N1K is an integer where N1 is as defined in Table 5.2.2.2.1a-1 of [6, TS 38.214] p' = p+qN/20≤p<N/2p+K+q-1N/2N/2≤p<N where q=0,1,…,K-1 is the number of the CSI-RS resource within the aggregated CSI-RS resource. - if the higher-layer parameter portMappingMethod equals ‘method2’ p'=N2pN2'+qN2' + p mod N2' N2'=N2K where q=0,1,…,K-1 is the number of the CSI-RS resource within the aggregated CSI-RS resource, N2K is an integer, and N2 is as defined in Table 5.2.2.2.1a-1 of [6, TS 38.214].

K=4 and (16, 4) for (N1, N2) of aggregated CSI-RS resources agreed in RAN4#115 meeting for 128 CSI-RS port [3]. Furthermore, according to RAN1’s design principle, when (N1, N2) per CSI-RS resource is (4, 4), the (N1, N2) of aggregated CSI-RS resources should be (16, 4) by mapping method1, when (N1, N2) per CSI-RS resource is (8, 2), the (N1, N2) of aggregated CSI-RS resources should be (8, 8) by mapping method2. Thus, for 128 CSI-RS ports with 4 CSI-RS resources and (N1, N2) as (16, 4), mapping method1 should be used.

Observation 10: For 128 CSI-RS ports with K=4 CSI-RS resources, when the aggregated (N1, N2) is (16, 4), it could only be achieved by (N1, N2) = (4, 4) per CSI-RS resource with mapping method1.

Secondly, for the K CSI-RS resources should be within the same slot or different slots, RAN1 has below agreement in RAN1#116bis meeting.

Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, all the K>1 NZP CSI-RS resources also share the same QCL, PCoffset, and PCoffsetSS. In addition: - ‘within 1 slot’ should be basic feature and ‘within 2 consecutive slots’ should be UE capability

Accordingly, 38.214 defines X∈{1,2}  slots for K CSI-RS resources as following.

38.214 chapter 5.2.2.3.1

For a NZP-CSI-RS-ResourceSet for channel measurement with 2≤K≤4 resources, linked to a CSI-ReportConfig configured with codebookType set to 'typeI-SinglePanel-r19', 'typeI-MultiPanel-r19', 'eTypeII-r19', 'typeII-FePortSelection-r19' or 'eTypeII-Doppler-r19', the slot offsets of the K CSI-RS resources are configured within X∈{1,2} slots, without DL/UL switching in between the K resources, where X=1 implies that the K resources are configured in the same slot, and X=2 implies that the K resources are configured within two adjacent slots. If the NZP-CSI-RS-ResourceSet for channel measurement is aperiodic and the linked CSI-ReportConfig is configured with codebookType set to 'typeI-SinglePanel-r19', 'typeI-MultiPanel-r19', 'eTypeII-r19', 'typeII-FePortSelection-r19', each of the K CSI-RS resources can be configured by higher layer parameter additionalOneSlotOffset with one additional slot offset relative to the slot offset configured by the higher layer parameter aperiodicTriggeringOffset in the resource set. For an aperiodic NZP-CSI-RS-ResourceSet for channel measurement with KDOPP∈{4,8,12} CSI-RS resource groups of 2≤K≤4 resources each, linked to a CSI-ReportConfig configured with codebookType set to 'eTypeII-Doppler-r19', the k-th resource in all the KDOPP resource groups, with k=0,…,K-1, can be configured by higher layer parameter additionalOneSlotOffsetDopp with one additional slot offset relative to their respective CSI-RS resource group slot offset configured by the higher layer parameters aperiodicTriggeringOffset in the resource set and aperiodicResourceOffset in the codebook configuration.

Considering it is important to confine multiple CSI-RS resources in the same slot, to guarantee the phase coherence between CSI-RS ports from different CSI-RS resources. Otherwise, if multiple CSI-RS resources are distributed in two different slots, it would require UE to maintain downlink Rx phase coherence across different slots. There are extra efforts needed in UE implementation, e.g., UE has to lock the RF/hardware setting such as LNA and AGC across slots, to make sure the Rx phase does not change across those slots. From performance point of view, putting multiple CSI-RS resources in a same slot would achieve better channel estimation performance. If they are in two different slots, the channel variation due to Doppler or FTL residual error will result selecting a precoder which does not match with the real channel in a slot, because the PDSCH transmission applying this precoder is transmitted in one single slot, not across two slots. From UE implementation point of view, putting them in a same slot would reduce channel estimation latency, remove the necessity to do buffering for channel estimation, remove the necessity to lock RF/hardware setting across slots. Finally, the ‘within 2 consecutive slots’ depends on UE capability. Therefore, configure K CSI-RS resources in one slot should be the best option.

Thirdly, for 128 CSI-RS ports with K=4 CSI-RS resources in one slot, CSI-RS density equal to 1 seems not workable. Thus, we would like to propose CSI-RS density as 0.5 for 128 CSI-RS ports.

Proposal 11: If introduced, prefer to use following configurations for 128 CSI-RS ports,

- portMappingMethod as method1

- K CSI-RS resources in one slot

- CSI-RS density equal to 0.5.

- Same row configuration of CSI-RS resources as 64 CSI-RS ports, i.e., row#17 of TS38.211 Table 7.4.1.5.3-1

- typeI-codebookMode-r19 as both Scheme A and Scheme B

  - Assumptions for eTypeII-r19

MCS level

In last RAN4#116 meeting, the agreement to use the same MCS value(s) as Type I codebook agreed [4].  Our companion contribution [7] shows the SNR working points at 90% of maximum throughput are really low with MCS20. Therefore, we prefer to use MCS22 for 64 CSI-RS ports.

Proposal 12: Use MCS22 for eTypeII-r19 with 64 CSI-RS ports PMI reporting requirements.

  - Assumptions for typeII-Doppler-r19 (If introduced)

Duplex mode

Since the objective description in WID [1][2] clearly mentioned this objective is targeting for FR1. Both FR1 FDD and FR1 TDD should support related PMI reporting requirements for typeII-Doppler-r19.

Proposal 13: For typeII-Doppler-r19, the PMI reporting requirements of both FR1 FDD and FR1 TDD should be defined.

Number of Rx antennas

Since typical number of Rx antennas are 2 and 4 for FR1, the new PMI reporting requirements for typeII-Doppler-r19 should be defined for both 2 and 4 Rx antennas.

Proposal 14: For typeII-Doppler-r19, the PMI reporting requirements of both 2 and 4 Rx antennas should be defined.

Propagation channel and correlation configuration

As legacy PMI reporting requirements of doppler codebook are based on TDLA30-20 and XP Medium, we think it is a good starting point to use the same configuration.

Proposal 15: Use TDLA30-20 with XP Medium as the propagation channel and correlation configuration for typeII-Doppler-r19 test.

Number of CSI-RS ports

For typeII-Doppler-r19, 48, 64 and 128 CSI-RS ports are introduced in RAN1. Agreements in RAN1#116 meeting as below. Accordingly, at least 64 ports PMI reporting requirements should be defined for typeII-Doppler-r19 as 64 CSI-RS ports is the basic feature.

Agreement For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II and Rel-18 Type-II Doppler for up to 128 CSI-RS ports, as well as Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, support the following (N1, N2) values: Total # CSI-RS ports across aggregated resources (=P); (N1, N2). 48; (8,3). (6,4). 64; (16,2). (8,4). 128; (16,4). (8,8). The support of total # CSI-RS ports across aggregated resources (=P) and (N1, N2) are subject to UE capability. - For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II regular codebook, the (N1,N2) values for P=64 are supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities - For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler regular codebook, the (N1,N2) values for P=64 are supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities

Proposal 16: For typeII-Doppler-r19, the PMI reporting requirements of basic feature P=64 should be defined.

N1 and N2 configuration

The following (N1, N2) values agreed in RAN1#116 meeting.

Agreement For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II and Rel-18 Type-II Doppler for up to 128 CSI-RS ports, as well as Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, support the following (N1, N2) values: Total # CSI-RS ports across aggregated resources (=P); (N1, N2). 48; (8,3). (6,4). 64; (16,2). (8,4). 128; (16,4). (8,8).

Considering the meaningful deployment in practice, for 64 CSI-RS ports we prefer to set (N1, N2)= (8,4).

Proposal 17: For typeII-Doppler-r19, the PMI reporting requirements for 64 CSI-RS ports set (N1, N2)= (8,4).

Number of aggregated NZP CSI-RS resources

For more than 32 CSI-RS ports, CSI-RS resources aggregation of K NZP CSI-RS resource agreed in RAN1#117 meeting.

Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding aggregation of K NZP CSI-RS resources to attain 32 < P (or PCSI-RS) ≤ 128, support only the following combinations of K and P (or PCSI-RS): - For P (or PCSI-RS) = 48, K = 2 (each resource 24 ports) and 3 (each resource 16 ports) - For P (or PCSI-RS) = 64, K = 2 (each resource 32 ports) and 4 (each resource 16 ports) - For P (or PCSI-RS) = 128, K = 4 (each resource 32 ports)

For 64 CSI-RS ports set K=2 (each resource 32 ports) for the test assumption of typeII-Doppler-r19.

Proposal 18: For typeII-Doppler-r19, the PMI reporting requirements for 64 CSI-RS ports set K=2 (each resource 32 ports).

typeII-ri-restriction-r19

The RI restriction as same as Rel-16, Rel-17 and Rel-18 type-II codebooks, it is a 4bit bitmap. Considering rank 2 is enough to verify the implementation of new codebooks, we prefer to set the RI restriction as 0010.

Proposal 19: For typeII-Doppler-r19, set typeII-ri-restriction-r19 as 0010 as the starting point.

numberOfPMI-SubbandsPerCQI-subband-r19

This parameter has the same meaning as legacy R in Rel-16/Rel-17/Rel-18 Type-II codebook, we propose to use 1 as legacy setting.

Proposal 20: For typeII-Doppler-r19, set numberOfPMI-SubbandsPerCQI-subband-r19 as 1 as the starting point.

The number of configured resource groups Kdopp

For the number of configured resource groups, each resource group comprises K NZP CSI-RS resources, where K={2,3,4}. The available values are 4, 8 and 12. We prefer to use 4 as the starting point, which is as same as the setting for Rel-18 doppler codebook.

Proposal 21: For typeII-Doppler-r19, the number of configured resource groups Kdopp set as 4 as the starting point.

- Conclusion

In this contribution, we provide analysis and views on UE demodulation performance and CSI requirements for Rel-19 NR MIMO Phase 5 WI.

The proposals and observations could be summarized as:

For General Scope

Observation 1: The enhancements of CSI support up to 128 CSI-RS ports are not merely about increasing the number of antenna ports but also significantly improve on UE channel state estimation/reporting capabilities.

Observation 2: It is hard to conclude UE could support all five Rel-19 codebook types once it supports one Rel-19 codebook type 'typeI-SinglePanel-r19'.

Observation 3: Even up to 128 CSI-RS ports could be verified by 'typeI-SinglePanel-r19' test, and ‘eTypeII’ codebook could be verified by ‘eTypeII-r16’ test, it is still unable to deduce the conclusion that UE could support ‘eTypeII-r19’ considering higher computational complexity of eTypeII codebook.

Observation 4: eType II codebook is a typical codebook type widely used in Trial network, and eType II has been supported by some companies’ product.

Observation 5: TU allocated for defining requirements for MIMO Phase 5 WI, and we have enough time to define related requirements. If we miss this time slot, it would be hard to find another time slot to define performance requirements for eTypeII-r19 in the future. Then we have to face the problem that no performance test requirements to promise the PMI reporting performance for UE supporting eTypeII-r19.

Proposal 1: Prefer to introduce new PMI reporting requirements for enhanced codebook ‘eTypeII-r19’ with first priority.

Observation 6: For UEs which supporting SRS port grouping, it seems the implementation on UE receiver is still flexible, for 8Rx receiver, it could be two separate 4Rx MIMO detectors, could be two joint 4Rx MIMO detectors and even could be joint 8Rx MIMO detector.

Proposal 2: The SRS port grouping should not be equivalent and restricted with receiver type as two separate 4Rx MIMO detectors.

Proposal 3: About the verification of the mapping between SRS port grouping and codewords, we are open to discuss the test methodology if any.

Observation 7: The functions to estimate and report delay/frequency/phase offsets should be straightforward to be implemented and delay/frequency offsets could be ensured by RRM accuracy requirements.

Observation 8: The performance of delay/frequency/phase offsets compensation highly depends on network implementation, especially the phase offset for phase coherence of multi-TRPs should be guaranteed by network when it transmits signal, but not guaranteed by UE measurement and reporting.

Proposal 4: For UE reporting enhancement for CJT, prefer not to introduce new requirements for CJT calibration offset reporting for time/frequency/phase offsets.

Observation 9: When linking CJTC Dd and Rel-18 eType-II CJT CSI reports are jointly triggered or separately triggered with delayOffsetCompensation indication, UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI.

Proposal 5: Do not prefer to introduce new PMI reporting test for CJTC Dd and Rel-18 eType-II CJT CSI reports with delay offset compensation, since delay offset compensation is also needed at BS/TE side.

Test Assumption for typeI-SinglePanel-r19

Proposal 6: For NZP-CSI-RS configurations for 64 CSI-RS ports, prefer to use below table.

| NZP CSI-RS | CSI-RS resource 1 and 2 | CSI-RS resource Type |  | Aperiodic |
| --- | --- | --- | --- | --- |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  | CSI-RS resource 1 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (3, 10) |
|  | CSI-RS resource 2 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (5, 12) |
|  | CSI-RS resource 1 and 2 | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |

Proposal 7: Use scaling factor a to guarantee channel correlation matrix as a positive semi-definite matrix, for 64Tx (8,4,2) 4Rx, a=0.00091 and for 64Tx (8,4,2) 2Rx, a=0.0005.

Proposal 8: Use MCS22 (Table 1) for typeI-SinglePanel-r19 with 64 CSI-RS ports PMI reporting requirements.

Proposal 9: Use legacy test metric for typeI-SinglePanel-r19 with 64 CSI-RS ports PMI reporting requirements.

Proposal 10: Specify additional test case for typeI-SinglePanel-r19, with typeI-codebookMode-r19 as Scheme B.

Observation 10: For 128 CSI-RS ports with K=4 CSI-RS resources, when the aggregated (N1, N2) is (16, 4), it could only be achieved by (N1, N2) = (4, 4) per CSI-RS resource with mapping method1.

Proposal 11: If introduced, prefer to use following configurations for 128 CSI-RS ports,

- portMappingMethod as method1

- K CSI-RS resources in one slot

- CSI-RS density equal to 0.5.

- Same row configuration of CSI-RS resources as 64 CSI-RS ports, i.e., row#17 of TS38.211 Table 7.4.1.5.3-1

- typeI-codebookMode-r19 as both Scheme A and Scheme B

Test Assumption for eTypeII-r19

Proposal 12: Use MCS22 for eTypeII-r19 with 64 CSI-RS ports PMI reporting requirements.

For typeII-Doppler-r19

Proposal 13: For typeII-Doppler-r19, the PMI reporting requirements of both FR1 FDD and FR1 TDD should be defined.

Proposal 14: For typeII-Doppler-r19, the PMI reporting requirements of both 2 and 4 Rx antennas should be defined.

Proposal 15: Use TDLA30-20 with XP Medium as the propagation channel and correlation configuration for typeII-Doppler-r19 test.

Proposal 16: For typeII-Doppler-r19, the PMI reporting requirements of basic feature P=64 should be defined.

Proposal 17: For typeII-Doppler-r19, the PMI reporting requirements for 64 CSI-RS ports set (N1, N2)= (8,4).

Proposal 18: For typeII-Doppler-r19, the PMI reporting requirements for 64 CSI-RS ports set K=2 (each resource 32 ports).

Proposal 19: For typeII-Doppler-r19, set typeII-ri-restriction-r19 as 0010 as the starting point.

Proposal 20: For typeII-Doppler-r19, set numberOfPMI-SubbandsPerCQI-subband-r19 as 1 as the starting point.

Proposal 21: For typeII-Doppler-r19, the number of configured resource groups Kdopp set as 4 as the starting point.

- Reference

[1] RP-234007, New WID: NR MIMO Phase 5, RAN#102 meeting, Samsung

[2] RP-242394, WID revision: NR MIMO Phase 5, RAN#105 meeting, Samsung

[3] R4-2508627, Way Forward for [115][329] NR_MIMO_demod, RAN4#115 meeting, Samsung

[4] R4-2512527, Way Forward for [116][327] NR_MIMO_demod, RAN4#116 meeting, Samsung

[5] R4-2504903, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4#114bis, MediaTek.

[6] R4-2508272, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4#115, MediaTek.

[7] R4-2513872, Initial simulation results for Rel-19 MIMO Phase 5 Demod, RAN4#116bis, Samsung