3GPP TSG RAN WG4 Meeting #116-bis	R4-2513910

Prague, CZ, 13th – 17th Oct, 2025

Agenda item:			6.10.3

Source:	vivo

Title:	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In the last meeting, regarding the RRM test cases design for Fast SCell activation for UEs supporting Rel-18 EMR, there were some progress and the outcome was captured in summary[1]. In this document, we will provide our further analysis based on it.

- Discussion

Issue 2-2-1: Test case list Agreement: - Scenario: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 - PUCCH SCell activation for FR1 and FR2 - FFS: - Direct SCell activation at SCell addition for FR2 - UE capabilities and network configurations: - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - FFS: - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Regarding whether test cases should be defined for Configuration 3, the existing EMR tests aim to verify whether the UE can properly perform early measurements in idle mode and complete EMR result reporting. However, the purpose of the tests here is mainly to verify whether the UE can utilize EMR results for fast SCell activation. From this perspective, we prefer to validate this scenario and defining test cases.

Proposal 1: For UE capabilities and network configurations, RAN4 to consider

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

To reduce the test effort, it is reasonable to apply the rule that if a UE supports Direct SCell Activation or PUCCH SCell Activation, the Normal SCell Activation test case can be skipped.

Proposal 2: If a UE supports Direct SCell Activation or PUCCH SCell Activation, the Normal SCell Activation test case can be skipped.

- Summary

Proposal 1: For UE capabilities and network configurations, RAN4 to consider

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Proposal 2: If a UE supports Direct SCell Activation or PUCCH SCell Activation, the Normal SCell Activation test case can be skipped.

- References

[1]	R4-2509055, Topic summary for [116][212] NR_RRM_Ph5_Part2, CATT