3GPP TSG-RAN WG4 Meeting #116bis	R4-2513961

Prague, Czech, 13th – 17th October 2025

Agenda Item:	6.19.4

Source:	Ericsson

Title:	Simulation results for NTN phase 3 demodulation

Document for:	Information

# 1	Introduction

In RAN4#116 meeting, companies had agreement on many simulation assumptions for Rel-19 NR-NTN Phase 3 SAN demodulation requirement [1]. In this contribution, simulation results are delivered for:

- OCC length-2 and down-selection of channel model and frequency offset.

- RedCap and eRedCap PDSCH with NTN channel models

# 2	Simulation results

During the last RAN4 meeting’s discussion, companies have agreed on the basic test cases for RedCap and eRedCap with NTN support. It’s encouraged to bring simulation results in this meeting for alignment. In the following sections we provided our simulation results for both RedCap and eRedCap cases.

## 2.1	RedCap with NTN support

Following the agreements from the last meeting in Bangalore, we have run simulations for the following agreed test cases for RedCap UEs.

The SNR point of the 70% maximum throughput is captured in the last column.

Table 2.1-1 Simulation results for agreed RedCap test cases

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR at 70% max Tput |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 | 0.7dB |
| 2 | MCS4, 52 | NTN-TDLC5-200 | 32 | 1x1 | -0.9dB |
| 3 | MCS13, 52 | NTN-TDLC5-200 | 16 | 1x1 | 7.3dB |

## 2.2	eRedCap with NTN support

Following the agreements from the last meeting in Bangalore, we have also provided our simulation results for those agreed test cases for eRedCap UEs.

The SNR point of the 70% maximum throughput is captured in the last column.

Please note 1 that Test 1 is the same requirement as RedCap Test 1 in the above chapter, so we reused the simulation results here.

Please also note that Test 3 is the same requirement as TS 38.101-5 Table 8.2.1.2.2.1.1-3 Test 1-1. In This case we reused our previous simulation results here.

Table 2.2-1 Simulation results for agreed eRedCap test cases, without baseband bandwidth reduction

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR at 70% max Tput |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 | 0.7dB |
| 2 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x1 | 7.3dB |
| 3 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x2 | -2.7dB |
| 4 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x2 | 3.6dB |

Table 2.2-2 Simulation results for agreed eRedCap test cases, with baseband bandwidth reduction

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR at 70% max Tput |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x1 | 1.2dB |
| 2 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x1 | 7.1dB |
| 3 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x2 | -3.2dB |
| 4 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x2 | 3.4dB |

## 2.3	SAN with OCC length-2 demodulation requirements

Table 2.3-1: Test parameters

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Transform precoding |  | Enabled |  |
| Channel bandwidth |  | 15 kHz SCS: 5MHz 30 kHz SCS: 10MHz |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0,2,3,1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port | UE0 | UE1 |
|  |  | 0 | 1 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |  |
| Time domain | PUSCH mapping type | A |  |
| resource | Start symbol | 0 |  |
| assignment | Allocation length | 14 |  |
|  | PUSCH aggregation factor | n2 |  |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| Test Metric |  | SNR@70% Normalized throughput per UE |  |

Table 2.3-2: Minimum requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| TX | RX | Propagation conditions and correlation matrix | Frequency offset for multiplexed UEs (Hz) | MCS In Table 2 (TS 38.214 Table 6.1.4.1-2) | SNR [dB] |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-200 Low | 0 | 5 | -6.21 |
|  |  |  | 200 |  | -6.23 |
|  | 2 |  | 0 | 5 | -10.0 |
|  |  |  | 200 |  | -10.0 |
| 1 | 1 | NTN-TDLC5-200 Low | 0 | 5 | -8.27 |
|  |  |  | 200 |  | -8.26 |
|  | 2 |  | 0 | 5 | -11.44 |
|  |  |  | 200 |  | -11.44 |

# 3	Summary

In this contribution, we shared the simulation results for RedCap/eRedCap with NTN support and SAN demodulation with OCC performance requirements.

# References

[1] R4-2512644, “Way Forward for [116][331] NR_NTN_Ph3_demod”, Ericsson, RAN4#116, Bangalore.