# 3GPP TSG-RAN WG4 Meeting #116bis                                                                   R4-2513962

Prague, Czech Republic, Oct. 13-17, 2025

Source:                Tejas Networks

Title:                     SBFD BS to SBFD BS adjacent channel coexistence

Agenda Item:       7.2.4

Document for:     Discussion

# Introduction

In the RAN#109 plenary meeting, the objectives of the new WID on NR BS RF requirement evolution for FR1 and testing phase 2 were presented, as outlined in [1]. One of the key objectives includes addressing adjacent channel coexistence between SBFD BS.

- SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated. - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs

This paper presents our initial perspective on adjacent channel coexistence between SBFD base stations.

# Discussion

## SBFD BS to SBFD BS adjacent channel coexistence

Release 20 introduces enhancements for FR1 wide-area bands n104, n79, and n41 to enable SBFD only deployments without dependency on legacy TDD. The goal is to improve UL coverage and reduce latency while ensuring robust coexistence in new wide TDD bands. Compared to TDD–SBFD coexistence, SBFD–SBFD provides better performance but requires stricter ACIR through enhanced BS TX and RX specifications. To mitigate coexistence challenges, careful DL subband placement is critical. Four boundary configurations (UD/DU, UD/DUD, DUD/DU, DUD/DUD) have been identified as starting points to protect UL transmissions at spectrum edges from adjacent DL interference. An operator’s DL subband can serve as a guard for a neighbouring operator’s UL, leveraging SBFD’s inherent split-band structure without adding filter complexity. This approach supports improved adjacent-channel coexistence and maximizes SBFD benefits in wide-area deployments. When all operators deploy SBFD, inter-operator interference is significantly reduced. This requires tailored TX/RX requirements for in-band emissions and blocking to prevent DL leakage into adjacent UL. Full-band suppression using SBFD filtering is mandatory for all operators. These measures address inter-operator interference only; other interference types remain outside the scope. Full-site SBFD deployment is essential to avoid high inter-site CLI.

Observation 1: Full site SBFD deployment and SBFD filtering for full band suppression are mandatory to minimize inter-operator interference.

Proposal 1: Recommend full-site SBFD deployment to prevent inter-site CLI and maximize SBFD benefits.

## Simulation analysis

We analyze a coexistence scenario involving UMa deployments by two operators using adjacent channels in the same geographic area. The first operator, referred to as the victim, deploys SBFD, while the second operator, the aggressor, also uses SBFD. The evaluation focuses on the wideband interference power received by the uplink of the victim SBFD network due to the downlink transmissions from the aggressor SBFD network, as discussed in our earlier paper [2]. The baseline noise figure model is based on the default parameters specified in 3GPP TR 38.858 Annex E [3] for the DUD/DUD configuration. For UMa deployments where SBFD acts as the aggressor, the 99th percentile in-band blocking power is –30.47 dBm with a 100% GS, and –3.82 dBm with a 10% GS [2].

Observation 2: For UMa deployments with SBFD as the aggressor network and a 100% GS, the 99th percentile of in-band blocking power is –30.47 dBm. With a 10% GS, the in-band blocking power is –3.82 dBm.

Proposal 2: Confirm optional SBFD compliance (Note 1) and TDD fallback (Note 2) in TS 38.104/38.141-2, ensuring no legacy impact.

# Conclusion

Observation 1: Full-site SBFD deployment and SBFD filtering for full-band suppression are mandatory to minimize inter-operator interference.

Proposal 1: Recommend full-site SBFD deployment to prevent inter-site CLI and maximize SBFD benefits.

Observation 2: For UMa deployments with SBFD as the aggressor network and a 100% GS, the 99th percentile of in-band blocking power is –30.47 dBm. With a 10% GS, the in-band blocking power is –3.82 dBm.

Proposal 2: Confirm optional SBFD compliance (Note 1) and TDD fallback (Note 2) in TS 38.104/38.141-2, ensuring no legacy impact.

# Reference

- RP-252905, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2.

- R4-2505469, Views on modification of existing Rx requirements for SBFD, Tejas Networks Ltd.

- TR 38.858, “Study on evolution of NR duplex operation,” 3GPP.