3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513969

Prague, CZ, October 13th – 17th 2025

Agenda Item:	7.2.4

Source:	Ericsson

Title:	On SBFD-SBFD co-existence

Document for:	Approval

# 1	Introduction

In RAN#109, a new RAN4 led Rel-20 WI [1] on NR base station (BS) RF requirement evolution for FR1 and testing phase 2 was approved, which consists of two topics: Enhancement of co-location requirements and SBFD BS to SBFD BS adjacent channel coexistence. In this contribution, we present overview of SBFD BS to SBFD BS adjacent channel coexistence.

The objective of Rel-20 WI SBFD topic is to study coexistence among operators all deploying FR1 WA SBFD-capable BSs in the same band, with a focus on n104, n79, n41, and considering both co-located and non-collocated scenarios. The following discussion provides an overview of the key technical challenges and proposes a structured approach to expand the Rel-20 technical work.

- Objective of SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated. - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs

# 2	General assumptions on deployment scenarios

The fundamental concept enabling SBFD-SBFD coexistence is that the existence of DL subbands allows for a large frequency range within which analogue filters can achieve roll-off. As illustrated in Figures 1 and 2, each operator must provide suppression of unwanted emissions towards the others. For example, Operator 1 must attenuate emissions from its DL subband towards Operator 2, where the filter can roll off within Operator 2’s DL subband. Similarly, Operator 2 must protect its receiver from Operator 1’s DL subband by providing attenuation within its own DL subband. This mutual filtering is the mechanism through which SBFD operators suppress inter-operator interference.

- All operators in the band need to negotiate together to deploy SBFD with enhanced filtering dedicated to the SBFD-SBFD adjacent channel coexistence scenario.

- The proposed scenario will only suppress the inter-operator interference, meanwhile, the challenges related to other types of interference, e.g., self-interference, inter-sector interference, inter-site interference, remain and should be addressed separately.

- Operators need to deploy SBFD at every site on the same carrier to avoid severe inter-site interference that would degrade SBFD performance.

Figure 1: SBFD-SBFD co-existence arrangement (three operators)

Figure 2: SBFD-SBFD co-existence arrangement (two operators)

The BS design to meet SBFD-SBFD co-existence requirements would need to incorporate analogue filters based on the subband configurations of both its own and other operators. Consequently, the SBFD BSs deployed by the operators would need to assume a fixed carrier and subband allocation between operators that may not be adjustable without hardware changes by all involved operators.

- Carrier and sub-band allocations within a band would need to be fixed due to hardware restrictions.

The purpose of operators collaborating in order to deploy SBFD would be significantly undermined if there would be constraints on the operators also having to co-ordinate their BS sites in order to avoid being close to one another. In case there would be three operators within a band, co-ordination would become even more complex, since every operator would need to maintain at least a certain distance from every other operator. Rel-20 work should ensure full flexibility for operator site deployment. This should enable SBFD to function when operators are very close to each other (for example, within 10 m of one another), and also when they are sharing the same tower.

- Rel-20 work should ensure that all kinds of deployment scenarios can work, including re-use of existing sites, close (e.g., within 10 m of one another) and co-located BSs (tower sharing), without requiring operators’ collaboration on deploying sites.

RAN4 agrees that Rel-20 SBFD-SBFD co-existence work focuses on close (e.g., within 10 m of one another) and co-located BSs (tower sharing) to ensure operators to be able to reuse existing sites.

# 3	Technical works in Rel-20

To specify the necessary requirements, the following key areas should be studied:

## Step 1 - Deployment scenarios and assumptions

To start the Rel-20 work, it is important to set a clear reference scenario, which will be the basis for all subsequent simulations and analyses, ensuring that results from different companies are comparable and that the final requirements meet expectations.

- Deployment scenarios: Define and agree on a reference deployment scenario, including BS class, number of operators, grid-shift assumption, co-location model, example spectrum allocation and antenna structure assumption. Our proposal is as follows.

- BS class – FR1 WA BS.

- Number of operators – two operators.

- Grid-shift for co-existence – 5%, with ISD = 500 m.

- Co-location model.

- Example spectrum allocation – 4 GHz, which is aligned with Rel-18 assumptions.

- Antenna structure assumptions.

- Subband configurations: Reuse Rel-18 subband configurations.

## Step 2 - System-level co-existence simulation

With the reference scenarios established, the next step is to quantitatively analyze the interference and determine the necessary suppression levels. This is achieved through system-level coexistence simulations.

- System-level analysis: Conduct system-level co-existence simulations for the agreed reference scenarios to quantify the required suppression between operators.

## Step 3 - Co-location modelling

Unlike co-existence, which occurs in the far field, co-location is in the near field. Discussion is needed to do the modelling, which is related to the other part of this WI.

## Step 4 - Requirement derivation

The final step is to translate the quantitative simulation results into necessary BS RF requirements.

- Derive the specific requirement levels for:

  - SBFD-capable BS TX unwanted emissions: The necessary suppression level towards neighbouring operators’ UL subbands.

  - SBFD-capable BS RX in-band blocking: The necessary tolerance level for interference from neighbouring operators’ DL subbands.

RAN4 to expand the Rel-20 technical work in a structured way: The work begins by establishing reference deployment scenarios, proceeding to system-level co-existence simulation to evaluate performance, co-location modelling, and if needed, derive necessary RF requirements based on the worst case of co-existence and co-location.

# Conclusion

In the previous sections we made the following observations:

Observation 1	All operators in the band need to negotiate together to deploy SBFD with enhanced filtering dedicated to the SBFD-SBFD adjacent channel coexistence scenario.

Observation 2	The proposed scenario will only suppress the inter-operator interference, meanwhile, the challenges related to other types of interference, e.g., self-interference, inter-sector interference, inter-site interference, remain and should be addressed separately.

Observation 3	Operators need to deploy SBFD at every site on the same carrier to avoid severe inter-site interference that would degrade SBFD performance.

Observation 4	Carrier and sub-band allocations within a band would need to be fixed due to hardware restrictions.

Observation 5	Rel-20 work should ensure that all kinds of deployment scenarios can work, including re-use of existing sites, close (e.g., within 10 m of one another) and co-located BSs (tower sharing), without requiring operators’ collaboration on deploying sites.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 agrees that Rel-20 SBFD-SBFD co-existence work focuses on close (e.g., within 10 m of one another) and co-located BSs (tower sharing) to ensure operators to be able to reuse existing sites.

Proposal 2	RAN4 to expand the Rel-20 technical work in a structured way: The work begins by establishing reference deployment scenarios, proceeding to system-level co-existence simulation to evaluate performance, co-location modelling, and if needed, derive necessary RF requirements based on the worst case of co-existence and co-location.

# References

RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”